University of Limerick

Ollscoil Luimnigh

Limerick Ireland

Final Year Project Report

Title

An Experimental and Computer Aided Investigation into the Behaviour of a Thermally Uncompensated Pendulum and a Computer Aided Investigation into the Theoretical Performance of a Compensated Pendulum, with Particular Reference to Change in Mass Moment of Inertia with Temperature.

Project Supervisor:

Dr. John Jarvis

Written by:

Cormac Eason		9731318

A Report Submitted in Part Fulfilment of the Requirements

for the award of Bachelor of Mechanical Engineering.

Submitted to the UL Department of Mechanical and Aeronautical Engineering on 12/4/2001

Acknowledgements

The completion of this project would not have been possible without the help of the following people:

Dr. John Jarvis, Project Supervisor, for invaluable advice on the subject of Finite Element Analysis as well as all things horological.

Dr. Richard Stephen, for his compensated pendulum design, as well as his suggestions on experimental methods.

Mr. John Cunningham, for his assistance in constructing the electronic parts of my experimental equipment, as well as his patience in dealing with the not inconsiderable difficulties learning to program with LabVIEW presented to me.

Mr. Adrian Mulchinock, for collecting much of the literature I needed for my project during the course of his own two years ago.

## Summary

The aim of this project was to design and construct a test pendulum for the purpose of experimentally measuring the effect a temperature change has on an uncompensated pendulum in terms of the change in its length, as well as the change in its mass moment of inertia.

A compensated pendulum was also analysed theoretically, using a combination of ProEngineer and ProMechanica to calculate the mass properties of the pendulum before and after a temperature change of 25ºC. From these calculations, the effectiveness of the compensation could be found.

The significance of the mass moment of inertia was also investigated by means of manual calculations using Microsoft Excel to recalculate the mass moment of inertia of the test pendulum for various temperatures, allowing changes in mass moment of inertia and their effect on the periodic time to be analysed.

### Experimental Pendulum

The test pendulum was calculated to increase in period by 193.28s from an initial value of 1.405769274s, using Microsoft Excel. This correlated with calculations performed using ProEngineer and the design dimensions of the pendulum to 0.12%.

The ProEngineer and Excel calculations using the design dimensions correlate to 0.00686%, indicating that the design chosen for the pendulum can be analysed to a high degree of accuracy using manual calculations and several assumptions, rather than resorting immediately to computer analysis.

The Experimental results were affected to a far greater degree than was expected by the air currents in the oven used to heat the pendulum during the experiment. Because of this, the results of the experimental work are obviously in error, as the pendulum’s periodic time decreased by 513.38s instead of increasing as predicted. This suggests that if further work is to be done using this pendulum, it must be shielded from air currents during testing.

The correct trend of an increase in the periodic time was attained when the pendulum rod was heated on its own, without there being a change in the air flow around the pendulum. However, since the temperature could not be regulated or measured accurately for this experiment, no conclusion can be drawn as to whether the change accurately reflects the predicted behaviour.

### Compensated Pendulum

The compensated pendulum will change period by -26.80398096 s from a calculated initial period of 1.503166198 s. This means the pendulum will lose 1.54 Seconds for every day it spends heated to 25ºC over the temperature it was calibrated at.

More accurate dimensions, coefficients of thermal expansion and drawings are needed if a more accurate prediction as to the pendulum’s performance is required. Also, changes such as variation in the pendulum arc and the other environmental factors discussed in this report will have to be considered in order to produce a more accurate prediction for the pendulum.

### The Effects of Mass Moments of Inertia

It was found that the change in the mass moment of inertia of the test pendulum for the temperature range of the experiments can be approximated as linear, though it is a second order change in reality. The periodic time of the pendulum can also be safely approximated to change linearly with temperature.

## Nomenclature

The following terms are used in the subsequent report. The units given here are correct unless another unit is specified directly.

| Symbol | Meaning |  | Unit |
| --- | --- | --- | --- |
| A | Amplitude of Vibration |  | m |
| a | The acceleration of a point mass |  | ms2 |
| ap | Value of a dimension of a part which expands due to a temperature rise |  | m |
| c | Coefficient of thermal expansion |  | mm°C |
| cB | Coefficient of thermal expansion of Brass |  | mm°C |
| cCF | Coefficient of thermal expansion of Carbon Fibre |  | mm°C |
| cS | Coefficient of thermal expansion of Steel |  | mm°C |
| d | Distance from axis through Centre of Gravity of part to new parallel axis |  | m |
| dm | Mass of infinitesimally small part of a body. |  | kg |
| dW | Weight of infinitesimally small part of a body. |  | N |
| F | A Force |  | N |
| h | Distance from Pivot to Centre of Gravity of Compound Pendulum |  | m |
| I | Mass moment of inertia of a body about a specified axis |  | kg⋅m2 |
| I | Moment of Inertia of a part about its Centre of Gravity |  | kg⋅m2 |
| IG | Moment of Inertia of a part about its Centre of Gravity |  | kg⋅m2 |
| Ip | Mass moment of inertia of part with dimension ap about its centre of gravity |  | kg⋅m2 |
| IXX | Moment of Inertia of part about the Pendulum's Axis of Rotation, or other specified axis. |  | kg⋅m2 |
| K | Constant of integration |  | m2s2 |
| k | Spring stiffness |  | N/m |
| L | Length of Simple Pendulum |  | m |
| LB | Total length of brass parts of compensated pendulum |  | m |
| LCF | Total length of carbon fibre parts of compensated pendulum |  | m |
| LF | Final length of pendulum after temperature change |  | m |
| LG | Distance from pivot to centre of gravity of bob |  | m |
| LI | Initial length of pendulum before temperature change |  | m |
| LS | Total length of steel parts of compensated pendulum |  | m |
| L1,L2 | Length of steel parts of compensated pendulum |  | m |
| M | Moment |  | Nm |
| m | The mass of a body |  | kg |
| p | Natural number from 0 to 3 |  | No unit |
| r | Distance from chosen axis to infinitesimally small mass dm |  | m |
| rO | Distance from axis through centre of gravity of body to infinitesimally small mass dm |  | m |
| T | Torque |  | Nm |
| u | Distance from axis through centre of gravity of body to mass dm in direction of d |  | m |
| v | The speed of a point mass |  | ms |
| W | Weight of a part |  | N |
| x | Linear displacement of a point mass from its equilibrium position |  | m |
| x | The acceleration of a point mass |  | ms2 |
| x | Distance along x-axis to centre of gravity of a body |  | m |
| y | Distance along y-axis to centre of gravity of a body |  | m |
| z | Distance along z-axis to centre of gravity of a body |  | m |
| Greek Symbols |  |  |  |
|  | Angular acceleration | Rad/s2 |  |
| ΔT | Change in Temperature | 25 ºC |  |
| δIp | Change between consecutive values of Ip | Ip+1−Ip |  |
|  | The angle through which the pendulum swings from the centre of its arc | Radians (Degrees only if Specified) |  |
| θ | Angular acceleration | Rad/s2 |  |
| π | Ratio between circumference of a circle and its radius | 3.141592654 (No Unit) |  |
| ρS | Density of steel | kgm3 |  |
| ρCF | Density of carbon fibre | kgm3 |  |
| ρB | Density of brass | kgm3 |  |
| τ | Periodic time of Pendulum | Seconds (s) |  |
|  | The time offset for the Pendulum | Radians |  |
|  | Natural circular frequency of an oscillation | Rad/s |  |
| n | Natural circular frequency of an oscillation | Rad/s |  |

## Abbreviations and Other Terms Used

| Arc | This is the total angle through which a pendulum swings. |
| --- | --- |
| CAD | Computer Aided Design. |
| Cycloid | This is a shape which is made by tracing the path a point on the circumference of a circle follows as the circle is rolled on a curve. |
| D | Depth |
| DH | Distance of Holes in Pendulum Rod from the Ends of the Rod |
| Dia | Diameter of Part |
| G | The point at which the centre of gravity of a body resides |
| H | Height of part |
| Horology | This is the study of timekeeping including clocks and watches. |
| MHz | Megahertz = 1 million cycles per second. |
| Isochronous | The ability to maintain the same periodic time for each period. |
| L | Length. |
| LEQ | Equivalent Length of Pendulum (See h). |
| MOI | Mass Moment of Inertia. |
| m | Milli- Used as a prefix to a unit, multiplying it by 10-3 |
| n | Nano- Used as a prefix to a unit, multiplying it by 10-9 |
| Parametric | This is a type of CAD program which stores a model as a set of instructions on how to draw the model instead of a set of points. This allows the dimensions of the model to be changed and the model to be regenerated. |
| Period | This is the time taken for a pendulum to perform a full cycle, i.e. to travel from the centre of its swing fully to the right, back the full way to the left and back to the centre of the swing. In horology, a pendulum will sometimes be classified by half its period, for example a ‘seconds’ pendulum has a periodic time of two seconds. |
| Pin Dia | Diameter. |
| Regulator | This is the term used to describe a precise mechanical clock. |
| Vol | Volume of part |
| W | Width. |
| μ | Micro- Used as a prefix to a unit, multiplying it by 1×10-6 |