Table of Contents

1.0	Introduction	7

2.0	Background Theory	9

2.1	Moments	9

2.2	Centre of Gravity	10

2.3	The Difference Between the Centre of Gravity and the Centroid	13

2.4	Mass Moments of Inertia	13

2.5	Parallel Axis Theorem	15

2.6	Simple Harmonic Motion	16

2.7	Modelling the Behaviour of a Simple Pendulum	19

2.8	Modelling the Behaviour of a Compound Pendulum	20

2.9	Coefficient of Thermal Expansion	21

2.10	Compensation Calculations	22

3.0	Errors in Mechanical Clocks	24

3.1	Circular Error	24

3.2	What causes a Pendulum to Change Rate?	25

3.2.1	Environmental Conditions	25

3.2.2	Temperature Change	27

3.2.3	Pressure	27

3.2.4	Gravity	28

3.2.5	Energy input	28

3.2.6	Conclusion	29

4.0	Experimental Work	32

4.1	Introduction and Aims	32

4.2	Design Criteria For Test Pendulum	33

4.3	Equipment Used	36

4.4	Equipment Set up	37

4.5	Calibration Procedure for Test Pendulum	38

4.6	Experimental Procedure	40

4.7	Further Test to Confirm Pendulum Properties	43

5.0	Results and Discussion	44

5.1	Experiments Involving the Oven	44

5.2	Results from Experiments Involving the Independent Heating of the Pendulum Rod	48

5.3	The Modelling of the Periodic Time Decrease as the Experiment Progressed	50

6.0	The Effect of the Changing Mass Moment of Inertia	51

6.1	Conclusions	55

7.0	The Compensated pendulum	57

7.1	The construction of the Compensated Pendulum	59

7.2	Analysis of the compensated pendulum using ProEngineer and ProMechanica	61

7.3	Finite Element Analysis of the Split block and Suspension Spring	63

7.4	Results	65

7.5	Results for Compensated Pendulum	67

7.6	Sources of Error	68

7.6.1	Calculation Error	68

7.6.2	Experimental Error	69

8.0	References	73

8.1	Recommendations for Further Work	74

9.0	Appendices	76

9.1	Pendulum Nomenclature	77

9.2	Formulae used to Calculate the Mass Moment of Inertia for Standard shapes.	79

9.3	Theoretical Calculations for the Test Pendulum	80

9.3.1	Calculations for the Test Pendulum using Actual Dimensions	80

9.3.2	Calculations for the Test Pendulum using Theoretical Dimensions	81

9.4	Calibration of the K Type Thermocouple Amplifier	83

9.5	LabVIEW	84

9.5.1	Understanding LabVIEW diagrams	86

9.6	The temperature Logging Program	88

9.7	The Periodic Time Logging Program	89

9.8	Construction Drawings of the Test Pendulum	93

9.9	Details of the Compensated Pendulum	100

9.9.1	The Parts of the Compensated Pendulum	100

9.9.2	The Amalgamated Parts which were Analysed	101

9.10	Dimensioned Drawings of Compensated Pendulum Parts	101

9.11	Amalgamated Parts Analysed	108

9.12	Comparison of Separate Parts of Compensated Pendulum with Combined Parts	112

9.13	Compensated Pendulum Mass Properties as Calculated by ProEngineer	113

9.13.1	Initial Pendulum Mass Properties with respect to Co-ordinate system at top of suspension spring. Ixx is inertia about pendulum rotation axis.	113

9.13.2	Deformed Pendulum Mass Properties with respect to Co-ordinate system at top of suspension spring. IXX is inertia about pendulum rotation axis.	115

9.14	Carbon Fibre Rod Details	118

9.15	Properties of the Materials used	119

9.16	Benchmarking	120

9.17	Finite Element Analysis Run Summaries	123

9.17.1	Run Summary for Brass Benchmark Test	123

9.17.2	Run Summary for Carbon Fibre Benchmark Test	125

9.17.3	Run Summary for Steel Benchmark Test	127

9.17.4	Run Summary for Split Block and Spring Contact Analysis	130

9.18	Finite Element Analysis Report Files	133

9.18.1	Carbon Fibre Displacement Results	133

9.18.2	Brass Displacement Results	134

9.18.3	Steel Displacement Results	135

9.18.4	Result for displacement of a point at the end of the Split Block calculated using Finite Element Analysis	136

9.19	Summary of Displacement Calculation Results	137

9.20	Raw Data from Low Temperature Tests	140

9.21	Raw Data from High Temperature Tests	148

List of Figures

Figure 2.2.1: Experimentally Determining the Centre of Gravity of a Body	11

Figure 2.2.2: Theoretical Calculation of the Centre of Gravity	11

Figure 2.4.1: Calculation of Moment of Inertia	14

Figure 2.5.1: The Parallel Axis Theorem	15

Figure 2.6.1: Spring Mass System Demonstrating Simple Harmonic Motion	17

Figure 2.7.1: A Simple Pendulum	19

Figure 2.8.1: A Compound Pendulum	20

Figure 2.10.1: Compensated Pendulum	22

Figure 3.1.1: Circular Error Graph	24

Figure 3.1.2: Circular Cheeks used to make the arc of the pendulum cycloidal	25

Figure 4.4.1: The Experimental Pendulum set up in the Oven.	37

Figure 4.4.2: Close-up of Pendulum Bob, showing Proximity Switch, threaded bar and thermocouple.	37

Figure 4.4.3: Equipment used to log Temperature and Periodic Time.	37

Figure 4.4.4: Power Supply, Timer/Counter, Thermocouple Reader and LabVIEW Connector Board.	37

Figure 4.4.5: Wiring Diagram for Experimental Equipment.	38

Figure 4.5.1: Confirming that the pendulum frame is Level.	38

Figure 5.1.1: Results of the Experiments performed using the oven (The Periodic time measured is half the actual Period)	45

Figure 5.2.1: Results from Heating Pendulum Rod on its Own	49

Figure 5.3.1: A best fit line modelling the change in the period of the pendulum averaged over three experiments as a logarithmic decrement.	50

Figure 5.3.1: Mass Moment of Inertia changes in the test Pendulum due to temperature changes.	52

Figure 5.3.2: Variation of Periodic Time with Temperature for the Test Pendulum	52

Figure 6.1.1: The Compensated Pendulum	57

Figure 7.3.1: The Finite Element Analysis Model	63

Figure 7.4.1: A plane cut through the deformed model, showing stress	66

Figure 7.4.2: A contour plot showing the displacements of the different parts of the model. The thin lines shown the displacement of the spring.	66

Figure 9.1.1: Clock and Pendulum Nomenclature	77

Figure 9.2.1: Cuboid Mass Moment of Inertia	79

Figure 9.2.2: Cylinder Mass Moment of Inertia	79

Figure 9.4.1: Thermocouple Amplifier	83

Figure 9.6.1: Temperature Logging Program Block Diagram	88

Figure 9.6.2: Control Window for Temperature Logging Program	89

Figure 9.7.1: Control Window for the Periodic Time Logging Program	90

Figure 9.7.2: Block Diagram for the Periodic Time Logging Program	90

Figure 9.8.1:Test Pendulum Assembly	93

Figure 9.8.2: Close up of Ruler, proximity switch, threaded bar and Bob	93

Figure 9.10.1: Block Pin and Clamp Pin are both exactly the same	101

Figure 9.10.2: Pendulum Bob	102

Figure 9.10.3: Bob Pin	102

Figure 9.10.4: Carbon Fibre Rod	103

Figure 9.10.5: Clamp	103

Figure 9.10.6: Mounting	104

Figure 9.10.7: Rating Nut	104

Figure 9.10.8: Rivet	105

Figure 9.10.9: Rod Base	105

Figure 9.10.10: Screw	106

Figure 9.10.11: Suspension Spring	106

Figure 9.10.12: Rod Top	107

Figure 9.10.13: Sleeve	107

Figure 9.10.14: Split Block	107

Figure 9.11.1: Base Assembly	108

Figure 9.11.2: Deformed Base Assembly	108

Figure 9.11.3: Deformed Pendulum Bob	109

Figure 9.11.4: Deformed Carbon Fibre Rod	109

Figure 9.11.5: Deformed Sleeve	110

Figure 9.11.6: Assembly of Bottom of Pendulum, showing bob, bob pin, rating nut, rod base, carbon fibre rod and sleeve.	110

Figure 9.11.7: Top Assembly	111

Figure 9.11.8: Deformed Top Assembly	111

Figure 9.11.9: Top of Pendulum assembly, showing mounting, split block, block pin, rivet, suspension spring, clamp, screw, clamp pin, top rod and carbon fibre rod.	111

Figure 9.16.1: The Finite Element Analysis Benchmark Model	120

Figure 9.16.2: Initial (Purple) and Deformed (Blue) Benchmark models for Carbon Fibre and Steel Respectively	121

Figure 9.16.3: The Queried X-Displacements in millimetres for the Steel Model (Values circled in red are maximum or minimum values).	121

List of Tables

Table 1: Effects of Different Environmental Variables on Pendulum	31

Table 2: Summary of Experimental Results Compared with Theoretical and ProEngineer Calculations.	46

Table 3: Comparison of Percentage Differences Between Results	47

Table 4: Components of the Compensated Pendulum	60

Table 5: Amalgamated Parts List	60

Table 6: Mass Properties of the Compensated Pendulum Assembly before and after the Temperature Change as calculated by ProEngineer.	67

Table 7: Density changes in the parts of the Compensated Pendulum before and after the 25ºC Temperature rise.	112

# Introduction

Timekeeping has long been one of mankind’s greatest fascinations. From structures such as the Newgrange Passage Tomb, built in the stone age, the inner chamber of which sees sunlight only at sunrise on the Winter solstice, to the modern atomic clocks, which are accurate to within seconds for the whole life of the universe, knowing precisely what time it is and the measurement of time passing has long obsessed mankind.

Since time flows continuously, it would make sense to use a continuously flowing medium to measure it. This is done in timekeepers such as hourglasses and water clocks, but it is extremely difficult to regulate continuous flow, so these timekeepers are highly prone to error.

It was found that setting a weight at the end of a rod and allowing it to oscillate through a small angle provides a more easily regulated measure of time. This is why, even today, mechanical clocks rely on pendulums or other types of simple harmonic motion to provide them with regulation.

However, as with all mechanical systems, there are a considerable number of variables which affect the operation of a pendulum, and these must be understood and compensated for before a pendulum can be built which can be hoped to achieve a high level of accuracy.

The most obvious source of error in a pendulum is that of temperature variation. It is well known that most materials expand as they are heated. It is also known that the longer a pendulum is, the slower its rate of oscillation is. When these facts are known, it is obvious that some system must be put in place to ensure that the pendulum remains isochronous, or beats out the same periodic time, regardless of temperature.

This project looks in particular at the effect of a 25ºC temperature change on both the length of the pendulum and on its mass moment of inertia, both of which are properties which will change with temperature. A solid modelling parametric CAD program called ProEngineer is used for the mass property calculations, as it can deal quite easily with the awkward shapes of the components of the pendulum.

The ProMechanica Finite Element Analysis software was also used to analyse the interaction of four connected parts of the pendulum. These were made from different materials, resulting in a complex contact stress problem, which can be solved relatively easily using Finite Element Analysis. This problem would be extremely difficult to solve satisfactorily using standard analytical methods because it would require assumptions to be made to simplify the model.

The second aspect of this project was to design and build an uncompensated pendulum, which can be used in experiments to measure the effect of a temperature change on the pendulum.

The third aspect of the project was to investigate the effect of the mass moment of inertia on the behaviour of the test pendulum as the temperature changed.

# Background Theory

## Moments

A moment is the term used to describe a turning force or torque. It is calculated by multiplying the linear force applied by its perpendicular distance from the point about which the moment is to be calculated[6].

In Figure 2.1 below, a rigid beam of zero mass is allowed to pivot about the point O, and forces FA, FB and FC are applied perpendicular to the beam at A, B and C. The distances between each force and the point O are a, b and c respectively. In order that the beam remains in equilibrium under these forces, the sum of the moments due to these forces must equal zero.

Figure 2.1: Calculation of Moments

To calculate the moments due to the forces, each force is multiplied by its perpendicular distance from the pivot O. It can be seen that the effect of the forces FA and FB will be to turn the beam in an anticlockwise direction and the effect of the force FC is to turn the beam clockwise. In order to differentiate mathematically between clockwise and anticlockwise moments it is convenient to set the point O as the origin for a co-ordinate system.

This results in the distance a being negative as it is on the negative side of the x axis, while b and c are positive. The same procedure is followed for the forces, making FB positive as it acts in the positive y direction, while FA and FC are both negative. The overall effect of this is that anticlockwise moments are positive while clockwise moments are negative.

The equation for the sum of the moments is as follows:

M=(−a)(−FA)+(b)(FB)+(c)(−FC)=0	Equation 1

## Centre of Gravity

The centre of gravity of a body is the point through which gravity causes its weight to act, regardless of the orientation of the body. The centre of gravity allows irregularly shaped bodies to be replaced by point masses once it is known where the centre of gravity is, and therefore where to put the mass. In order for a body to be in equilibrium, the sum of the moments exerted by the forces acting on the body including gravity and the net force acting on the body must equal zero. If this is not the case, the body will accelerate, either in a linear or angular fashion, resulting in a dynamics problem. In order to model this behaviour, the mass moment of inertia must be known also. This is covered in Section 2.4.

The centre of gravity for a body of arbitrary dimensions can be determined experimentally by suspending it on a string A from three different points of the body. Since the string supplies the only force supporting the body and the body is in equilibrium, the imaginary line made by the string through the body at each point must pass through the body's centre of gravity. Where the three lines cross, therefore, is the centre of gravity of the body.

Figure 2.2.1: Experimentally Determining the Centre of Gravity of a Body

In order to mathematically calculate the centre of gravity, consider a body of mass m and weight mg = W. It is divided into an infinite number of smaller masses, each of mass dm. The force of gravity causes all of these masses to exert a force dm·g = dW in the -y direction Relative to the co-ordinate system in Figure 2.2.2.

Figure 2.2.2: Theoretical Calculation of the Centre of Gravity

Summing the moments exerted by each dW about each axis using integration gives the moment exerted by the whole body about that axis. This is equal to the moment the total mass will exert through its centre of gravity about the axis. When the centre of gravity is assumed to be a distance x from the x axis, y from the y axis and z from the z axis the following equations can be derived:

x⋅W=xdW

y⋅W=ydW

z⋅W=zdW

Equations 2

These can also be expressed as

x=xdWW

y=ydWW

z=zdWW

Equations 3

If a completely exact calculation of the centre of gravity is required, it should be noted that gravity acts towards the centre of the earth, and therefore gravitational forces are not parallel as assumed previously. Also, gravitational force varies with the inverse square of the distance moved from the centre of the Earth, therefore the masses dm positioned further away from the centre of the Earth will be multiplied by a smaller value of g than masses nearer the Earth's centre.

This means that the position of the centre of gravity will change depending on the orientation of a body. However, since the parts being analysed in this project are extremely small with respect to the size of the Earth, the assumptions made here are acceptable[6].

## The Difference Between the Centre of Gravity and the Centroid

The centroid is calculated by dividing the body being analysed into infinitely small volumes of size dV, and using these to calculate the x, y and z values, substituting dV for dW and V for W in the centre of gravity Equations , y=ydWW and z=zdWW, (Equations 3). The centre of gravity and the centroid coincide for bodies of uniform density and, as all bodies analysed in subsequent calculations are assumed to have uniform density, the terms can be used interchangeably[6].

## Mass Moments of Inertia

The mass moment of inertia is a measure of the resistance of a body to angular acceleration. It is the angular equivalent of the mass in linear acceleration. This can be seen in the equations below where F is the applied force, m is the mass of the body and a is its linear acceleration. For the angular equation, T is the applied torque or moment, I is the mass moment of inertia of the body and α (alpha) is its angular acceleration. The angular acceleration α can also be written as d2θdt2 or θ.

F=ma

Equation 4

T=Iα

Equation 5

The calculation of the mass moment of inertia is similar to the calculation of the centre of gravity in that it involves dividing the body to be analysed into an infinite number of smaller masses and using integration to calculate the total effect of these masses.

Figure 2.4.3: Calculation of Moment of Inertia

Consider the body in Figure 2.4.3. It is being accelerated at the rate of α rad/s2 about the axis O-O. This means that at any instant the point P is accelerating linearly at a rate of α⋅r m/s2. The mass of the infinitesimally small part of the body at point P is dm. When these values are substituted into the linear acceleration equation  (Equation 4), the resulting equation for any mass dm is:

F=dm⋅α⋅r	Equation 6

Since Moment or Torque is force times distance;

T=F⋅r=dm⋅α⋅r2	Equation 7

Comparing this with  (Equation 5) gives:

I=r2⋅dm	Equation 8

To calculate the moment of inertia for the whole body, all the dm values must be added together using integration, therefore the mass moment of inertia about the axis O-O is[7]:

IO=r2⋅dm	Equation 9

## Parallel Axis Theorem

The parallel axis theorem is used when the mass moment of inertia of a body about an axis through its centroid is known. It allows the moment of inertia of the body to be calculated through any axis parallel to this axis.

Figure 2.5.4 shows a body with its centroid or centre of gravity at G. An axis passes through G, meeting the axis u at O. A second axis through C is parallel to this axis. The mass moment of Inertia of the body about G is I.

Figure 2.5.4: The Parallel Axis Theorem

To fill values into the integral for the new axis, the distance r must be calculated. The cosine rule (r2=ro2+d2−2⋅ro⋅d⋅Cosθ) is used to calculate this value as follows:

I=r2dm=ro2+d2−2⋅r0⋅d⋅Cos(180−θ)  dm	Equation 10

=ro2+d2+2⋅ro⋅d⋅Cosθ  dm

=ro2⋅dm+d2⋅dm+2⋅d⋅u⋅dm

Equation 11

From Equation 11, I=ro2⋅dm, the second term is md2, and the third term equals zero. The reason for this is that since the axis through O also passes through G, the centre of gravity, the average value of u when the masses dm for the whole body are added together will equal zero. This gives the Parallel Axis Theorem result[7]:

I=I+md2	Equation 12

## Simple Harmonic Motion

A body performs Simple harmonic motion when its acceleration is proportional to its displacement from a certain point and directed towards this point. This is the type of motion which a pendulum will undergo when it is displaced from its neutral position and allowed to swing freely with no external forces acting on it. For Linear motion, the equation describing this is a∝−x. For convenience, the constant of proportionality is made equal to ωn2, a number which must be positive since it has been squared, giving the result:

a=−ωn2⋅x	Equation 13

Also, a=d2xdt2=x	Equation 14

Applying this to a physical problem, consider the spring-mass system shown in Figure 2.6.5 part  below. If the mass is displaced a distance x = A from its equilibrium position PEq. and released, it will perform simple harmonic motion about the equilibrium position. The free body diagram for the displaced mass is given in . The same free body diagram is shown again in , with the net force exerted on the mass by the spring shown.

Figure 2.6.5: Spring Mass System Demonstrating Simple Harmonic Motion

The equation of motion for the spring mass system is kx=−mx. Setting ωn=km gives:

x+ωn2⋅x=0

Equation 15

This is a differential equation describing the movement of the mass. The following transformation can be used in order to solve this equation:

x=d2xdt2=dvdt, (v is the speed at which the mass is moving)	Equation 16

dvdt=dxdt⋅dvdx=d2xdt2	But v=dxdt

⇒v⋅dvdx=d2xdt2=x	Equation 17

Substituting this into  gives:

v⋅dvdx+ωn2⋅x=0⇒v⋅dv=−ωn2⋅x⋅dx

Solving gives:

v22=−ωn2⋅x22+K, Where K is a constant of integration.

Equation 18

For all simple harmonic motion, v = 0 when x = A, where A is the amplitude of the oscillation and v is the speed of the particle. Substituting these conditions into Equation 18. gives K=ωn2⋅A22, giving the overall equation:

v22=−ωn2⋅x22+ωn2⋅A22	Equation 19

⇒v=±ωn⋅A2−ωn⋅x2

⇒v=±ωn⋅A2−x2	Equation 20

But since v = dxdt, the following integral can be derived:

±dxA2−x2=ωn⋅dt	Equation 21

Solving gives:

Sin−1xA=ωn⋅t+ψ, where ψ is a second constant of Integration.

⇒x=A⋅Sin(ωn⋅t+ψ)	Equation 22

This equation is for a sine wave of amplitude A metres, with a circular frequency of ωn radians per second, and a phase angle of ψ radians. This equation will model undamped linear simple harmonic motion for all cases, provided the correct initial conditions are substituted into the equation[8].

## Modelling the Behaviour of a Simple Pendulum

A simple pendulum consists of a point mass m at the end of a rigid massless beam of length L. It is displaced a distance x from its initial position and allowed to oscillate as shown in Figure 2.7.6. Splitting the force due to the point mass into its components perpendicular and parallel to the beam, it can be seen that the force which is driving the pendulum to return to the centre of its swing is mg⋅Sinθ. The linear acceleration for the pendulum mass is a or x. Using the linear equation of motion F=ma (Equation 4), the following result is obtained.

Figure 2.7.6: A Simple Pendulum

mg⋅Sinθ=−m⋅a

Equation 23

Assuming that the angle  is small, Sin =  when  is expressed in radians. This means that Sinθ=θ=xL. This gives the result:

x+gL⋅x=0

Equation 24

Comparing Equation 24 to x+ωn2⋅x=0 (Equation 15) gives:

ωn=gL Rad/s

From this, the periodic time is calculated using the formula τ=2πωn giving:

τ=2π⋅Lg Seconds

Equation 25

## Modelling the Behaviour of a Compound Pendulum

A compound pendulum not only has mass, but also has a moment of inertia. Any pendulum which exists in three dimensions must be analysed as a compound pendulum, using the T=Iα formula (Equation 5). I is defined as the moment of inertia of the compound pendulum about its pivot axis.

The compound pendulum analysis is very similar to that for a simple pendulum. However, the compound pendulum analysis equates torque values rather than force values.

Figure 2.8.7: A Compound Pendulum

Using Equation 5, and assuming that the angle  is small (Below 2º), Sin   when  is in radians, the motion of the pendulum can be expressed as:

mg⋅h⋅Sinθ=−Iθ⇒θ+mg⋅hIθ=0

Equation 26

Comparing this to x+ωn2⋅x=0 (Equation 15) gives:

ωn=mg⋅hI Rad/s

Therefore, the periodic time is

τ=2πωn=2π⋅Img⋅h Seconds.

Equation 27

It should be noted that for both pendulum periodic time formulae, the value of the angle  must be small in order that the formula gives a reasonably correct answer. In reality, a maximum value of 2º is acceptable see Section 3.1.

## Coefficient of Thermal Expansion

All materials experience slight dimensional changes when they are heated. The coefficient of thermal expansion is a way of quantifying this change. The coefficient of thermal expansion for a particular material is defined as the change in length of a one unit long bar of this material after it undergoes a temperature change of 1ºC.

The formula used to calculate this change is shown below:

LF=LI(1+ΔT⋅c)	Equation 28

Where LF is the new length of the part, LI is the initial length, T is the temperature change and c is the coefficient of thermal expansion.

## Compensation Calculations

In the diagram of the compensated pendulum below, L1 and L2 are the lengths of the two steel parts of the pendulum. Since these will behave the same way as a single steel rod of the same total length, the total length of the steel parts is:

L1+L2=LS	Equation 29

It is assumed that the centre of gravity of the pendulum is the same as the centre of gravity of the Bob. It is for this reason that the length LG must remain constant in order for the pendulum to be compensated.

The periodic time of the pendulum is designed to be 1.5 Seconds. Using the simple pendulum periodic time formula, Equation 27, LG is calculated to be:

Figure 2.10.8: Compensated Pendulum

LG=1.52π2⋅g=0.559m	Equation 30

The length of the brass part of the pendulum is also known, LB=0.05m. This means that there is enough information to solve for LS and LCF. The equation used to solve for these values equates the initial lengths of the components with the final lengths after an arbitrary temperature rise ΔT.

LG=LS+LCF−LB=LS(1+ΔT⋅cS)+LCF(1+ΔT⋅cCF)−LB(1+ΔT⋅cB)

Equation 31

This simplifies to

LCF⋅ΔT⋅cCF=LS⋅ΔT⋅cS+LB⋅ΔT⋅cB	Equation 32

Substituting LS=LG+LB−LCF (Derived from Equation 31), gives the solution for the length of the Carbon Fibre rod:

LCF=LB⋅(cB−cS)−LG⋅cScCF−cS=0.4872m	Equation 33

This gives LS = 0.1218m [11].

# Errors in Mechanical Clocks

## Circular Error

The first thing which must be said about pendulums is that they do not actually perform simple harmonic motion. As has been shown previously in this report, in deriving the equations of motion for the pendulum see Section 2.7, it is assumed that the Sine of the angle  through which the pendulum is displaced from the centre is equal to the angle in radians. This is quite acceptable in most cases, but in the use of pendulums for precision timekeeping, it can be a major source of error.

Figure 3.1.9: Circular Error Graph

The graph above shows how the error climbs exponentially as the half arc angle  rises past 2º or so. It is for this reason that the pendulum has to move through exactly the same arc angle for each oscillation, or else it will not remain isochronous. As the arc angle rises, the periodic time of the pendulum will also increase, causing the clock rate to decrease.

To compensate for circular error, the pendulum must be made to swing through a cycloidal arc rather then in a circular arc. The effect of this is that as the pendulum swings further from the centre of its arc, its effective length should drop, thereby causing the rate of the clock to increase until the pendulum returns to its design arc. A means of doing this was invented by John Harrison, a clockmaker in England in the 1700’s, and is shown in Figure 3.1.10.

Figure 3.1.10: Circular Cheeks used to make the arc of the pendulum cycloidal

The adjusting screws and the empty slot on the centre line of the cheeks are for fitting them onto a lathe in order to adjust the radius of the cheeks and in doing so, the circular error of the pendulum. The usefulness of this adjustment will be seen later. It should be noted that the aim of these cheeks is not to eradicate circular error, but to adjust it.

## What causes a Pendulum to Change Rate?

### Environmental Conditions

In order for a pendulum to keep perfect time, it must be kept in a totally stable environment. That is, the pressure, temperature, gravity and energy input to the pendulum and energy losses from the pendulum must all remain exactly the same.

Many attempts have been made to do this, but in order to keep the environment stable, the pendulum must be mounted on a completely rigid support, in a total vacuum, with a constant energy driving it. It must also have a frictionless suspension and not be exposed to changes in temperature or to light, which also adds energy to the pendulum.

Even gravitational effects caused by the moon and the sun acting on the Earth (The same force which causes the tides), have to be isolated in order to ensure a completely isochronous pendulum. This makes the construction of an isolated pendulum highly impractical and expensive.

It should also be noted however, that even if these effects could all be isolated, the inherent problems which cause the periodic time to deviate would still not be eliminated. In fact, if a pendulum was designed to operate in total isolation and some small outside effect such as a stray vibration caused the pendulum’s arc to change, there would be no restoring force to bring it back to its design conditions. Because of this, the outside influence would cause the pendulum to compound errors instead of losing them over time.

It is for this reason that clocks with pendulums designed to operate in highly controlled environments can often be as inaccurate as those which are left to operate in normal changing conditions.

The changes in conditions to which a pendulum will be exposed are as follows:

### Temperature Change

Changes in temperature will cause the parts of the pendulum to expand or contract. The effect the change in temperature has on the pendulum shape is the subject under investigation in this project. The length of the pendulum must be exactly regulated as in order to maintain an accuracy of 1 second in 100 days, which has been the target accuracy for mechanical regulators since the 1700’s, the length of the pendulum must not change by more than 230nm[1].

The temperature also affects the density and therefore the viscosity of the air through which the pendulum travels, reducing the viscosity as the temperature rises. Also, if oil is used in the clock mechanism, this will change viscosity and affect the motion of the pendulum too. It is partly for this reason that virtually all regulators run without oil.

### Pressure

An increase in pressure will cause the density of the air to rise, increasing the air resistance on the pendulum. This causes the arc of the pendulum to drop, and since it takes more energy to push it through the thicker air, the clock rate will also drop. Air pressure changes also increase or decrease the weight of the pendulum by a small amount, as the air surrounding the pendulum will act to float it, thereby reducing the mg⋅Sinθ restoring force on the pendulum as in Section 2.7. This also causes the rate of the clock to drop.

### Gravity

The force exerted due to gravity changes with distance from the centre of the Earth, as well as with the relative positions of the sun and moon. However, assuming the clock is left in the one place, the cyclic changes due to the sun and moon are never enough to cause an error of 1 second in 100 days. An increase in gravitational force causes the clock to run faster, with a decrease having the opposite effect (See Section 2.7).

### Energy input

If a pendulum is set swinging in air, without an outside energy source to drive it, it will eventually stop due to air and mechanical friction. A graph of this can be seen in Figure 5.3.19 from the experimental work in this project.

Increasing the energy driving the pendulum will cause the pendulum to swing through a wider arc. From the previous discussion on circular error in section 3.1, this should cause the clock rate to drop. However, the actual effect of an increase in energy input on most pendulums is to make the clock run faster, as the escapement has too much control over the pendulum motion.

This effect can be minimised by making the crutch, which drives the pendulum as in Figure 9.1.26, as short as possible in relation to the pendulum length, increasing the mass of the bob or reducing the recoil forces from the escapement. Another method is to increase the arc angle of the pendulum and use circular error to make the clock run more slowly, while the escapement is trying to drive the clock more quickly, thus making both errors cancel eachother out. It is this method of dealing with errors which has been investigated as a solution to the problem of the effect the environment has on pendulums.

If oil is used in the clock mechanism, as it collects grit and dust, more energy will be required to keep the clock running. This also means that a clock using oil will have to be recalibrated every time it is cleaned, as well as while it is running to compensate for changes in the oil. This is the second reason oiled mechanisms are not used.

### Conclusion

What is needed is a system which uses its environment as a source of self regulation, rather than a system which must be isolated from the environment to work properly. Such a system was developed, built and refined by a Yorkshire man named John Harrison (1693-1776) as a result of devoting over sixty years of his life to the pursuit of more accurate timekeeping during the 1700's.

Harrison's system uses the resistance of the pendulum as it moves through the air as a sink for energy applied to the pendulum by the escapement mechanism. To do this, he used a pendulum arc of 12º, contrary to accepted pendulum theory (Section 2.7). However, once the effect of circular error can be controlled, as in Figure 3.1.10, this has several benefits in terms of the precision of the clock.

The reason this arrangement improves accuracy is that as the pendulum swings through a large arc, it has far more kinetic energy than a small arc pendulum of the same mass, but it also loses more energy through air friction than an equivalent small arc pendulum too. It is known that a light pendulum which has a large energy throughput will remember a disturbance for far less time than a heavy pendulum with a short arc, and this is the part of the reason Harrison used a large arc in his regulators.

The large arc also causes more air to wash over the pendulum, allowing it to reach thermal equilibrium more quickly, making the temperature compensation in the pendulum more effective. As can be seen in Table 1, circular error will counteract the effects of all the variables previously discussed, except for temperature fluctuations. It is for this reason that the temperature compensation is necessary.

Table 1: Effects of Different Environmental Variables on Pendulum

| Variable | Change in Variable | Individual Effects of this Change | Effect of this Change on the clock Rate | Effect of Circular Deviation on Clock Rate |
| --- | --- | --- | --- | --- |
| Gravity |   | Pendulum Heavier Pendulum Lighter |   | 0 0 |
| Temperature |   | Rod Expands Air Thinner Rod Contracts Air Thicker |  Small   Small  | 0  0  |
| Barometric Pressure |   | Air Thicker Pendulum Lighter Air Thinner Pendulum Heavier |  Small   Small  |  0  0 |
| Energy Input |   | Pendulum Faster Pendulum Slower |   |   |
| Overcompensation for Temperature |   | Temperature Rises Temperature Falls |   |   |

Reference [1].

# Experimental Work

## Introduction and Aims

The purpose of the experimental work in this project was to examine the effects of temperature changes on the timekeeping of a pendulum. It was originally planned that the compensated pendulum designed by Dr. Richard Stephen would be used, but no convenient source for the carbon fibre rod used in this pendulum could be found.

It is also likely that any carbon fibre rod sourced would have a different coefficient of expansion to the one used in the compensated pendulum analysed, and since there was no way of measuring the coefficient of thermal expansion available, this would make the construction of a correctly compensated pendulum unfeasible.

The staff in the engineering workshops were reluctant to commit to making the pendulum to the precision needed in order that its compensation would work correctly, as there were many other projects in progress at the same time, and constructing the pendulum would have taken up a disproportionate amount of workshop time.

If the compensated pendulum was used, there would also have been a problem with measuring the effect of the temperature change, as it should theoretically be zero. The compensated pendulum uses a spring suspension, making it far more susceptible to higher modes of vibration in the pendulum itself, as well as twisting, all of which would add to the experimental error, rendering any measurements made unreliable at best. In fact, in order to get reliable results from experiments using this pendulum, it would be necessary to fit an escapement mechanism to it and measure the periodic time over several days, while the temperature was also regulated.

It was concluded from this that it would be far more convenient if a pendulum test rig was designed and built specifically for the experiments required. The criteria for this design are in Section 4.2.

The aims of the experiments were, first of all, to establish the need for temperature compensation, and secondly to compare the theoretically calculated periodic time of the pendulum with the actual measured periodic time and see whether the measured change due to temperature was the same as the theoretically calculated change. The effect of air resistance on the pendulum was also investigated as this is the force which causes the pendulum arc to drop during the experiments performed.

In order to perform the experiments necessary, a pendulum of known dimensions and material was needed, as well as a means of measuring the periodic time and regulating the temperature at which the pendulum was held. The following sections describe the experimental equipment and procedure used for the experiments.

## Design Criteria For Test Pendulum

The most important aspect of the pendulum design was that it should fit into the oven to which it was allocated for the experimental work. If there was no way to regulate the temperature to which the pendulum is exposed, the experiments could not have been performed. This criterion was the force that drove the limiting dimensions for the pendulum length and the size of the supporting frame. The approximate dimensions of the inside of the oven were 690mm high x 590mm wide x 350mm Deep.

The test pendulum was designed to be easy to build. This allowed it to be constructed using basic lathes, a milling machine and a pillar drill. All this equipment was readily available and the simple design allowed the pendulum to be built over a two-week period. This design would also have allowed the pendulum to be made by students if the machine room staff were too busy.

The simple shapes of the parts from which the pendulum is made allow easy mass moment of area calculations to be performed.

The parts of the pendulum are also all made from the same material. This allows it to be analysed as a solid assembly, rather than as separate parts as would be necessary for pendulums using a mix of materials, such as the compensated pendulum.

The pendulum was designed with robustness in mind, as it was known that it would have to be transported from where it was made to set up the experimental equipment as well as for the experiments to be performed. Also, since the area where the pendulum was to be stored was left open most of the time, there was always the possibility that it would be tampered with. This is part of the reason that the calibration procedure for the pendulum was made as simple as possible, it was also done to remove changes in calibration as a source of error since the pendulum calibration can be checked quickly every time it is used.

The main dimensions of the pendulum can be easily measured and confirmed. This allows the correct values to be put into the theoretical calculations for the pendulum period.

A proximity switch was chosen to measure the period of the experimental pendulum for the following reasons;

- It was easy to source a proximity switch, as there was a supply of them readily available in the University.

- The proximity switch used is designed to operate at temperatures up to 70°C (Farnell Catalogue), which is well within the temperature range of the planned experiments.

- The rise time for the signal from the switch was measured using a HP Oscilloscope with a clock speed of 150MHz, giving a sub 7ns resolution, to be under 1.74 microseconds. This means that the switch reaction time will not have a significant effect on the measurements taken using the switch.

- Steel was the material chosen for the pendulum, so a proximity switch is the ideal method to use to monitor its movement, as the pendulum rod can be detected directly. This means that no modifications have to be made to the rod to allow the period to be measured.

- The cylindrical pendulum rod allows the proximity switch to be highly selective in its registering of the rod passing, as the switch will only pick up the nearest edge of the rod instead of the whole width of it. The sensitivity can be adjusted by sliding the pendulum forwards or backwards on the knife-edge.

Since there was nothing to drive the pendulum, the emphasis of the design was to remove as much friction as possible from the system because, during the course of the experiment, the arc through which the pendulum swings reduces, causing the periodic time of the pendulum to fall also. A knife-edge pendulum suspension was chosen as the easiest way to do this. It is not only easy to make and uses very little energy in operation, but it is also far less susceptible to higher modes of vibration such as twisting and forwards and backwards vibration, which cause considerable problems if a suspension spring is used.

## Equipment Used

K Type Thermocouple Amplifier (See Section 9.3 for calibration details)

Blackstar Apollo 10 Universal Counter Timer with 2MHz and 100MHz clocks.

IMEX NE 481 Dual Voltage/Current Supply

Potential Divider Circuit with Diode to Prevent Back e.m.f.

Schönbook Electronic PNP, 10 to 35V DC Proximity Switch, Serial No. ILII214

Gallenkamp size three Oven BS. Model CV-160, 13A 250V. Approximate Internal Dimensions: 690H x 590W x 350D. Thermostat Calibrated to ±5º.

Intel Pentium 133 with 48Mb RAM. Running LabVIEW Version 5.1.1 Software and with the PC-1200AI LabVIEW Interface Card Installed.

Radionics Heat gun 500S 1500W

HP Oscilloscope: Model Number: HP54602B with 150MHz Clock (Used for testing the proximity switch).

## Equipment Set up

The equipment was set up as in Figure 4.4.11, Figure 4.4.12, Figure 4.4.13 and Figure 4.4.14.

| Figure 4.4.11: The Experimental Pendulum set up in the Oven. | Figure 4.4.12: Close-up of Pendulum Bob, showing Proximity Switch, threaded bar and thermocouple. |
| --- | --- |
| Figure 4.4.13: Equipment used to log Temperature and Periodic Time. | Figure 4.4.14: Power Supply, Timer/Counter, Thermocouple Reader and LabVIEW Connector Board. |

The wiring diagram for the electronic connections is shown in Figure 4.4.15.

Figure 4.4.15: Wiring Diagram for Experimental Equipment.

## Calibration Procedure for Test Pendulum

It was confirmed that the pendulum rig frame was level by using a setsquare and spirit level to check that the front left leg was level and the left hand upright of the frame was vertical. The levelling procedure is shown in Figure 4.5.16. Pieces of cardboard were inserted under the legs to level the frame if it was not level to start with. After each subsequent stage in the calibration, the pendulum was again confirmed to be level.

Figure 4.5.16: Confirming that the pendulum frame is Level.

Next the 20V supply to the proximity switch was turned on and it was confirmed that the proximity switch registered the pendulum rod passing its tip. If the switch did not operate, it was moved towards the pendulum rod by loosening the nuts holding it to the plate adjusting it forwards and tightening the nuts again. Fine-tuning can then be performed by sliding the pendulum towards or away from the switch on its knife-edge mounting.

The centring of the switch was also checked by ensuring that when the pendulum came to rest in the centre of its swing, the switch was activated. A visual check was also performed to ensure that the pendulum spent approximately the same amount of time at each side of the proximity switch when it was sent through a small arc. If this was not the case, the centring of the proximity switch was adjusted by gently tapping the plate it is mounted in, thus causing it to move slightly in the required direction. The play between the holes drilled for the bolts in the plate and the bolts themselves allows this movement to take place.

As has been demonstrated previously in Section 2.7, keeping the half arc of the pendulum below 2° will almost eliminate the effects of circular error on the pendulum, thus rendering it effectively isochronous during the course of the experiment. The arc angle can be measured using a ruler fitted to the pendulum frame to measure the displacement of the tip of the pendulum rod.

To calculate the displacement of the end of the pendulum rod required to keep the pendulum half arc angle under 2°, basic trigonometry can be used as follows:

Actual Rod Length Measured from knife-edge pivot to free end:	0.5739 m

Half Arc Angle required	2°.

Displacement of end of rod with 2° half arc angle:

= 0.5739  Sin (2°) = 0.020028821m. 	Equation 34

Therefore, a displacement of 20mm will give a half arc angle under 2°.

The threaded rod at the left hand side of the pendulum frame was adjusted to prevent the pendulum from moving more than 20mm from its central position, which was measured on the ruler at the base of the pendulum. The threaded rod makes the arc angle for the experiments repeatable.

## Experimental Procedure

The following experiments were performed on Saturday 24th February between 12:00 and 20:00.

The equipment was set up and calibrated as described in sections 4.4 and 4.5, with the pendulum rig installed in the oven. The thermocouple was positioned to measure the temperature of the air near the pendulum as shown in Figure 4.4.12.

The oven was then turned on and set to the temperature required. (As the oven is regulated to a minimum of 40°C, the first tests were performed at room temperature, without turning the oven on.)

The pendulum was then made to oscillate by moving the pendulum to the left until the bob touched the tip of the threaded rod. The pendulum was then released and allowed to swing freely. It was visually examined for excessive forward and reverse vibration and the starting procedure was repeated if this was the case. The oven door was then closed.

The counter/timer and the LabVIEW temperature logging program were started simultaneously. The temperature logger was set to log the temperature every 5 seconds for a 15 minute time period.

The average periodic time reading calculated by the counter timer for every 10 cycles was recorded in a text file on the computer while the LabVIEW program was logging the temperature.

When the temperature logger indicated that the time for the experiment had elapsed, a last periodic time reading was taken, and the time and temperature log files were saved to the hard disk of the computer. This completes one set of experimental readings. This procedure was repeated twice more at room temperature.

The oven was then set to a temperature equal to the average temperature recorded during the previous readings plus 25ºC, i.e. 42ºC. A 30-minute time period was left to allow the air in the oven and the parts of the pendulum to reach thermal equilibrium. The temperature in the oven was measured using the thermocouple and temperature-logging program, over a period of 1 minute to confirm that the temperature in the oven was stable to ±5ºC prior to beginning the elevated temperature experiments.

While oven was running, the door was opened and the pendulum oscillation started. The oven door was gently closed as soon as it had been visually confirmed that there were no significant forwards and backwards vibrations in the pendulum.

After waiting for 1 minute to allow the air temperature in the oven to stabilise, the oven was turned off. A time of 30 seconds was waited for, to allow the oven’s fan to stop and the air currents in the oven to die out.

The timer/counter and the temperature logger were started simultaneously at the end of the 30 seconds. The temperature logger was set to log the temperature every 5 seconds for 14 minutes. The reason for the change in time is that the pendulum will have been oscillating for 1.5 minutes before any readings are taken in the elevated temperature tests, so in order to compare like readings, 1.5 minutes are added to every time reading taken for the elevated temperature tests as can be seen in the results graphed in Figure 5.1.17.

The average for every 10 periods was recorded from the timer counter as before, with the record of periodic time continuing until the temperature logger indicated that the time for the experiment had elapsed.

This procedure was repeated twice more for the elevated temperature, recording the average period measurement over every 10 samples, turning the oven on for 30 minutes between each experiment. The stability of the temperature was confirmed by ensuring that the thermocouple reading remains within ±5°C for at least a minute before starting a new experiment.

## Further Test to Confirm Pendulum Properties

This test was performed on Wednesday 28 March 2001, between 15:00 and 16:30.

Since the readings taken in the previous experiment gave results which were clearly inconsistent with the known behaviour of uncompensated pendulums, see Section 5.1, a second test had to be performed, using a heat gun to heat the pendulum rod directly, in order to confirm that it does expand as its temperature rises.

The pendulum was positioned in an open room and calibrated as in section 4.5. The pendulum was started and left oscillating for 30 seconds to allow it to stabilise. Periodic time readings were then taken for the pendulum over the next minute. The ambient air temperature was measured during the experiment in order to get an average value for the initial temperature of the pendulum.

The pendulum rod was then heated for 3 minutes using a heat gun, giving the rod an estimated temperature of 50°C ± 10ºC, while the knife edge and pendulum bob were only heated by conduction from the pendulum rod. After the 3 minutes were up, the pendulum was started again, and readings for its periodic time were taken over the next 2 minutes, after first allowing 30 seconds for it to stabilise.

The pendulum was then allowed to cool for 20 minutes and its periodic time was measured over 1 minute, again after a 30 second delay for stabilisation.

# Results and Discussion

## Experiments Involving the Oven

The results of the experiments using the oven are shown in the following graphs and tables. The raw data from which the graphs were produced can be found in the Appendix in Sections 9.20 and 9.21.

Figure 5.1.17 displays all the information collected during the course of the experiments described in Section 4.6. As can be seen from the results, there is clearly something wrong with the conditions under which this experiment was performed. As an increase in periodic time was expected, when a decrease is what actually occurred. It should also be noted that the periodic time measured was for the half period of the pendulum, as the proximity switch will register the pendulum rod passing it twice for every full oscillation of the pendulum.

The elevated temperature period readings also fluctuate enormously, indicating that there is something causing the pendulum to deviate which was not present for the room temperature readings. The only difference in conditions between the two sets of experiments was that the oven had been turned on prior to the elevated temperature readings being taken, causing air currents to be produced in the oven as it was fan assisted.

Fluctuations in the temperature readings can also be seen for both high and low temperature experiments, indicating that these fluctuations are not due to the oven, but more likely due to the equipment used to measure the temperature.

The LabVIEW software or hardware is thought to be the reason for the temperature measurement problem, as plugging a multimeter into the outlets from the thermocouple amplifier results in a steady measurement, and since some readings taken 5 seconds apart vary by 20ºC, it is not possible that these readings are correct.

However, when the obviously incorrect values have been removed from the data set, the average temperatures calculated using the remaining readings should give an adequate measurement of temperature for the purpose of this experiment.

Figure 5.1.17: Results of the Experiments performed using the oven (The Periodic time measured is half the actual Period)

Table 2: Summary of Experimental Results Compared with Theoretical and ProEngineer Calculations.

| Tests at Room Temperature | Average Temperature | Periodic times from Experimental Results | Average Period (Sample is from 100 to 600 seconds in each Data Set) | Theoretical Results Calculated for a 25°C Temperature Change |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Manual Calculation | ProEngineer Calculation |
| 1 | 17.25°C | 1413412.80 μs | 1.413412797 s |  |  |
| 2 | 17.28°C | 1413086.46 μs | 1.413086458 s |  |  |
| 3 | 17.23°C | 1413193.11 μs | 1.413193114 s |  |  |
| Average | 17.26°C | 1413230.79 μs | 1.413230789 s | 1.405769274 s | 1.407497449 s |
| Elevated Temperature Tests |  |  |  |  |  |
| 1 | 39.87°C | 1412715.91 μs | 1.412715908 s |  |  |
| 2 | 39.65°C | 1412741.99 μs | 1.41274199 s |  |  |
| 3 | 42.67°C | 1412694.34 μs | 1.412694336 s |  |  |
| Average | 40.73°C | 1412717.41 μs | 1.412717411 s | 1.405962554 s | 1.40769097 s |
| Change | 23.47°C | -513.38 μs | -0.000513378 s | 0.00019328 s | 0.000193521 s |
| % Change | 136.045608 % | -0.036326546 % | -0.036326546 % | 0.0137491 % | 0.013749317 % |

The average temperature readings were calculated after removing obviously incorrect Temperature readings i.e. Those under 14ºC for room temperature and those under 35ºC for the high temperature readings. Between 8 and 16% of the temperatures read over the period of the test were rejected in this way. It should be noted that in almost all cases, figures outside the tolerances given occurred singly, with it being unusual to have two consecutive measurements rejected. The ProEngineer calculation is for a pendulum with the design dimensions. The manual calculation result is for a pendulum of the same dimensions as the actual pendulum tested.

Table 2 shows, the compiled results comparing the experimental results at 17ºC and 41ºC with the theoretically calculated results using the moment of area formulae in Section 9.2, as well as the results from the calculations performed by ProEngineer on a CAD model of the pendulum.

Table 3: Comparison of Percentage Differences Between Results

| Values Above Hatched Diagonal Boxes are for Room Temperature Results. Values below Diagonal are for Elevated Temperature Results. |  |  |  |
| --- | --- | --- | --- |
| % Deviation | Experimental | Manual | ProEngineer |
| Experimental |  | -0.527975709 % | -0.405690316 % |
| Manual | -0.478146393 % |  | 0.122934457 % |
| ProEngineer | -0.355799482 % | 0.122934719 % |  |

Table 3 compares the percentage errors between the different methods used to find the periodic time of the pendulum. The correlation between the ProEngineer and the manual calculations was expected to be good, though the ProEngineer model is for a pendulum of exactly the design dimensions as given in Section 9.8, while the manual calculations are for a pendulum of the same dimensions as the test pendulum constructed.

ProEngineer is more accurate in its calculations, as it is dealing with the exact shape of the pendulum, rather than the simplified assembly of regularly shaped parts which was analysed using manual calculations in Section 9.3. However, the results from ProEngineer correlate with the results from the manual calculations to 0.00686% if the same dimensions are used for both sets of calculations, indicating that the assumptions made in the manual calculations are acceptable.

## Results from Experiments Involving the Independent Heating of the Pendulum Rod

The periodic times measured when the pendulum rod was heated independently are graphed in Figure 5.2.18. The ambient air temperature during these experiments was 21°C ± 1°C. The estimated temperature of the pendulum rod after it was heated for 3 minutes was 50°C ± 10°C, though the large tolerance indicates the low level of confidence with which the reading was taken since the temperature was measured by holding the thermocouple against the rod for several seconds.

However, this uncertainty is acceptable for the rod temperature measurement, as the most important aspect of the experiment is that the rod was heated to a significantly higher temperature than for the initial readings and not the value of the change. The aim of this experiment was to confirm that the change in temperature had the correct effect on the periodic time. These results cannot be interpreted as reliable readings and were only taken to show that the trend is correct.

Figure 5.2.18: Results from Heating Pendulum Rod on its Own

It can be seen from Figure 5.2.18 that there is a clear increase in the periodic time of the pendulum after the rod is heated. This effect is also shown to be temperature related as, after the pendulum cooled, the periodic time returned to its initial value.

The reason consecutive periodic time readings vary is mostly due to air currents in the room, as there was a noticeable deviation in the period after people opened the door to enter or leave the room. The pendulum was completely exposed to the air currents in the room for the duration of the experiment, but since there was no major change in these currents while the experiment progressed, the temperature effect on the pendulum could be measured.

## The Modelling of the Periodic Time Decrease as the Experiment Progressed

Figure 5.3.19: A best fit line modelling the change in the period of the pendulum averaged over three experiments as a logarithmic decrement.

The graph in Figure 5.3.19 shows a logarithmic best-fit line overlaid on averaged values for the periodic time of the test pendulum. The period decays because, due to air friction, the pendulum arc drops as the experiment progresses.

The Equation of the line, as well as its R² value, which is the correlation coefficient for the line, are also shown on the graph. The nearer the R² value is to 1, the better the points in the graph match the best-fit line.

Best-fit lines were also calculated for the same data, using linear, polynomial and exponential approximations, but the best correlation was achieved using the logarithmic line. This allows the conclusion that the pendulum period decays in a logarithmic fashion to be reached from the experimental information gathered.

# The Effect of the Changing Mass Moment of Inertia

The formulae for calculating the mass moment of inertia for the two component shapes of the test pendulum (Appendix Section 9.2), indicate that a linear change in temperature and therefore in the dimensions of the part, results in a second order polynomial change in the mass moment of inertia.

However, when calculations for the test pendulum were repeated for several different temperatures, and the mass moments of inertia graphed as in Figure 5.3.20, the actual change in the mass moment of inertia for the pendulum looked to be linear. The change in the periodic time of the pendulum also looked to be linear as in Figure 5.3.21.

Figure 5.3.20: Mass Moment of Inertia changes in the test Pendulum due to temperature changes.

Figure 5.3.21: Variation of Periodic Time with Temperature for the Test Pendulum

The reason for this linearity is due to the small size of the changes in the dimensions of the pendulum. Using the formula for the mass moment of inertia of a cuboid, the following results are obtained.

These calculations are based on the knife edge of the experimental pendulum, though they work for the cylindrical portions and for the parallel axis theorem when it is applied to get the mass moment of inertia about the pivot axis, as the changes in the dimension values will be of the same order. Using the coefficient of thermal expansion for steel, if there is a 10ºC temperature rise, the dimensions of the pendulum will change by a factor of approximately 1×10−4.

From Section 9.2, IG=m(a2+b2)12 for a cuboid. For the knife edge a=b, giving the result that I=ma26 for this part of the model at the initial temperature.

For each ap, the temperature is raised by 10ºC, giving the following results for Ip:

a0=a⇒I0=m6a2a1=a+a×10−4⇒I1=m6(a2+2a×10−4+a×10−8)

The value of Ip+1−Ip, the difference between consecutive mass moment of inertia calculations is δ Ip, this gives:

δ I0=m6(2a×10−4+a×10−8)

a2=a+2a×10−4⇒I2=m6(a2+4a×10−4+4a×10−8)

δ I1=m6(2a×10−4+3a×10−8)

a3=a+3a×10−4⇒I3=m6(a2+6a×10−4+9a×10−8)

δ I2=m6(2a×10−4+5a×10−8)

From these results, it can be seen that the change in mass moment of inertia can be divided into two parts. The first part is what shows on the graph, which is a linear increase of m6(2a×10−4) between every value. The second part is a second order increase, but in order for this to be as large as the linear part of the increase, the temperature would have to rise by 100,000ºC, resulting in the non-linear change being of magnitude m6(1.9999a×10−4).

The result of this is that the change in the periodic time of the pendulum will also be effectively linear for the temperature range at which it was tested. From this it can be concluded that the change in mass moment of inertia can be treated as a linear effect for calculations involving temperature compensation in pendulums, unless very high coefficients of expansion are involved. Therefore, a linear system of compensation is adequate to deal with the effect of the mass moment of inertia change. This is most likely the reason little research has been done into the effect the mass moment of inertia change has on the behaviour of a pendulum, as the changes described in Section 3.2 have been investigated because they have been isolated as definite sources of error in the operation of actual clocks.

## Conclusions

The change that was expected from the experiments involving the oven should be an increase in the periodic time of approximately 193 s. Instead, a decrease in periodic time of approximately 513 s was recorded. The reason for this is that the air currents in the oven after it has been turned on remain for the duration of the elevated temperature experiment.

Therefore, if the effect of temperature change is to be isolated successfully, the pendulum rig must be shielded from outside air currents as these are a more significant source of error than the change in temperature.

The effect of circular error was also evident in the results as shown by the drop in periodic time as the experiment progresses. This drop can be approximated as a logarithmic decrease to a high level of accuracy.

The fluctuations in the measured temperature from the LabVIEW temperature logging program can not be explained either by heat losses from the oven or air currents in the oven as the change in temperature was too large and occurred over too short a time period to allow either of these explanations to be likely.

It is because of this that the most likely cause of this error is in the Thermocouple amplifier or the LabVIEW hardware or software. Since the Analogue-Digital converter on the LabVIEW card is capable of measuring voltages to 4 decimal places and at high speed, if there were small gaps in the output signal from the thermocouple amplifier, LabVIEW would pick these up, but a multimeter, which was the only other means used to measure the output from the thermocouple amplifier, will give correct results as it takes more time to stabilise on a voltage reading.

# The Compensated pendulum

The compensated pendulum that has been analysed was designed and built by Dr. Richard Stephen as part of his 30 day Vienna regulator. The clock is weight driven and runs for 30 days between windings. As a result, it was designed with the reduction of drivetrain friction as the top priority.

In order to allow the clock to remain accurate for the 30 days for which it runs between windings, the pendulum is temperature compensated, which means that it is designed to beat out the same periodic time regardless of the expansion of its parts due to temperature. The purpose of this project is to gauge the effectiveness of this compensation.

Figure 6.1.22: The Compensated Pendulum

The compensation in the pendulum was designed to keep the centre of gravity of the pendulum bob in the same place regardless of temperature. This allows the effective length of the pendulum, that is, the distance from the pendulum pivot to the pendulum's centre of gravity, denoted h in Section 2.8, to remain constant.

Compensation is achieved by making the pendulum rod from two different materials, arranged in the correct ratio so that as one material expands due to the temperature, the second material will expand in the opposite direction or contract in order to cancel out the effect of the first one.

For the pendulum in question, the rod is made from steel and carbon fibre. The steel expands due to temperature increases while the carbon fibre contracts to counteract this expansion. In a pendulum where the bob is supported at its centre of gravity, the ratio of the lengths of the two materials used to make up the pendulum should be in inverse proportion to their coefficients of thermal expansion.

However, in the case of the pendulum being analysed, the pendulum bob is supported at its base, causing the bob to expand upwards as the pendulum is heated. This expansion was accounted for in the design calculations, but these calculations assumed the bob to be symmetrical about a horizontal plane through its centre of gravity, which is not the case in reality. This, as will be seen later is one of the reasons the pendulum is not perfectly compensated.

The pendulum model which was analysed was modelled in three dimensions based on a set of 2D drawings published in the British Horological Journal of August 2000. Initially all the parts were drawn as separate ProEngineer files. These were then imported into an assembly drawing to create the full assembly.

In order to confirm the interpretation of the drawings, image files displaying the assembly from several angles were emailed to Dr. Richard Stephen, the pendulum's designer. He was then able to confirm that the assembly was correct, as well as that the material assigned to each part was correct.

## The construction of the Compensated Pendulum

The full compensated pendulum assembly is made up of 15 separate parts. These are named as in Table 4. Dimensioned Drawings of the Pendulum can be seen in Appendix 9.10.

However, since neither the Mounting or the Split block oscillate with the pendulum, these do not contribute to the overall mass of the pendulum. However, the Split block does contribute to the periodic time since the oscillating length of the pendulum is measured from the bottom edge of the split block, where the suspension spring touches it, to the centre of gravity of the pendulum assembly. This relationship was investigated using Finite Element Analysis in Section 7.3.

Table 4: Components of the Compensated Pendulum

| No | Component | Description |
| --- | --- | --- |
| 1 | Block Pin | Holds Split Block in position in Mounting |
| 2 | Bob | This is a weight hung from the pendulum rod. It gives the pendulum its characteristic behaviour along with the length of the rod |
| 3 | Bob Pin | Prevents bob from turning on pendulum rod. Works in conjunction with Sleeve |
| 4 | CF Rod | Carbon Fibre, Contracts as temperature rises to compensate pendulum |
| 5 | Clamp | Clamps to Rod Top |
| 6 | Clamp Pin | Locates suspension spring in clamp |
| 7 | Mounting | Supports pendulum, connecting it to the rest of the clock |
| 8 | Rating Nut | This supports the bob and allows the pendulum to be calibrated by winding the rating nut up and down |
| 9 | Rivet | Holds suspension spring rigidly to Split Block |
| 10 | Rod Base | Fits to the end of the Carbon Fibre Rod. Threaded for the rating nut and drilled to hold the Bob Pin |
| 11 | Rod Top | Clamps the other end of the suspension spring |
| 12 | Screw | Clamps the suspension spring between Rod Top and Clamp |
| 13 | Sleeve | This is an interference fit into bob and engages with Bob Pin |
| 14 | Split Block | Slit block to hold suspension spring |
| 15 | Spring | Suspension Spring, flexes to allow pendulum to swing |

See Appendix Sections 9.10 and 9.11 for dimensioned drawings of the pendulum parts.

Table 5: Amalgamated Parts List

| No | Component | Description |
| --- | --- | --- |
| 1 | Base Assembly | This is an amalgamation of the Bob Pin, Rating Nut and Rod Base. |
| 2 | Bob | This is the same as the Bob in Table 4. |
| 3 | Carbon Fibre Rod | This is the same as the Carbon Fibre rod in Table 4. |
| 4 | Sleeve | This is the same as the Sleeve in Table 4. |
| 5 | Top Assembly | Amalgamation of the clamp, clamp pin, Rod Top, screw and the part of the suspension spring which is not in the split block. |

## Analysis of the compensated pendulum using ProEngineer and ProMechanica

In order to analyse the pendulum, unnecessary aesthetic features of the parts were deleted. These features were not dimensioned in the drawings being worked from and the removal of these features has a negligible effect on the overall mass properties of the model, but makes its analysis less accurate as the dimensions of all these features need to be known exactly if a completely exact result is required. The deleted features are as follows;

- All screw threads.

- The fillets and ridges around the edges of the rating nut.

- The groove in the screw that clamps the suspension spring into the top of the pendulum rod.

- The grooves cut into the reduced diameters at the ends of the steel parts of the pendulum rod to glue the carbon fibre rod in position more securely.

See Appendix Section 9.10 and 9.11 for the details of the original assembly drawings as compared to the drawings of the amalgamated parts.

Since no dimensions were given for the rating nut, the only requirement for the design of this part was that it should be large enough to do its job and not heavy enough in comparison to the bob that it has a significant effect on the calculated compensation. The rating nut drawn was based on the photographs of the finished clock published by the British Horological Institute.

To simplify the analysis, as well as increase the speed at which it can be performed, parts which were connected rigidly together and made from the same material were replaced by a single part, as the overall effect of a temperature change on a rigidly connected set of parts of the same material is exactly the same as on a single solid part the same shape.

In order to find the new dimensions for the pendulum, the formula LF=LI(1+ΔT⋅c) from Section 2.9 was used. The drawings of the amalgamated parts were copied and the dimensions of the parts in the new files were changed to the dimensions calculated for after a 25ºC temperature rise.

ProEngineer was then used to calculate the mass properties of the initial parts and deformed parts. This information can be seen in Section 9.11 of the Appendix. The new volume of the deformed part was also calculated in ProEngineer. Since the mass of each part will remain constant, the change in density due to each part's expansion was then calculated as in Table 7.

Once the new density was calculated for the deformed parts, it was put into the ProEngineer model as a material property, thus allowing the mass properties for the deformed part to be calculated correctly. The accuracy of the new density calculations can be checked by comparing the mass derived by ProEngineer for each deformed part with the mass of the original part calculated using the density at room temperature.

The amalgamated parts were then assembled into the full pendulum, assembling the initial and deformed parts in separate drawings and calculating the mass properties for the initial and deformed pendulum assemblies using ProEngineer. These results can be seen in Table 6 and Section 9.13.

## Finite Element Analysis of the Split block and Suspension Spring

Since the block pin, the rivet and the suspension spring are made of steel and the split block is made from brass, a temperature change will cause stresses to build up between these parts. Trying to predict this behaviour using manual calculations would be extremely difficult, as the contact areas are curved, causing a variable stress distribution where the pin and rivet interact with the block.

Figure 7.3.23: The Finite Element Analysis Model

Since the benchmark testing in Section 9.16 gave acceptable results, Finite Element Analysis could then be confidently applied to the problem of the interaction of the split block and the suspension spring.

A Finite Element Analysis model was built as shown in Figure 7.3.23, modelling the suspension spring as a plate element, rigidly linked to the rivet and block pin, which are modelled in 3D, with the split block modelled as a solid block with holes for the pin and rivet, but no slit for the suspension spring, as it was not necessary due to the spring being modelled as a 2D part, with its thickness set as a property.

The model was constrained as shown in the diagram in Figure 7.3.23, with the spring corners constrained to only allow expansion in the XY plane, and the surfaces of the block prevented from turning.

Contact areas were set up between the curved surfaces of the pin and rivet, and the surfaces of the holes made for them in the block. A contact area, basically, is where ProMechanica makes elements which link surfaces which are in contact, in order to model how they interact as the model is loaded. Without the contact areas set up, the volumes would pass through eachother without interacting.

Note also that the rivet protrudes beyond the surface of the split block. The reason for this is that ProMechanica will automatically assume any curves or points which coincide to be rigidly linked. This means that contact analysis could not be used for the model as the rivet and block would already be linked rigidly.

In order to make the cross sectional area of the split block the same as for the actual part, the thickness of the block was reduced by 0.1mm, as this is the thickness of the slot which was cut into the block in the actual pendulum.

A measure was set for the Y displacement of a point on the bottom corner of the block, relative to the co-ordinate system at the bottom corner of the spring. A temperature change will cause the split block to move over the suspension spring, reducing the free length of the suspension spring, but the expansion of the suspension spring will act against this, to lengthen the pendulum. This is shown in Figure 7.4.25.

## Results

The results shown in Figure 7.4.24 and Figure 7.4.25 indicate that the Finite Element Analysis model is deforming as expected. It was known that the brass block would expand relative to the other parts as it has a higher coefficient of expansion than the steel parts, causing the diameters of the holes in the block to increase relative to the diameters of the pin and rivet.

The suspension spring is pulling the pin and rivet together, causing spaces to open up at the top of the hole for the pin and at the bottom of the rivet hole. The spaces shown in Figure 7.4.24 and Figure 7.4.25 are greatly exaggerated due to the result being scaled in order to make the change visible. The end of the split block was found to displace by 1.8370089819655×10-3 mm relative to the bottom of the spring, indicating a net increase in the length of the pendulum, though this increase would have been 2.75×10-3 mm if the edge of the block did not move relative to the suspension spring.

Figure 7.4.24: A plane cut through the deformed model, showing stress

Figure 7.4.25: A contour plot showing the displacements of the different parts of the model. The thin lines shown the displacement of the spring.

## Results for Compensated Pendulum

Table 6: Mass Properties of the Compensated Pendulum Assembly before and after the Temperature Change as calculated by ProEngineer.

|  | Original |  | Deformed |
| --- | --- | --- | --- |
| Volume | 98613.393 mm³ |  | 98740.203 mm³ |
| Surface Area | 38407.444 mm² |  | 38429.187 mm² |
| Average Density | 7.86526E-09 Mg/mm³ |  | 7.85516E-09 Mg/mm³ |
| Mass | 0.00077562 Mg |  | 0.000775621 Mg |
| Centre of Gravity Distance | -552.31794 mm |  | -552.2966 mm |
| Inertia about top of Spring | 240.44349 Mg·mm² |  | 240.42605 Mg·mm² |
| Periodic Time | 1.503166198 s |  | 1.503139394 s |
| Change in Period: | -26.80398096 µs |  |  |
| Pendulum Cumulative Error, Assuming Temperature Remains 25ºC Above the Calibration Temperature |  |  |  |
| Maintained over a Day = |  | -1.5406848 Sec/Day |  |
| Maintained over a Month = |  | -46.2205 Sec/Month |  |

These results show that the change in period for a temperature change of 25ºC is about -27µs per period. This means that as the temperature rises, the clock will tend to run faster rather than more slowly as is the case for the experimental pendulum. This is due to the effect of the sleeve, the rating nut and the split block which are not accounted for in the compensation calculations, as well as the extra length in the base of the pendulum rod to allow the rating nut to be adjusted.

## Sources of Error

### Calculation Error

ProMechanica will output displacement values to 14 significant digits, though rounding error contributes to the lower order decimal places as can be seen in the benchmark analysis 9.18.1, 9.18.2 and 9.18.3.

ProEngineer will also take dimensional inputs to 14 significant digits, though it usually rounds these to 6 places when displaying them. The calculation of the volume by ProEngineer is given to 8 significant places, as is centre of gravity distance, which may have some effect on the accuracy of the calculation. The density change calculated using these volumes gives a figure that has less then 8 significant places, so the error is more due to rounding in the volume calculation rather than in the density calculation, which was performed using Microsoft Excel to 15 significant places.

Also, since the Carbon fibre rod is a composite, it cannot be assumed that its coefficient of expansion is the same in all directions as was done for this analysis. This will be a source of error, though, since the property of most interest is the longitudinal expansion of the rod, and this was measured experimentally by Richard Stephen [3], variations in the expansion of the rod in other directions will have far less effect on the overall pendulum behaviour than the length change.

### Experimental Error

The dimensions used for the experimental pendulum in the analysis of the significance of the moment of inertia are as in the drawings. The actual values deviate significantly from these, particularly the dimensions of the bob see dimensions used in calculations in Section 9.3.1.

However, the change in periodic time is quite similar overall. The calculations shown in Table 2 are for the measured dimensions of the bob, taken at approximately 21°C ± 1°C.

Circular error causes the pendulum's period to drop as the experiment progresses. This is not really a problem as the pendulum half arc is held to under 2 degrees as recommended in 3.1. In the graph showing the results, the trend of a decreasing period as time passes can be seen Figure 5.3.19. The circular error effect gives the graphs a logarithmic decrement, but does not prevent them from being compared to each other overall.

The pendulum may have lost energy from its swing more quickly than it should have due to the cardboard used to level the frame of the rig damping the pendulum vibrations. Errors in one set of experiments performed by Robert Matthys were caused by the roof beam he was hanging a pendulum from flexing[5]. Clocks using the same length pendulum have also been noticed to interfere with eachother for the same reason[3].

The pendulum is not compensated for pressure changes. This should not cause significant problems as each set of experiments was run over one day and the duration of each experiment was kept to approximately 15 minutes each time. This will totally eliminate the significance of gravitational variation, though this is negligible even in precision regulators[1].

The counter timer used to measure the period of the pendulum has no logging capability. This means that it was necessary to manually type the 8 digit average of each 10 periods, approximately every 7 seconds, while the computer logged the temperature automatically every 5 seconds. This leaves potential for errors in the transcription of readings from the counter timer to the computer during the experiment, though the 7 seconds was more than enough to allow the period to be typed and verified for each measurement.

Forwards and backwards vibration in the pendulum may affect the periodic time. However, after the first two or three cycles there is no visible vibration in the pendulum except for the periodic oscillation being measured. If the pendulum is started carefully, even the initial vibrations can't be seen or felt by touching the frame supporting the pendulum, so this source of error is unlikely to be significant.

Though the design of the experimental pendulum stated that it should be made entirely from the same material, this does not eliminate the possibility that parts are expanding at different rates. Since the thermal expansion coefficient is not usually considered an important property, metal producers do not control it very tightly. This means that there will almost certainly be variation from part to part in the test pendulum. There can also be significant variations when the heat treatment is changed[3]. The significance of this error cannot be quantified without measuring the expansion coefficient for each part of the test pendulum and no facilities were available to perform this task.

In order to start the pendulum, the door of the oven had to be opened each time. This causes significant heat loss from the air around the pendulum and the pendulum itself, but this was reduced by opening the door for as short a time as possible (Under 5 seconds), and leaving the oven turned on for a minute after the pendulum was started so the temperature could stabilise again.

Another possible source of error is that the pendulum was not stood on a completely level surface. This was virtually eliminated by the use of a spirit level to ensure that the pendulum  was hanging vertically before the experiment started. The pendulum support frame was not moved during the course of the experiments, which means that all readings taken will have the same error due to the rig being slightly off level.

From the variation in temperature results it is thought that the thermocouple amplifier is the most likely a source of error in these measurements. More details of this are given in Section 9.6.

There is also error due to the thermocouple measuring the air temperature around the pendulum rather than the actual temperature of the pendulum. Ideally, thermocouples should be fitted to holes in several parts of the pendulum, allowing the temperature gradients throughout the pendulum to be measured. If the thermocouple was fitted to the pendulum, however, the spring stiffness and hysteresis in the wiring connecting it to the thermocouple reader will cause the pendulum to lose more energy with each swing than it is losing due to air resistance, reducing the possible duration of each test.

Error due to air currents around the pendulum was kept to a minimum as the oven, which is fan assisted, will be switched off while readings are being taken. Because the temperature is being logged, the temperature variation in the oven will be known as the experiment progresses, allowing compensated readings to be generated if the temperature drop is significant. However, the experiment does make the assumption that the air currents in the oven die down completely after the oven is turned off. The actual results in Section 5.0, indicate that this was most certainly not the case.

Also, in a British Horological Journal article[5], an investigation was done into the effects of changing the size of the clock case on the timekeeping of a pendulum. The results of this indicated that, first of all, a pendulum of the size used in the practical experimentation for this report was more susceptible to air currents than a larger pendulum or a spherical bob, as the mass to surface area ratio for the small pendulum is lower, therefore it will be affected more by the air surrounding it.

In the experiments described in the British Horological Journal, the nearer the walls of the case were to the pendulum, the more they affected it, due to the pendulum itself producing air currents as it oscillated. In the case of the experiments performed in this project, however, the air currents were already present while the high temperature readings were being taken.

# References

- Burgess, Martin: ‘The Scandalous Neglect of Harrison’s Regulator Science’ (From ‘The Quest for Longitude: The Proceeding of the Longitude Symposium, Harvard University, Cambridge Massachusetts, November 4-6 1993, p255-278).

- Holman, J.P.: Experimental Methods For Engineers, 5th Ed. 1989. p.299 to 313.

- http://www.bhi.co.uk (The British Horological Institute Website)

- National Instruments: LabVIEW user Manual and http://www.ni.com, the National Instruments Website LabVIEW help pages.

- Matthys, Robert: The Clock Case and the Pendulum, British Horological Journal August 1999, p263-266

- Meriam, J.L. and Kraige, L.G., Engineering Mechanics Volume 1, Statics 1993, p241-245,

- Meriam, J.L. and Kraige, L.G., Engineering Mechanics Volume 2, Dynamics 1993, p594-597, p707, p657-661

- Murphy O.: Fundamental Applied Maths, Folens 1986, p301-334

- Roark and Young 5th Edition, Material Properties.

- Stephen, Richard: ‘A 30-day Vienna Regulator, 1’ British Horological Journal March 2000 p100-102.

- Stephen, Richard: ‘A 30-day Vienna Regulator, 6’ British Horological Journal, August 2000 p271-273.

## Recommendations for Further Work

If further experiments are to be performed on the test pendulum, then some system to screen it from outside air currents must be built. Building an enclosure for the pendulum which has its own heating system would allow this error to be removed.

A levelling system using threaded bars or bolts put through the legs of the pendulum rig frame would give more accurate levelling control, as well as less vibration damping than the cardboard used for the previous experiments.

Fitting an escapement mechanism to the pendulum would allow it to be tested over a longer time period. The escapement should be weight driven, as this is the most likely arrangement to give a constant driving force to the escape wheel, as well as being easier to implement than a spring arrangement, which would probably need a remontoire See Section 9.1.

Converting the pendulum to use a suspension spring is not recommended, unless an escapement is driving the pendulum and it is shielded from outside air currents already. An arrangement such as this, with circular cheeks fitted to the suspension spring, would allow the use of circular error to compensate for variations in arc angle to be investigated.

Designing a system to allow the pendulum to be started remotely would make the experiments more accurate, as the oven would not have to be opened in order to start the pendulum. However, this is not worth doing until error due to the air currents in the oven has been eliminated, or an alternative means of heating the pendulum has been found.

The pendulum can be redesigned to have a variable mass moment of inertia. This can be done through the use of interchangeable bobs of the same size made from different materials. Another, more interesting option would be to use a bob whose mass moment of inertia can be changed by making it from a stack of washers of different materials, which slide onto the pendulum rod, the order of which can be changed to maintain the same centre of gravity, but change the mass moment of inertia.

This would allow mass moment of inertia changes to be investigated separately to the temperature effects, as the movement of the bob’s centre of gravity would be the same regardless of the order the washers were fitted in, once the temperature change is the same. This would allow the same equipment to be used to examine both mass moment of inertia changes and temperature changes.

# Appendices

## Pendulum Nomenclature

Figure 9.1.26: Clock and Pendulum Nomenclature

Anchor: This is the anchor shaped plate the pallets are connected to.

Back Cock: This is the part of the clock which the pendulum mounts onto. The Mounting part of the compensated pendulum serves this function.

Collet: This is the flange on the pallet arbor that the Pallets are connected to.

Crutch: This connects the pendulum to the escapement mechanism.

Crutch Pin: This connects the crutch to the pendulum rod.

Discharge Corner: This is the tip of the entrance or exit pallet tooth.

Entrance Pallet: This is the first tooth on the anchor. It engages with the escape wheel.

Escape Wheel: This is a gear wheel driven clockwise in the preceding diagram. The turning of the escape wheel is regulated by the pallets gripping the teeth of the wheel.

Exit Pallet: This is the second tooth on the escapement mechanism which hooks into the escape wheel. It is further around the escape wheel in the direction the escape wheel travels than the Entrance Pallet

Face: This is the surface of the pallets, along which the teeth of the escape wheel slide as the escapement operates.

Pallet Arbor: This is the shaft the Escapement Pallets are fitted to. The crutch is also fitted to the pallet arbor.

Pendulum Bob: This is a weight hung from the end of the pendulum rod. It is usually very heavy in comparison to the other oscillating parts of the pendulum, allowing the distance from the pendulum pivot to the centre of gravity of the bob to be approximated as the effective length h of the pendulum (See Section 2.8).

Pendulum Rod: This connects the spring and the bob together. It should be as rigid as possible in order to prevent the pendulum assembly vibrating at higher frequencies than the pendulum oscillation frequency, and in doing so affecting timekeeping.

Rating Nut: This screws up and down on the end of the pendulum rod and supports the bob. Adjusting the rating nut changes the effective length of the pendulum, allowing its periodic time to be adjusted.

Remontoire: This is a mechanism designed to deliver a constant torque to the escape wheel to eliminate escapement variation. It works by storing energy from the weights or springs used to drive the clock, and delivering it to the escape wheel in a controlled manner.

Suspension Spring: This is a flat plate made from spring steel, which flexes to allow the pendulum to oscillate.

## Formulae used to Calculate the Mass Moment of Inertia for Standard shapes.

For a Cuboid[7]:

IXX=m(a2+b2)12

Figure 9.2.27: Cuboid Mass Moment of Inertia

For a Cylinder[7]:

IXX=14mr2+112ml2

Figure 9.2.28: Cylinder Mass Moment of Inertia

## Theoretical Calculations for the Test Pendulum

### Calculations for the Test Pendulum using Actual Dimensions

| (Figures in Bold Are the Driving Dimensions required to define the Shapes. Dimensions used were Measured from the Test Pendulum on 21/3/2001. The Temperature was 21 ± 1ºC) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Standard Temperature: |  |  |  | 25 °C | Temperature Rise |  | Only accounting for length change instead of MOI Change |
|  | Knife Edge: |  |  |  | Knife Edge: |  |  |  |
|  | Assumed solid Square Block. |  |  |  | Assumed solid Square Block. |  |  | Knife Edge: |
|  | Diagonal | 0.0306 m |  |  | Diagonal | 0.030608415 m |  | 0.030608415 m |
|  | L | 0.021637468 m |  |  | L | 0.021643418 m |  | 0.021643418 m |
|  | W | 0.021637468 m |  |  | W | 0.021643418 m |  | 0.021643418 m |
|  | H | 0.0601 m |  |  | H | 0.060116528 m |  | 0.060116528 m |
|  | IG | 1.71892E-05 kgm² |  |  | IG | 1.71986E-05 kgm² |  | 1.71892E-05 kgm² |
|  | d | -0.0153 m |  |  | d | -0.015304208 m |  | -0.015304208 m |
|  | Volume | 2.81376E-05 m³ |  |  | Volume | 2.81608E-05 m³ |  | 2.81608E-05 m³ |
|  | Mass | 0.220289411 kg |  |  | Mass | 0.220289411 kg |  | 0.220289411 kg |
|  | IXX | 6.87567E-05 kgm² |  |  | IXX | 6.87946E-05 kgm² |  | 6.87851E-05 kgm² |
|  | Pendulum Rod: |  |  |  | Pendulum Rod: |  |  | Pendulum Rod: |
|  | Assumed solid Cylinder. |  |  |  | Assumed solid Cylinder. |  |  |  |
|  | L | 0.6091 m |  |  | L | 0.609267503 m |  | 0.609267503 m |
|  | Diameter | 0.01 m |  |  | Diameter | 0.01000275 m |  | 0.01000275 m |
|  | DH | 0.0199 m |  |  | DH | 0.019905473 m |  | 0.019905473 m |
|  | DT | 0.0352 m |  |  | DT | 0.03520968 m |  | 0.03520968 m |
|  | IG | 0.011581599 kgm² |  |  | IG | 0.011587969 kgm² |  | 0.011581599 kgm² |
|  | d | 0.26935 m |  |  | d | 0.269424071 m |  | 0.269424071 m |
|  | Volume | 4.78386E-05 m³ |  |  | Volume | 4.78781E-05 m³ |  | 4.78781E-05 m³ |
|  | Mass | 0.374528416 kg |  |  | Mass | 0.374528416 kg |  | 0.374528416 kg |
|  | IXX | 0.038753419 kgm² |  |  | IXX | 0.038774736 kgm² |  | 0.038768366 kgm² |
|  | Pendulum Bob: |  |  |  | Pendulum Bob: |  |  | Pendulum Bob: |
| Assumed Cylinder with 10mm hole. |  |  |  | Assumed Cylinder with 10mm hole. |  |  |  |  |
|  | L | 0.1011 m |  |  | L | 0.101127803 m |  | 0.101127803 m |
|  | Diameter | 0.05075 m |  |  | Diameter | 0.050763956 m |  | 0.050763956 m |
|  | Volume | 0.000204509 m³ |  |  | Volume | 0.000204678 m³ |  | 0.000204678 m³ |
|  | Mass | 1.60110356 kg |  |  | Mass | 1.60110356 kg |  | 1.60110356 kg |
|  | IG | 0.001621502 kgm² |  |  | IG | 0.001622394 kgm² |  | 0.001621502 kgm² |
|  | Hole L | 0.1011 m |  |  | Hole L | 0.101127803 m |  | 0.101127803 m |
|  | Hole Dia | 0.01 m |  |  | Hole Dia | 0.01000275 m |  | 0.01000275 m |
|  | Hole Vol | -7.94038E-06 m³ |  |  | Hole Vol | -7.94693E-06 m³ |  | -7.94693E-06 m³ |
|  | Hole Mass | -0.062165199 kg |  |  | Hole Mass | -0.062165199 kg |  | -0.062165199 kg |
|  | Hole IG | -5.33388E-05 kgm² |  |  | Hole IG | -5.33682E-05 kgm² |  | -5.33388E-05 kgm² |
|  | Total Vol | 0.000196569 m³ |  |  | Total Vol | 0.000196731 m³ |  | 0.000196731 m³ |
|  | Total Mass | 1.538938361 kg |  |  | Total Mass | 1.538938361 kg |  | 1.538938361 kg |
|  | Total IG | 0.001568163 kgm² |  |  | Total IG | 0.001569026 kgm² |  | 0.001568163 kgm² |
|  | d | 0.5006 m |  |  | d | 0.500737665 m |  | 0.500737665 m |
|  | IXX | 0.38722667 kgm² |  |  | IXX | 0.387439674 kgm² |  | 0.387438812 kgm² |

|  |  | Overall Totals: |  |  |  | Overall Totals: |  |  |  | Overall Totals: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Volume | 0.000272545 m³ |  |  | Volume | 0.00027277 m³ |  |  | 0.00027277 m³ |
|  |  | Mass | 2.133756188 kg |  |  | Mass | 2.133756188 kg |  |  | 2.133756188 kg |
|  |  | IXX | 0.426048846 kgm² |  |  | IXX | 0.426283205 kgm² |  |  | 0.426275962 kgm² |
|  | Periodic Time Calculation: |  |  |  |  | Periodic Time Calculation: |  |  | Periodic Time Calculation: |  |
|  | LEQ |  | 0.406748132 m |  |  | LEQ | 0.406859988 m |  | 0.406859988 m |  |
|  | τ |  | 1.405769274 s |  |  | τ | 1.405962554 s |  | 1.40595061 s |  |
|  | Change in τ |  | 0.00019328 s |  |  |  | 0.000181336 s |  | -6.179710186 % |  |

### Calculations for the Test Pendulum using Theoretical Dimensions

| (Figures in Bold Are the Driving Dimensions required to define the Shapes) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Standard Temperature: |  |  |  | 25 °C | Temperature Rise |  | Not accounting for changes in |
|  | Knife Edge: |  |  |  | Knife Edge: |  |  | Moment of Inertia |
|  | Assumed solid Square Block. |  |  |  | Assumed solid Square Block. |  |  | Knife Edge: |
|  | Diagonal | 0.03 m |  |  | Diagonal | 0.03000825 m |  | 0.03000825 m |
|  | L | 0.021213203 m |  |  | L | 0.021219037 m |  | 0.021219037 m |
|  | W | 0.021213203 m |  |  | W | 0.021219037 m |  | 0.021219037 m |
|  | H | 0.06 m |  |  | H | 0.0600165 m |  | 0.0600165 m |
|  | IG | 1.58537E-08 Mgm² |  |  | IG | 1.58624E-08 Mgm² |  | 1.58537E-08 Mgm² |
|  | d | -0.015 m |  |  | d | -0.015004125 m |  | -0.015004125 m |
|  | Volume | 2.70000E-05 m³ |  |  | Volume | 2.70223E-05 m³ |  | 2.70223E-05 m³ |
|  | Mass | 0.000211383 Mg |  |  | Mass | 0.000211383 Mg |  | 0.000211383 Mg |
|  | IXX | 6.34149E-08 Mgm² |  |  | IXX | 6.34498E-08 Mgm² |  | 6.34411E-08 Mgm² |
|  | Pendulum Rod: |  |  |  | Pendulum Rod: |  |  | Pendulum Rod: |
|  | Assumed solid Cylinder. |  |  |  | Assumed solid Cylinder. |  |  |  |
|  | L | 0.61 m |  |  | L | 0.61016775 m |  | 0.61016775 m |
|  | Diameter | 0.01 m |  |  | Diameter | 0.01000275 m |  | 0.01000275 m |
|  | DH | 0.02 m |  |  | DH | 0.0200055 m |  | 0.0200055 m |
|  | Pin Dia | 0.005 m |  |  | Pin Dia | 0.005001375 m |  | 0.005001375 m |
|  | IG | 1.1633E-05 Mgm² |  |  | IG | 1.16394E-05 Mgm² |  | 1.1633E-05 Mgm² |
|  | d | 0.27 m |  |  | d | 0.27007425 m |  | 0.27007425 m |
|  | Volume | 4.79093E-05 m³ |  |  | Volume | 4.79488E-05 m³ |  | 4.79488E-05 m³ |
|  | Mass | 0.000375082 Mg |  |  | Mass | 0.000375082 Mg |  | 0.000375082 Mg |
|  | IXX | 3.89765E-05 Mgm² |  |  | IXX | 3.89979E-05 Mgm² |  | 3.89915E-05 Mgm² |

|  | Pendulum Bob: |  |  |  | Pendulum Bob: |  |  | Pendulum Bob: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assumed Cylinder with 10mm hole. |  |  |  | Assumed Cylinder with 10mm hole. |  |  |  |  |
|  | L | 0.1 m |  |  | L | 0.1000275 m |  | 0.1000275 m |
|  | Diameter | 0.05 m |  |  | Diameter | 0.05001375 m |  | 0.05001375 m |
|  | Volume | 0.00019635 m³ |  |  | Volume | 0.000196512 m³ |  | 0.000196512 m³ |
|  | Mass | 0.001537221 Mg |  |  | Mass | 1.537E-03 Mg |  | 1.537E-03 Mg |
|  | IG | 1.52121E-06 Mgm² |  |  | IG | 1.52204E-06 Mgm² |  | 1.52121E-06 Mgm² |
|  | Hole L | 0.1 m |  |  | Hole L | 0.1000275 m |  | 0.1000275 m |
|  | Hole Dia | 0.01 m |  |  | Hole Dia | 0.01000275 m |  | 0.01000275 m |
|  | Hole Vol | -7.85398E-06 m³ |  |  | Hole Vol | -7.86046E-06 m |  | -7.86046E-06 m |
|  | Hole Mass | -6.149E-05 Mg |  |  | Hole Mass | -6.149E-05 Mg |  | -6.149E-05 Mg |
|  | Hole IG | -5.1625E-08 Mgm² |  |  | Hole IG | -5.16534E-08 Mgm² |  | -5.1625E-08 Mgm² |
|  | Total Vol | 0.000188496 m³ |  |  | Total Vol | 0.000188651 m³ |  | 0.000188651 m³ |
|  | Total Mass | 0.001475732 Mg |  |  | Total Mass | 0.001475732 Mg |  | 0.001475732 Mg |
|  | Total IG | 1.46958E-06 Mgm² |  |  | Total IG | 1.47039E-06 Mgm² |  | 1.46958E-06 Mgm² |
|  | d | 0.5025 m |  |  | d | 0.502638188 m |  | 0.502638188 m |
|  | IXX | 0.000374101 Mgm² |  |  | IXX | 0.000374307 Mgm² |  | 0.000374306 Mgm² |

|  | Overall Totals: |  |  |  |  |  |  |  | Overall Totals: |  |  |  | Overall Totals: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Volume |  | 0.000263405 m³ |  |  | Volume |  |  |  | 0.000263622 m³ |  |  | 0.000263622 m³ |
|  | Mass |  | 0.002062197 Mg |  |  | Mass |  |  |  | 0.002062197 Mg |  |  | 0.002062197 Mg |
| IXX |  | 4.13140954254399E-04 Mgm² |  |  |  | IXX |  |  | 4.13368213023024E-04 Mgm² |  |  |  | 4.13360996887402E-04 Mgm² |
|  | Periodic Time Calculation: |  |  |  |  |  |  |  | Periodic Time Calculation: |  |  | Periodic Time Calculation: |  |
|  | LEQ |  |  | 0.407166107 m |  |  |  |  | LEQ | 0.407278078 m |  | 0.407278078 m |  |
|  | τ |  |  | 1.407400876 s |  |  |  |  | τ | 1.40759438 s |  | 1.407582094 s |  |
| Change in τ |  |  |  | 0.000193504 s |  |  |  |  |  | 0.000181218 s |  | -6.349307679 |  |
|  | Half Period: |  |  | 0.703700438 s |  |  |  |  |  | 0.70379719 s |  | 0.703791047 s |  |

## Calibration of the K Type Thermocouple Amplifier

When two dissimilar metals are brought into contact with each other, an e.m.f. or voltage is induced between them. This is known as the Seebeck effect. If there is temperature gradient along either of both metals, the e.m.f. (Electro-Motive Force) will change slightly. This is called the Thomson effect[2].

The e.m.f.’s developed by the thermocouple are extremely small and must be amplified before they can be measured by systems such as LabVIEW. This is done by the use of a thermocouple amplifier as shown in Figure 9.4.29.

Figure 9.4.29: Thermocouple Amplifier

In order to calibrate the amplifier, an adjustment screw (See red arrow in Figure 9.4.29) is used to change the gain of the circuit, thereby adjusting the amount by which the output voltage changes for a given temperature change.

The Thermocouple used for this experiment is a ‘K’ Type thermocouple, which means that it uses a Chromel Alumel junction to produce its e.m.f.. The e.m.f. at 10°C is approximately 0.412 millivolts[2].

In order to zero the voltage output from the thermocouple amplifier, a microchip designed for thermocouple amplifiers is used. This has a cold junction, which automatically fixes the zero on the thermocouple at 0ºC.

The thermocouple was calibrated using a second thermocouple that had previously been professionally calibrated. The reference temperatures used for the calibration were water at 80°C and room temperature at 20°C. Skin temperature at 31°C was used as a test to confirm that the calibration was correct. All readings agreed to within ±0.5º, which was adequate for the experimental requirements.

It should be noted that ‘K’ Type thermocouples are considered ‘satisfactory, but not recommended’ for use at temperatures below 315°C[2], however, the ± 1ºC accuracy recorded from the calibration procedure was considered adequate.

## LabVIEW

LabVIEW is a program written by the National Instruments Company. It is designed to be used for experimental and practical data gathering and processing, as well as for controlling other equipment through its analogue and digital outputs.

LabVIEW stands for Laboratory Virtual Instrument Engineering Workshop and was written in ‘G’, a 32 bit graphical programming language. LabVIEW programs have two parts, the front panel and the block diagram[4].

For the experimental work required for this project, LabVIEW was intended to be used to log both the pendulum periodic time from the proximity switch, as well as the temperature from the thermocouple fitted to the pendulum rig frame as in Section 4.4, completely automating the data gathering process for the experimental work.

LabVIEW is a relatively user friendly program, as it is programmed graphically. The resulting programs look much like a circuit diagrams, which makes it easier to find faults and to write the programs in the first place.

The LabVIEW interface is split into two basic parts. The first is the control window. This displays the results measured by LabVIEW, as well as allowing the person using the program to change the program settings. The second window is the diagram window. This displays the program as an electrical type schematic.

However, because of the user friendly interface, there is a lot of processor overhead, which affects the results measured by LabVIEW for precision timing applications. This caused considerable trouble as before the program was written, the National Instruments technical support was consulted, and they said that the program was capable of taking measurements to the accuracy required, since they assumed the computer being used was not a limiting factor.

Three weeks were spent, between learning to use LabVIEW and actually writing the program to log the periodic times, only to discover in testing a prototype program that the readings being taken were inconsistent.

The speed at which the program runs could be improved by using a faster computer, but the Pentium 133MHz which was allocated for the job should really have been more than adequate. The program, which was supposed to log all the periodic times while the experiment was in progress was never finished, as the initial testing done on a non-logging version of the program showed errors in the time readings as described in Section 9.7.

However, LabVIEW was still used to log the temperature readings from the thermocouple, as this was not a timing intensive operation, so a second program was written to do this. This program did exactly what was required of it, and the diagram for this program can be seen in Figure 9.6.30.

### Understanding LabVIEW diagrams

LabVIEW diagrams are laid out much like a circuit in that each block on the diagram performs some sort of operation on the circuit. This is determined by the inputs wired in to the block and the type of block chosen. The program block performs its function and outputs the result through more wires connected to its outputs.

Multiple blocks can be wired in parallel if necessary, making LabVIEW more flexible than most languages in the way it can be programmed, as most languages require linear command sequences.

Wires connecting blocks are colour coded to allow them to be identified quickly. The type of data carried by the wire determines its colour as follows;

- Blue - Integer Number

- Orange - Double Precision Number

- Pink - Text String

- Green - Boolean True/False

- Thick blue or orange - Multiple values of integer or double precision number types

Inputs can either be set by using constants in the diagram, which show up as thick bordered boxes with numbers in them, or else by taking values from the control window. This method allows the user to change values as they require without having to look at the block diagram each time.

Inputs from the control window are displayed as thick bordered boxes with an abbreviation for the number type in them such as DBL for double precision, I16 or I32 for 16 or 32 bit integers and V16 or V32 for 16 or 32 bit positive integers. The outputs from the program are boxes with thin borders, which use the same numerical notation as the inputs. The values contained in these blocks are displayed in the control window.

Blocks with yellow backgrounds are used to denote mathematical or Boolean functions. They use  as 'or' and  as 'and', with the rest of the symbols being familiar to most people. Blocks with yellow backgrounds and blue text are used to control the for and while loop functions.

For loops repeat the contents of the loop as many times as the value wired to the N box in the loop and while loops continuously repeat the contents of the loop until a certain condition changes from true to false. In the for loop i is the number of iterations the loop has performed at any given time. The circular arrow in the while loop is wired to a Boolean variable and controls when the loop stops and stops.

## The temperature Logging Program

The temperature log diagram uses a for loop with a delay which can be adjusted by changing the value of the ‘Time Interval between readings’ variable, the make the loop execute at given intervals. Note that the change in the thickness of the wires outputting from the for loop indicate that the wires carry multiple values.

These values are written to a text file as the logged temperature readings, as well as to an XY Graph, which displays the data gathered in the control window after the program has finished running. The values recorded by the temperature logging program can be seen in Section 9.20 and 9.21.

Figure 9.6.30: Temperature Logging Program Block Diagram

Figure 9.6.31: Control Window for Temperature Logging Program

## The Periodic Time Logging Program

This program was written to measure the periodic time of the pendulum for each period occurring during the experiment. It does this by measuring the width of the high pulse from the proximity switch and adding it to the width of the low pulse, also measured by LabVIEW through a different channel.

It was necessary to do this as LabVIEW can only be made to measure high pulses, so the signal from which the low pulse was measured had to be inverted, using a 7404 Inverter chip. This is a not gate and converts the high pulses to low pulses and vice versa. The wiring diagram for this arrangement is shown in the control window in Figure 9.7.32.

Figure 9.7.32: Control Window for the Periodic Time Logging Program

Figure 9.7.33: Block Diagram for the Periodic Time Logging Program

The pulses are measured using a system of counters, driven from a 2MHz clock built into the LabVIEW board. Figure 9.7.33 shows the block diagram of this program. This was thought to be accurate enough for the purposes of the experiment, though there were a few limiting factors, namely that the maximum counter size which LabVIEW can handle is 65,535, which means that the length of each period measured has to be less than 65,535 times the minimum time unit. Because of this, a sample rate of 90,000 Hz was chosen, allowing a maximum pulse width of 0.727 Seconds to be measured, as this will give the best time resolution, without causing the counters to overflow.

However, when the program was tested, it was discovered that when the same signal was sent into both pulse measuring loops, the values read from the program for Count and Count 2 disagreed by 1 every third period or so, showing an error in the values measured by the computer. This error is due to the computer taking too long to run the program, as, though the two loops are wired in parallel, the computer must execute them one after another, causing the reading from the second loop to be larger than the reading from the first.

There is also the possibility that there is a delay caused by the computer running the program too slowly before it even takes the first reading, leading to more error. Also, when the 7404 chip is fitted into the circuit, it may also add a small delay to the signal.

If the program was to be modified to write the periodic times to a separate text file, the two pulse length measuring loops would have to be placed in another loop, which would make the computer take even longer to run the program, increasing error further. The temperature logging program will also add to the error for the same reason, as it would also be running while the periodic time measuring program is running, and therefore take up computer processing time.

It was concluded from this that, though it is possible for LabVIEW to perform the periodic time logging function, no confidence can be expressed as to its accuracy. It is for this reason that the timer counter was used instead of LabVIEW for periodic time measurement, as this was designed for the sole purpose of taking these measurements. However, LabVIEW was used for recording the temperature read by the thermocouple at regular time intervals, as this does not require precision timing and saves a considerable amount of work.

## Construction Drawings of the Test Pendulum

The actual Dimensions of the test pendulum which was built can be found in the theoretical calculations Section 9.3.1, which uses the design dimensions and 9.3.2, which uses the actual dimensions.

Figure 9.8.34:Test Pendulum Assembly

Figure 9.8.35: Close up of Ruler, proximity switch, threaded bar and Bob

| PENDULUM ASSEMBLY DRAWINGS | First Angle Projection All Dimensions in Millimetres |
| --- | --- |
| Part Name: Pendulum Bob Material: Steel | Design by Cormac Eason 9731318 15/11/2000 |

| PENDULUM ASSEMBLY DRAWINGS | First Angle Projection All Dimensions in Millimetres |
| --- | --- |
| Part Name: Knife Edge Material: Steel | Design by Cormac Eason 9731318 15/11/2000 |

| PENDULUM ASSEMBLY DRAWINGS | First Angle Projection All Dimensions in Millimetres |
| --- | --- |
| Part Name: Pendulum Mounting Material: Steel | Design by Cormac Eason 9731318 15/11/2000 |

| PENDULUM ASSEMBLY DRAWINGS | First Angle Projection All Dimensions in Millimetres |
| --- | --- |
| Part Name: Pin (2 off) Material: Steel | Design by Cormac Eason 9731318 15/11/2000 |
| PENDULUM ASSEMBLY DRAWINGS | First Angle Projection All Dimensions in Millimetres |
| Part Name: Pendulum Rod Material: Steel | Design by Cormac Eason 9731318 15/11/2000 |

| PENDULUM ASSEMBLY DRAWINGS | First Angle Projection All Dimensions in Millimetres |
| --- | --- |
| Part Name: Proximity Switch Mounting Material: Steel | Design by Cormac Eason 9731318 15/11/2000 |

## Details of the Compensated Pendulum

### The Parts of the Compensated Pendulum

| Component | Material | Description |
| --- | --- | --- |
| Block Pin | Steel | Holds Split Block in position in Mounting |
| Bob | Brass | This is weight hung from the rod gives the pendulum its behaviour |
| Bob Pin | Steel | Prevents bob from turning on pendulum rod. Works in conjunction with Sleeve |
| Carbon Fibre Rod | Carbon Fibre | Contracts as temperature rises to compensate pendulum |
| Clamp | Steel | Clamps to Rod Top |
| Clamp Pin | Steel | Locates suspension spring in clamp |
| Mounting | Steel | Supports pendulum, connecting it to the rest of the clock |
| Rating Nut | Steel | This supports the bob and allows the pendulum to be calibrated by winding the rating nut up and down |
| Rivet | Steel | Holds suspension spring rigidly to Split Block |
| Rod Base | Steel | Fits to the end of the Carbon Fibre Rod. Threaded for the rating nut and drilled to hold the Bob Pin |
| Rod Top | Steel | Clamps the other end of the suspension spring |
| Screw | Steel | Clamps the suspension spring between Rod Top and Clamp |
| Sleeve | Steel | This is an interference fit into bob and engages with Bob Pin |
| Split Block | Brass | Slit block to hold suspension spring |
| Spring | Steel | Suspension Spring, flexes to allow pendulum to swing |

### The Amalgamated Parts which were Analysed

| Component | Material | Description |
| --- | --- | --- |
| Base Assembly | Steel | This is an amalgamation of the Bob Pin, Rating Nut and Rod Base. |
| Bob | Brass | This is the same as the Bob in the preceding Table. |
| Carbon Fibre Rod | Carbon Fibre | This is the same as the Carbon Fibre rod in the preceding Table. |
| Sleeve | Steel | This is the same as the Sleeve in the preceding Table. |
| Top Assembly | Steel | Amalgamation of the part of the clamp, clamp pin, Rod Top, screw and the part of the suspension spring which is not in the split block. |

## Dimensioned Drawings of Compensated Pendulum Parts

All Dimensions are in millimetres.

Figure 9.10.36: Block Pin and Clamp Pin are both exactly the same

Figure 9.10.37: Pendulum Bob

Figure 9.10.38: Bob Pin

Figure 9.10.39: Carbon Fibre Rod

Figure 9.10.40: Clamp

Figure 9.10.41: Mounting

Figure 9.10.42: Rating Nut

Figure 9.10.43: Rivet

Figure 9.10.44: Rod Base

Figure 9.10.45: Screw

Figure 9.10.46: Suspension Spring

Figure 9.10.47: Rod Top

Figure 9.10.48: Sleeve

Figure 9.10.49: Split Block

## Amalgamated Parts Analysed

Figure 9.11.50: Base Assembly

Figure 9.11.51: Deformed Base Assembly

Figure 9.11.52: Deformed Pendulum Bob

Figure 9.11.53: Deformed Carbon Fibre Rod

Figure 9.11.54: Deformed Sleeve

Figure 9.11.55: Assembly of Bottom of Pendulum, showing bob, bob pin, rating nut, rod base, carbon fibre rod and sleeve.

Figure 9.11.56: Top Assembly

Figure 9.11.57: Deformed Top Assembly

Figure 9.11.58: Top of Pendulum assembly, showing mounting, split block, block pin, rivet, suspension spring, clamp, screw, clamp pin, top rod and carbon fibre rod.

## Comparison of Separate Parts of Compensated Pendulum with Combined Parts

Table 7: Density changes in the parts of the Compensated Pendulum before and after the 25ºC Temperature rise.

| Part | Initial Volume | Deformed Volume | Material | New Density |
| --- | --- | --- | --- | --- |
| Bob | 85216.099 mm³ | 85337.589 mm³ | Brass | 8.39703E-09 Mg /mm³ |
| Carbon Fibre Rod | 6122.3358 mm³ | 6121.9684 mm³ | Carbon Fibre | 3.40095E-10 Mg /mm³ |
| Rod Base | 2640.0525 mm³ | 2641.915 mm³ | Steel | 7.82348E-09 Mg /mm³ |
| Sleeve | 3307.5628 mm³ | 3310.2923 mm³ | Steel | 7.82254E-09 Mg /mm³ |
| Rod Top | 1327.3429 mm³ | 1328.4377 mm³ | Steel | 7.82255E-09 Mg /mm³ |

## Compensated Pendulum Mass Properties as Calculated by ProEngineer

### Initial Pendulum Mass Properties with respect to Co-ordinate system at top of suspension spring. Ixx is inertia about pendulum rotation axis.

VOLUME =  9.8613393e+04  MM^3

SURFACE AREA =  3.8407444e+04  MM^2

AVERAGE DENSITY =  7.8652590e-09 TONNE / MM^3

MASS =  7.7561988e-04 TONNE

CENTER OF GRAVITY with respect to ACS0 coordinate frame:

X   Y   Z     8.3895504e-03 -5.5231794e+02  0.0000000e+00  MM

INERTIA with respect to ACS0 coordinate frame:  (TONNE * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.4044349e+02  3.5058641e-03  0.0000000e+00

Iyx Iyy Iyz  3.5058641e-03  4.0771561e-01  0.0000000e+00

Izx Izy Izz  0.0000000e+00  0.0000000e+00  2.4005738e+02

INERTIA at CENTER OF GRAVITY with respect to ACS0 coordinate frame:  (TONNE * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.8366886e+00 -8.8125062e-05  0.0000000e+00

Iyx Iyy Iyz -8.8125062e-05  4.0771556e-01 -5.8148438e-07

Izx Izy Izz  0.0000000e+00 -5.8148438e-07  3.4505778e+00

PRINCIPAL MOMENTS OF INERTIA:  (TONNE * MM^2)

I1  I2  I3   4.0771556e-01  3.4505778e+00  3.8366886e+00

ROTATION MATRIX from ACS0 orientation to PRINCIPAL AXES:

0.00003        0.00000        1.00000

1.00000        0.00000       -0.00003

0.00000        1.00000        0.00000

ROTATION ANGLES from ACS0 orientation to PRINCIPAL AXES (degrees):

angles about x  y  z   0.000         89.999         90.000

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 2.2927367e+01  6.6699326e+01  7.0332137e+01  MM

---------------------------------------------

MASS PROPERTIES OF COMPONENTS OF THE ASSEMBLY

(in assembly units and the ACS0 coordinate frame)

DENSITY             MASS          C.G.:  X        Y         Z

SPRINGBASE            MATERIAL:                              STEEL

7.82900e-09  1.03918e-05  0.00000e+00 -3.31527e+01  0.00000e+00

CF_ROD            MATERIAL:                              CARBON FIBRE

3.40075e-10  2.08205e-06  0.00000e+00 -3.05400e+02  0.00000e+00

PENDULUM_BASE            MATERIAL:                              STEEL

7.82900e-09  2.06690e-05 -4.74147e-03 -5.96982e+02  0.00000e+00

BOB            MATERIAL:                              BRASS

8.40900e-09  7.16582e-04  0.00000e+00 -5.59757e+02  0.00000e+00

SLEEVE            MATERIAL:                              STEEL

7.82900e-09  2.58949e-05  2.55097e-01 -5.39000e+02  0.00000e+00

### Deformed Pendulum Mass Properties with respect to Co-ordinate system at top of suspension spring. IXX is inertia about pendulum rotation axis.

VOLUME =  9.8740203e+04  MM^3

SURFACE AREA =  3.8429187e+04  MM^2

AVERAGE DENSITY =  7.8551717e-09 TONNE / MM^3

MASS =  7.7562125e-04 TONNE

CENTER OF GRAVITY with respect to ACS0 coordinate frame:

X   Y   Z     8.3918120e-03 -5.5229660e+02  0.0000000e+00  MM

INERTIA with respect to ACS0 coordinate frame:  (TONNE * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.4042605e+02  3.5066400e-03  0.0000000e+00

Iyx Iyy Iyz  3.5066400e-03  4.0810290e-01  0.0000000e+00

Izx Izy Izz  0.0000000e+00  0.0000000e+00  2.4003957e+02

INERTIA at CENTER OF GRAVITY with respect to ACS0 coordinate frame:  (TONNE * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.8371102e+00 -8.8185465e-05  0.0000000e+00

Iyx Iyy Iyz -8.8185465e-05  4.0810284e-01 -5.8203632e-07

Izx Izy Izz  0.0000000e+00 -5.8203632e-07  3.4506324e+00

PRINCIPAL MOMENTS OF INERTIA:  (TONNE * MM^2)

I1  I2  I3   4.0810284e-01  3.4506324e+00  3.8371102e+00

ROTATION MATRIX from ACS0 orientation to PRINCIPAL AXES:

0.00003        0.00000        1.00000

1.00000        0.00000       -0.00003

0.00000        1.00000        0.00000

ROTATION ANGLES from ACS0 orientation to PRINCIPAL AXES (degrees):

angles about x  y  z   0.000         89.999         90.000

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 2.2938233e+01  6.6699795e+01  7.0335938e+01  MM

---------------------------------------------

MASS PROPERTIES OF COMPONENTS OF THE ASSEMBLY

(in assembly units and the ACS0 coordinate frame)

DENSITY             MASS          C.G.:  X        Y         Z

SPRINGBASE_DEFORMED            MATERIAL:                         STEEL

7.82255e-09  1.03918e-05  0.00000e+00 -3.31609e+01  0.00000e+00

CF_ROD_DEFORMED            MATERIAL:                 CARBON FIBRE

3.40099e-10  2.08208e-06  0.00000e+00 -3.05411e+02  0.00000e+00

PENDULUM_BASE_DEFORMED     MATERIAL:                     STEEL

7.82395e-09  2.06702e-05 -4.74334e-03 -5.96989e+02  0.00000e+00

BOB_DEFORMED            MATERIAL:                              BRASS

8.39703e-09  7.16582e-04  0.00000e+00 -5.59735e+02  0.00000e+00

SLEEVE_DEFORMED            MATERIAL:                              STEEL

7.82254e-09  2.58949e-05  2.55167e-01 -5.38974e+02  0.00000e+00

## Carbon Fibre Rod Details

Precise details of the Carbon Fibre rod were not available, so the following measurements were taken for a rod in the possession of Dr. Richard Stephen, thus allowing the density of the Carbon Fibre to be calculated.

Length of rod	750 mm

OD of rod	4.90 mm

ID of rod	2.90 mm

Mass of rod	12.5 ± 0.5 grammes

Derived Density of Rod Material	3.40075E-10 Mg/mm³

## Properties of the Materials used

|  | Steel: |  |  | ProEngineer Units |  | In SI Units |
| --- | --- | --- | --- | --- | --- | --- |
|  | Density: |  | ρS | 7.829E-09 tonnes/mm³ | = | 7829 kg/m³ |
|  | Thermal Expansion Coefficient: |  | cS | 1.100E-05 mm/mm°C | = | 1.100E-05 m/m°C |
|  | Brass: |  |  |  |  |  |
|  | Density: |  | ρB | 8.409E-09 tonnes/mm³ | = | 8409 kg/m³ |
|  | Thermal Expansion Coefficient: |  | cB | 1.900E-05 mm/mm°C | = | 1.900E-05 m/m°C |
|  | Carbon Fibre: |  |  |  |  |  |
|  | Density: |  | ρCF | 3.40075E-10 tonnes/mm³ | = | 340.0746647 kg/m³ |
|  | Thermal Expansion Coefficient: |  | cCF | -8.000E-07 mm/mm°C | = | -8.000E-07 m/m°C |
|  | Acceleration due to Gravity: |  | g | 9806.65 mm/s² | = | 9.80665 m/s² |

Material Densities are from Roark and Young 5th Edition[9] as used by ProMechanica Library. Material coefficients of expansion are from Richard Stephen’s Article in the British Horological Journal[11].

## Benchmarking

In order to be confident that the Finite Element Analysis was giving the correct results, it was necessary to perform some basic analyses on parts of known geometry for which the deformation after a temperature increase is known.

The shape chosen was a cylinder 1000 mm high with a diameter of 1000 mm. The deformed shape of this cylinder after a 25ºC temperature rise was then calculated theoretically for the three materials being used, using the expansion equation in Section 2.9.

The model was meshed and constrained as shown in Figure 9.16.59. The constraints prevent the point at the origin of the co-ordinate system from moving in any direction, while the point opposite it along the x-axis is free to move in the x direction. The point opposite the origin in the y direction is free to move in y, while the point on the opposite diagonal is free in both x and y, but constrained in z. These constraints hold the xy-plane through the centre of the cylinder vertical, while not affecting the overall expansion of the cylinder.

Figure 9.16.59: The Finite Element Analysis Benchmark Model

Figure 9.16.60 shows the resulting expansion for the metal parts and the contraction for the carbon fibre part as calculated by ProMechanica.

Figure 9.16.60: Initial (Purple) and Deformed (Blue) Benchmark models for Carbon Fibre and Steel Respectively

The comparison of these results can be seen in Section 9.19. It indicates that the finite element analysis has a high degree of accuracy, but that it does show some error due to rounding during calculations. This rounding error is of the order of 
6×10-10 %, and this becomes an error of 2×10-6% when these dimensions are used to calculate the volume of the deformed part.

Figure 9.16.61: The Queried X-Displacements in millimetres for the Steel Model (Values circled in red are maximum or minimum values).

However, even these error calculations suffer from rounding error as ProMechanica outputs results to 14 significant places, and Microsoft Excel, the program used to perform the error calculations operates to 15 significant places, causing some digits to be lost as the total number of digits in each new dimension is 18, due to it being the summation of 1000mm, the initial dimension, and x.xe-1, the change in the initial dimension, which is a number with 14 significant digits.

Figure 9.16.61 shows the results given by ProMechanica when the query function was used for the steel model. These results are exactly right except for some of the zero values, as the full results from ProMechanica rounded to 6 decimal places will give the same results as the theoretically calculated values.

## Finite Element Analysis Run Summaries

### Run Summary for Brass Benchmark Test

--------------------------------------------------------

Pro/MECHANICA STRUCTURE Version 22.3(305)

Summary for Design Study "BenchmarkBrass"

Mon Mar 12, 2001   17:19:13

--------------------------------------------------------

Run Settings

Memory allocation for block solver: 48.0

Pro/MECHANICA STRUCTURE Model Summary

Model Type: Three Dimensional

Points:         10

Edges:          29

Faces:          32

Springs:         0

Masses:          0

Beams:           0

Shells:          0

Solids:         12

Elements:       12

--------------------------------------------------------

Standard Design Study

Description:

Benchmark Test of Steel Cylinder to confirm its behaviour under 25 degree temperature rise.

Static Analysis "BenchmarkBrass":

Convergence Method: Multiple-Pass Adaptive

Plotting Grid:     10

Convergence Loop Log:                (17:19:14)

>> Pass  1 <<

Calculating Element Equations  (17:19:14)

Total Number of Equations:      22

Maximum Edge Order:              1

Solving Equations                       (17:19:15)

Post-Processing Solution              (17:19:15)

Calculating Disp and Stress Results (17:19:15)

Checking Convergence                    (17:19:17)

Elements Not Converged:         12

Edges Not Converged:            29

Measure Convergence:        100.0%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:19:17)

Elapsed Time     (sec):       6.13

CPU Time         (sec):       4.06

Memory Usage      (kb):      91504

Wrk Dir Dsk Usage (kb):          0

>> Pass  2 <<

Calculating Element Equations (17:19:17)

Total Number of Equations:     109

Maximum Edge Order:              2

Solving Equations                       (17:19:17)

Post-Processing Solution                (17:19:17)

Calculating Disp and Stress Results (17:19:17)

Checking Convergence                    (17:19:19)

Elements Not Converged:          8

Edges Not Converged:            29

Measure Convergence:         12.0%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:19:19)

Elapsed Time     (sec):       7.95

CPU Time         (sec):       5.67

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

>> Pass  3 <<

Calculating Element Equations         (17:19:19)

Total Number of Equations:     292

Maximum Edge Order:              3

Solving Equations                       (17:19:19)

Post-Processing Solution                (17:19:19)

Calculating Disp and Stress Results (17:19:20)

Checking Convergence                  (17:19:21)

Elements Not Converged:          0

Edges Not Converged:            12

Measure Convergence:          1.3%

Local Disp/Energy Index:      2.8%

Global RMS Stress Index:      0.0%

Resource Check                          (17:19:21)

Elapsed Time     (sec):      10.30

CPU Time         (sec):       7.77

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

>> Pass  4 <<

Calculating Element Equations           (17:19:21)

Total Number of Equations:     508

Maximum Edge Order:              4

Solving Equations                       (17:19:22)

Post-Processing Solution                (17:19:22)

Calculating Disp and Stress Results (17:19:22)

Checking Convergence                    (17:19:24)

Elements Not Converged:          0

Edges Not Converged:             0

Measure Convergence:          0.0%

Local Disp/Energy Index:      0.0%

Global RMS Stress Index:      0.0%

RMS Stress Error Estimates:

Load Set          Stress Error  % of Max Prin Str

----------------  ------------  -----------------

LoadSet1           6.58e-12    61.8% of  1.06e-11

Resource Check                          (17:19:25)

Elapsed Time     (sec):      13.98

CPU Time         (sec):      11.00

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

The analysis converged to within 1% on measures.

Total Mass of Model:  6.600958e+00

Total Cost of Model:  0.000000e+00

Mass Moments of Inertia about WCS Origin:

Ixx:  2.61278e+06

Ixy:  7.83621e-07  Iyy:  8.24693e+05

Ixz:  1.09139e-11  Iyz: -2.91038e-11  Izz: 2.61278e+06

Principal MMOI and Principal Axes Relative to WCS Origin:

Max Prin           Mid Prin           Min Prin

2.61278e+06      2.61278e+06      8.24693e+05

WCS X:  0.00000e+00  1.00000e+00  0.00000e+00

WCS Y:  0.00000e+00  0.00000e+00  1.00000e+00

WCS Z:  1.00000e+00  0.00000e+00  0.00000e+00

Center of Mass Location Relative to WCS Origin:

(-2.36813e-14,  5.00000e+02,  2.15285e-14)

Mass Moments of Inertia about the Center of Mass:

Ixx:  9.62544e+05

Ixy:  7.83542e-07  Iyy:  8.24693e+05

Ixz:  1.09139e-11  Iyz:  4.19504e-11  Izz:  9.62544e+05

Principal MMOI and Principal Axes Relative to COM:

Max Prin           Mid Prin           Min Prin

9.62544e+05    9.62544e+05      8.24693e+05

WCS X:  0.00000e+00   1.00000e+00  0.00000e+00

WCS Y:  0.00000e+00  0.00000e+00  1.00000e+00

WCS Z:  1.00000e+00  0.00000e+00  0.00000e+00

Constraint Set: ConstraintSet1

Load Set: LoadSet1

Resultant Load on Model:

in global X direction:  2.527572e-08

in global Y direction:  2.837242e-08

in global Z direction: -1.387300e-08

Measures:

Name              Value      Convergence

--------------     -------------  -----------

max_beam_bending:   0.000000e+00      0.0%

max_beam_tensile:   0.000000e+00      0.0%

max_beam_torsion:   0.000000e+00      0.0%

max_beam_total:     0.000000e+00      0.0%

max_disp_mag:       6.717514e-01      0.0%

max_disp_x:         4.750000e-01      0.0%

max_disp_y:         4.750000e-01      0.0%

max_disp_z:         2.375000e-01      0.0%

max_prin_mag:       1.064508e-11      1.9%

max_rot_mag:        0.000000e+00      0.0%

max_rot_x:          0.000000e+00      0.0%

max_rot_y:          0.000000e+00      0.0%

max_rot_z:          0.000000e+00      0.0%

max_stress_prin:    1.064508e-11      6.3%

max_stress_vm:      6.540717e-12     52.8%

max_stress_xx:     -6.684177e-12     35.3%

max_stress_xy:     -7.414438e-13      9.5%

max_stress_xz:     -1.106589e-12      6.0%

max_stress_yy:      1.058438e-11     14.3%

max_stress_yz:     -1.163393e-12     45.0%

max_stress_zz:     -5.687675e-12     87.4%

min_stress_prin:   -7.833536e-12     38.4%

strain_energy:      6.519258e-09     71.4%

Diameter:           4.750000e-01      0.0%

Height:             4.750000e-01      0.0%

Analysis "BenchmarkBrass" Completed  (17:19:25)

--------------------------------------------------------

Memory and Disk Usage:

Machine Type: Windows NT/x86

RAM Allocation for Solver (megabytes): 48.0

Total Elapsed Time (seconds): 14.19

Total CPU Time     (seconds): 11.09

Maximum Memory Usage (kilobytes): 92592

Working Directory Disk Usage (kilobytes): 0

Results Directory Size (kilobytes):

1139 .\BenchmarkBrass

--------------------------------------------------------

Run Completed

Mon Mar 12, 2001   17:19:25

--------------------------------------------------------

### Run Summary for Carbon Fibre Benchmark Test

--------------------------------------------------------

Pro/MECHANICA STRUCTURE Version 22.3(305)

Summary for Design Study "BenchMarkCF"

Mon Mar 12, 2001   17:14:58

--------------------------------------------------------

Run Settings

Memory allocation for block solver: 48.0

Pro/MECHANICA STRUCTURE Model Summary

Model Type: Three Dimensional

Points:         10

Edges:          29

Faces:          32

Springs:         0

Masses:          0

Beams:           0

Shells:          0

Solids:         12

Elements:       12

--------------------------------------------------------

Standard Design Study

Description:

Benchmark Test of Steel Cylinder to confirm its behaviour under 25 degree temperature rise.

Static Analysis "BenchMarkCF":

Convergence Method: Multiple-Pass Adaptive

Plotting Grid:     10

Convergence Loop Log: (17:15:00)

>> Pass  1 <<

Calculating Element Equations        (17:15:00)

Total Number of Equations:      22

Maximum Edge Order:              1

Solving Equations                       (17:15:00)

Post-Processing Solution                (17:15:00)

Calculating Disp and Stress Results (17:15:00)

Checking Convergence                    (17:15:02)

Elements Not Converged:         12

Edges Not Converged:            29

Measure Convergence:        100.0%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:15:02)

Elapsed Time     (sec):       5.63

CPU Time         (sec):       4.03

Memory Usage      (kb):      91504

Wrk Dir Dsk Usage (kb):          0

>> Pass  2 <<

Calculating Element Equations        (17:15:02)

Total Number of Equations:     109

Maximum Edge Order:              2

Solving Equations                       (17:15:03)

Post-Processing Solution                (17:15:03)

Calculating Disp and Stress Results (17:15:03)

Checking Convergence                    (17:15:04)

Elements Not Converged:         12

Edges Not Converged:            29

Measure Convergence:         11.3%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:15:04)

Elapsed Time     (sec):       7.27

CPU Time         (sec):       5.50

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

>> Pass  3 <<

Calculating Element Equations        (17:15:04)

Total Number of Equations:     292

Maximum Edge Order:              3

Solving Equations                       (17:15:04)

Post-Processing Solution                (17:15:04)

Calculating Disp and Stress Results (17:15:05)

Checking Convergence                    (17:15:06)

Elements Not Converged:          0

Edges Not Converged:            12

Measure Convergence:          1.3%

Local Disp/Energy Index:      2.7%

Global RMS Stress Index:      0.0%

Resource Check                          (17:15:06)

Elapsed Time     (sec):       9.44

CPU Time         (sec):       7.52

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

>> Pass  4 <<

Calculating Element Equations         (17:15:06)

Total Number of Equations:     508

Maximum Edge Order:              4

Solving Equations                       (17:15:07)

Post-Processing Solution                (17:15:07)

Calculating Disp and Stress Results (17:15:07)

Checking Convergence                    (17:15:09)

Elements Not Converged:          0

Edges Not Converged:             0

Measure Convergence:          0.0%

Local Disp/Energy Index:      0.0%

Global RMS Stress Index:      0.0%

RMS Stress Error Estimates:

Load Set          Stress Error  % of Max Prin Str

----------------  ------------  -----------------

LoadSet1        5.19e-14      62.1% of  8.35e-14

Resource Check                          (17:15:10)

Elapsed Time     (sec):      12.78

CPU Time         (sec):      10.77

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

The analysis converged to within 1% on

measures.

Total Mass of Model:  2.669545e-01

Total Cost of Model:  0.000000e+00

Mass Moments of Inertia about WCS Origin:

Ixx:  1.05666e+05

Ixy:  3.16900e-08  Iyy:  3.33521e+04

Ixz:  0.00000e+00  Iyz:  0.00000e+00  Izz:  1.05666e+05

Principal MMOI and Principal Axes Relative to WCS Origin:

Max Prin           Mid Prin           Min Prin

1.05666e+05    1.05666e+05     3.33521e+04

WCS X:  0.00000e+00     1.00000e+00     0.00000e+00

WCS Y:  0.00000e+00        0.00000e+00        1.00000e+00

WCS Z:  1.00000e+00        0.00000e+00        0.00000e+00

Center of Mass Location Relative to WCS Origin:

(-1.66354e-14,  5.00000e+02,  2.32895e-14)

Mass Moments of Inertia about the Center of Mass:

Ixx:  3.89270e+04

Ixy:  3.16878e-08  Iyy:  3.33521e+04

Ixz: -1.03426e-28  Iyz:  3.10862e-12  Izz:  3.89270e+04

Principal MMOI and Principal Axes Relative to COM:

Max Prin           Mid Prin           Min Prin

3.89270e+04    3.89270e+04     3.33521e+04

WCS X:  0.00000e+00        1.00000e+00        0.00000e+00

WCS Y:  0.00000e+00        0.00000e+00        1.00000e+00

WCS Z:  1.00000e+00        0.00000e+00        0.00000e+00

Constraint Set: ConstraintSet1

Load Set: LoadSet1

Resultant Load on Model:

in global X direction: -1.219896e-10

in global Y direction: -2.362952e-11

in global Z direction:  2.339130e-10

Measures:

Name              Value      Convergence

--------------     -------------  -----------

max_beam_bending:   0.000000e+00      0.0%

max_beam_tensile:   0.000000e+00      0.0%

max_beam_torsion:   0.000000e+00      0.0%

max_beam_total:     0.000000e+00      0.0%

max_disp_mag:       2.828427e-02      0.0%

max_disp_x:        -2.000000e-02      0.0%

max_disp_y:        -2.000000e-02      0.0%

max_disp_z:         1.000000e-02      0.0%

max_prin_mag:       8.353739e-14     48.4%

max_rot_mag:        0.000000e+00      0.0%

max_rot_x:          0.000000e+00      0.0%

max_rot_y:          0.000000e+00      0.0%

max_rot_z:          0.000000e+00      0.0%

max_stress_prin:    8.353739e-14     85.4%

max_stress_vm:      2.172273e-14     33.1%

max_stress_xx:      8.126304e-14     57.8%

max_stress_xy:     -4.941541e-15     56.3%

max_stress_xz:     -4.390926e-15     42.7%

max_stress_yy:      7.850041e-14     45.2%

max_stress_yz:     -3.153819e-15     45.7%

max_stress_zz:      6.927426e-14     50.0%

min_stress_prin:   -3.685547e-14     16.9%

strain_energy:      3.751666e-12     54.5%

Diameter:          -2.000000e-02      0.0%

Height:            -2.000000e-02      0.0%

Analysis "BenchMarkCF" Completed  (17:15:10)

--------------------------------------------------------

Memory and Disk Usage:

Machine Type: Windows NT/x86

RAM Allocation for Solver (megabytes): 48.0

Total Elapsed Time (seconds): 12.95

Total CPU Time     (seconds): 10.86

Maximum Memory Usage (kilobytes): 92592

Working Directory Disk Usage (kilobytes): 0

Results Directory Size (kilobytes):

1138 .\BenchMarkCF

--------------------------------------------------------

Run Completed

Mon Mar 12, 2001   17:15:10

--------------------------------------------------------

### Run Summary for Steel Benchmark Test

--------------------------------------------------------

Pro/MECHANICA STRUCTURE Version 22.3(305)

Summary for Design Study "BenchmarkSteel"

Mon Mar 12, 2001   17:12:15

--------------------------------------------------------

Run Settings

Memory allocation for block solver: 48.0

Pro/MECHANICA STRUCTURE Model Summary

Model Type: Three Dimensional

Points:         10

Edges:          29

Faces:          32

Springs:         0

Masses:          0

Beams:           0

Shells:          0

Solids:         12

Elements:       12

--------------------------------------------------------

Standard Design Study

Description:

Benchmark Test of Steel Cylinder to confirm its behaviour under 25 degree temperature rise.

Static Analysis "BenchmarkSteel":

Convergence Method: Multiple-Pass Adaptive

Plotting Grid:     10

Convergence Loop Log: (17:12:16)

>> Pass  1 <<

Calculating Element Equations        (17:12:16)

Total Number of Equations:      22

Maximum Edge Order:              1

Solving Equations                       (17:12:16)

Post-Processing Solution                (17:12:16)

Calculating Disp and Stress Results (17:12:16)

Checking Convergence                    (17:12:18)

Elements Not Converged:         12

Edges Not Converged:            29

Measure Convergence:        100.0%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:12:18)

Elapsed Time     (sec):       4.80

CPU Time         (sec):       4.02

Memory Usage      (kb):      91504

Wrk Dir Dsk Usage (kb):          0

>> Pass  2 <<

Calculating Element Equations        (17:12:18)

Total Number of Equations:     109

Maximum Edge Order:              2

Solving Equations                       (17:12:18)

Post-Processing Solution                (17:12:18)

Calculating Disp and Stress Results (17:12:18)

Checking Convergence                    (17:12:19)

Elements Not Converged:         12

Edges Not Converged:            29

Measure Convergence:         10.9%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:12:19)

Elapsed Time     (sec):       6.34

CPU Time         (sec):       5.53

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

>> Pass  3 <<

Calculating Element Equations         (17:12:19)

Total Number of Equations:     292

Maximum Edge Order:              3

Solving Equations                       (17:12:20)

Post-Processing Solution                (17:12:20)

Calculating Disp and Stress Results (17:12:20)

Checking Convergence                    (17:12:21)

Elements Not Converged:          0

Edges Not Converged:            12

Measure Convergence:          1.3%

Local Disp/Energy Index:      2.7%

Global RMS Stress Index:      0.0%

Resource Check                          (17:12:21)

Elapsed Time     (sec):       8.38

CPU Time         (sec):       7.50

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

>> Pass  4 <<

Calculating Element Equations         (17:12:21)

Total Number of Equations:     508

Maximum Edge Order:              4

Solving Equations                       (17:12:22)

Post-Processing Solution                (17:12:22)

Calculating Disp and Stress Results (17:12:22)

Checking Convergence                    (17:12:24)

Elements Not Converged:          0

Edges Not Converged:             0

Measure Convergence:          0.0%

Local Disp/Energy Index:      0.0%

Global RMS Stress Index:      0.0%

RMS Stress Error Estimates:

Load Set          Stress Error  % of Max Prin Str

----------------  ------------  -----------------

LoadSet1         1.48e-11      61.2% of  2.41e-11

Resource Check                          (17:12:25)

Elapsed Time     (sec):      11.86

CPU Time         (sec):      10.67

Memory Usage      (kb):      92592

Wrk Dir Dsk Usage (kb):          0

The analysis converged to within 1% on

measures.

Total Mass of Model:  6.145665e+00

Total Cost of Model:  0.000000e+00

Mass Moments of Inertia about WCS Origin:

Ixx:  2.43257e+06

Ixy:  7.29633e-07  Iyy:  7.67811e+05

Ixz:  7.27596e-12  Iyz: -1.45519e-11  Izz:  2.43257e+06

Principal MMOI and Principal Axes Relative to WCS Origin:

Max Prin           Mid Prin           Min Prin

2.43257e+06    2.43257e+06     7.67811e+05

WCS X:  0.00000e+00        1.00000e+00        0.00000e+00

WCS Y:  0.00000e+00        0.00000e+00        1.00000e+00

WCS Z:  1.00000e+00        0.00000e+00        0.00000e+00

Center of Mass Location Relative to WCS Origin:

(-3.00604e-14,  5.00000e+02,  9.24935e-15)

Mass Moments of Inertia about the Center of Mass:

Ixx:  8.96153e+05

Ixy:  7.29541e-07  Iyy:  7.67811e+05

Ixz:  7.27596e-12  Iyz:  1.38698e-11  Izz:  8.96153e+05

Principal MMOI and Principal Axes Relative to COM:

Max Prin           Mid Prin           Min Prin

8.96153e+05     8.96153e+05     7.67811e+05

WCS X:  0.00000e+00        1.00000e+00        0.00000e+00

WCS Y:  0.00000e+00        0.00000e+00        1.00000e+00

WCS Z:  1.00000e+00        0.00000e+00        0.00000e+00

Constraint Set: ConstraintSet1

Load Set: LoadSet1

Resultant Load on Model:

in global X direction: -4.841302e-08

in global Y direction: -3.551001e-08

in global Z direction: -3.806041e-08

Measures:

Name              Value      Convergence

--------------     -------------  -----------

max_beam_bending:   0.000000e+00      0.0%

max_beam_tensile:   0.000000e+00      0.0%

max_beam_torsion:   0.000000e+00      0.0%

max_beam_total:     0.000000e+00      0.0%

max_disp_mag:       3.889087e-01      0.0%

max_disp_x:         2.750000e-01      0.0%

max_disp_y:         2.750000e-01      0.0%

max_disp_z:         1.375000e-01      0.0%

max_prin_mag:      -2.411673e-11     51.5%

max_rot_mag:        0.000000e+00      0.0%

max_rot_x:          0.000000e+00      0.0%

max_rot_y:          0.000000e+00      0.0%

max_rot_z:          0.000000e+00      0.0%

max_stress_prin:    7.041881e-12     23.1%

max_stress_vm:      7.461390e-12     12.3%

max_stress_xx:     -1.966355e-11     52.2%

max_stress_xy:     -9.968400e-13     44.7%

max_stress_xz:      9.036722e-13     78.4%

max_stress_yy:     -2.408352e-11     52.4%

max_stress_yz:      8.794603e-13     17.8%

max_stress_zz:     -2.004753e-11     72.5%

min_stress_prin:   -2.411673e-11     51.5%

strain_energy:      1.024455e-08     77.3%

Diameter:           2.750000e-01      0.0%

Height:             2.750000e-01      0.0%

Analysis "BenchmarkSteel" Completed  (17:12:25)

--------------------------------------------------------

Memory and Disk Usage:

Machine Type: Windows NT/x86

RAM Allocation for Solver (megabytes): 48.0

Total Elapsed Time (seconds): 12.05

Total CPU Time     (seconds): 10.77

Maximum Memory Usage (kilobytes): 92592

Working Directory Disk Usage (kilobytes): 0

Results Directory Size (kilobytes):

1137 .\BenchmarkSteel

--------------------------------------------------------

Run Completed

Mon Mar 12, 2001   17:12:25

--------------------------------------------------------

### Run Summary for Split Block and Spring Contact Analysis

--------------------------------------------------------

Pro/MECHANICA STRUCTURE Version 22.3(305)

Summary for Design Study "FinalContact"

Tue Mar 06, 2001   17:32:35

--------------------------------------------------------

Run Settings

Memory allocation for block solver: 48.0

Pro/MECHANICA STRUCTURE Model Summary

Model Type: Three Dimensional

Points:        100

Edges:         347

Faces:         396

Springs:         0

Masses:          0

Beams:           0

Shells:         25

Solids:        149

Elements:      174

Contact Regions:          4

Links:          10

--------------------------------------------------------

Standard Design Study

Description:

Contact Analysis of the brass split block interface with the steel suspension spring and its pin and rivet

Static Analysis "FinalContact":

Contact Analysis

Convergence Method: Multiple-Pass Adaptive

Plotting Grid:      4

Convergence Loop Log: (17:32:40)

>> Pass  1 <<

Calculating Element Equations        (17:32:41)

Total Number of Equations:     324

Maximum Edge Order:              1

Solving Equations                       (17:32:42)

Load Increment 0 of 3

Load Factor:  0.00000e+00

Contact Area:  0.00000e+00

Calculating Disp and Stress Results (17:32:45)

Load Increment 1 of 3

Load Factor:  3.33333e-01

*Contact Area:  5.63673e+00

Calculating Disp and Stress Results (17:32:54)

Load Increment 2 of 3

Load Factor:  6.66667e-01

*Contact Area:  5.63673e+00

Calculating Disp and Stress Results (17:33:00)

Load Increment 3 of 3

Load Factor:  1.00000e+00

*Contact Area:  5.63673e+00

** Warning: Contact area is small in comparison to size of adjacent element edges for one or more contact regions for all load factors above marked with a "*". If you need pressure results near the contact regions, use single-pass adaptive convergence and select Localized Mesh Refinement.

Calculating Disp and Stress Results (17:33:05)

Post-Processing Solution                (17:33:07)

Checking Convergence                    (17:33:07)

Elements Not Converged:        174

Edges Not Converged:           347

Measure Convergence:        100.0%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:    100.0%

Resource Check                          (17:33:07)

Elapsed Time     (sec):      33.93

CPU Time         (sec):      28.66

Memory Usage      (kb):      83414

Wrk Dir Dsk Usage (kb):         26

>> Pass  2 <<

Calculating Element Equations    (17:33:09)

Total Number of Equations:    1469

Maximum Edge Order:              2

Solving Equations                       (17:33:09)

Load Increment 0 of 3

Load Factor:  0.00000e+00

Contact Area:  0.00000e+00

Calculating Disp and Stress Results (17:33:16)

Load Increment 1 of 3

Load Factor:  3.33333e-01

*Contact Area:  5.15557e+00

Calculating Disp and Stress Results (17:33:44)

Load Increment 2 of 3

Load Factor:  6.66667e-01

*Contact Area:  5.15557e+00

Calculating Disp and Stress Results (17:34:12)

Load Increment 3 of 3

Load Factor:  1.00000e+00

*Contact Area:  5.15557e+00

** Warning: Contact area is small in comparison to size of adjacent element edges for one or more contact regions for all load factors above marked with a "*". If you need pressure results near the contact regions, use single-pass adaptive convergence and select Localized Mesh Refinement.

Calculating Disp and Stress Results (17:34:40)

Post-Processing Solution                (17:34:41)

Checking Convergence                    (17:34:41)

Elements Not Converged:         89

Edges Not Converged:           264

Measure Convergence:          2.6%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:      0.8%

Resource Check                          (17:34:41)

Elapsed Time     (sec):     128.43

CPU Time         (sec):     121.35

Memory Usage      (kb):      84502

Wrk Dir Dsk Usage (kb):        178

>> Pass  3 <<

Calculating Element Equations        (17:34:45)

Total Number of Equations:    3542

Maximum Edge Order:              4

Solving Equations                       (17:34:47)

Load Increment 0 of 3

Load Factor:  0.00000e+00

Contact Area:  0.00000e+00

Calculating Disp and Stress Results (17:35:10)

Load Increment 1 of 3

Load Factor:  3.33333e-01

*Contact Area:  5.14344e+00

Calculating Disp and Stress Results (17:36:00)

Load Increment 2 of 3

Load Factor:  6.66667e-01

*Contact Area:  5.14344e+00

Calculating Disp and Stress Results (17:36:51)

Load Increment 3 of 3

Load Factor:  1.00000e+00

*Contact Area:  5.14344e+00

** Warning: Contact area is small in comparison to size of adjacent element edges for one or more contact regions for all load factors above marked with a "*". If you need pressure results near the contact regions, use single-pass adaptive convergence and select Localized Mesh Refinement.

Calculating Disp and Stress Results (17:37:41)

Post-Processing Solution                (17:37:43)

Checking Convergence                    (17:37:43)

Elements Not Converged:         43

Edges Not Converged:             0

Measure Convergence:          0.2%

Local Disp/Energy Index:    100.0%

Global RMS Stress Index:      0.7%

RMS Stress Error Estimates:

Load Set          Stress Error  % of Max Prin Str

----------------  ------------  -----------------

LoadSet1       1.38e+00       5.0% of  2.77e+01

Resource Check                          (17:37:45)

Elapsed Time     (sec):     312.21

CPU Time         (sec):     299.47

Memory Usage      (kb):      85078

Wrk Dir Dsk Usage (kb):       3619

The analysis converged to within 1% on measures.

Total Mass of Model:  1.339851e-06

Total Cost of Model:  0.000000e+00

Mass Moments of Inertia about WCS Origin:

Ixx:  2.66867e-04

Ixy:  3.62645e-05  Iyy:  9.77388e-06

Ixz:  2.97260e-12  Iyz: -1.03839e-11  Izz:  2.75649e-04

Principal MMOI and Principal Axes Relative to WCS Origin:

Max Prin           Mid Prin           Min Prin

2.75649e-04    2.71884e-04     4.75647e-06

WCS X:  4.05726e-07  9.90564e-01 -1.37050e-01

WCS Y:  1.62841e-08  1.37050e-01  9.90564e-01

WCS Z:  1.00000e+00  -4.04130e-07  3.94745e-08

Center of Mass Location Relative to WCS Origin:

(-2.00000e+00,  1.35331e+01,  4.83005e-07)

Mass Moments of Inertia about the Center of Mass:

Ixx:  2.14812e-05

Ixy:  7.50588e-12  Iyy:  4.41450e-06

Ixz: 1.67829e-12 Iyz: -1.62595e-12 Izz: 2.49039e-05

Principal MMOI and Principal Axes Relative to COM:

Max Prin           Mid Prin           Min Prin

2.49039e-05      2.14812e-05     4.41450e-06

WCS X:  4.90337e-07 1.00000e+00 -4.39797e-07

WCS Y: -7.93552e-08 4.39797e-07  1.00000e+00

WCS Z:  1.00000e+00  -4.90337e-07  7.93554e-08

Constraint Set: ConstraintSet1

Load Set: LoadSet1

Resultant Load on Model:

in global X direction: -1.079579e-12

in global Y direction: -8.966466e-12

in global Z direction:  1.294446e-12

Measures:

Name              Value      Convergence

--------------     -------------  -----------

contact_area:       5.143444e+00      0.2%

contact_max_pres:   4.292490e+00     30.6%

max_beam_bending:   0.000000e+00      0.0%

max_beam_tensile:   0.000000e+00      0.0%

max_beam_torsion:   0.000000e+00      0.0%

max_beam_total:     0.000000e+00      0.0%

max_disp_mag:       7.884895e-03      0.0%

max_disp_x:         2.284773e-03      0.3%

max_disp_y:         7.542785e-03      0.0%

max_disp_z:        -5.843065e-04      0.5%

max_prin_mag:       2.773742e+01      9.1%

max_rot_mag:        9.356744e-05     10.6%

max_rot_x:         -6.858245e-06     44.1%

max_rot_y:         -3.825938e-06     15.7%

max_rot_z:         -9.338629e-05     10.5%

max_stress_prin:    2.773742e+01      9.1%

max_stress_vm:      2.772074e+01     13.1%

max_stress_xx:     -5.235828e+00     76.3%

max_stress_xy:     -1.124145e+01      4.5%

max_stress_xz:      6.339312e-01      3.9%

max_stress_yy:      2.767054e+01      9.5%

max_stress_yz:     -1.341582e+00     11.4%

max_stress_zz:     -7.425485e+00     34.4%

min_stress_prin:   -1.630854e+01      9.9%

strain_energy:      7.571377e-04     52.3%

BlockDispl:         1.837009e-03      0.2%

cntRgn_001cntArea:  1.142975e+00      0.8%

cntRgn_001maxPres:  4.292490e+00     30.6%

cntRgn_002cntArea:  1.434668e+00      0.0%

cntRgn_002maxPres:  2.604255e+00     56.2%

cntRgn_003cntArea:  1.139767e+00      1.3%

cntRgn_003maxPres:  4.292490e+00     30.6%

cntRgn_004cntArea:  1.426034e+00      0.8%

cntRgn_004maxPres:  2.604255e+00     56.2%

Analysis "FinalContact" Completed  (17:37:45)

--------------------------------------------------------

Memory and Disk Usage:

Machine Type: Windows NT/x86

RAM Allocation for Solver (megabytes): 48.0

Total Elapsed Time (seconds): 312.44

Total CPU Time     (seconds): 299.67

Maximum Memory Usage (kilobytes): 85078

Working Directory Disk Usage (kilobytes): 3619

Results Directory Size (kilobytes):

2723 .\FinalContact

Maximum Data Base Working File Sizes (kilobytes):

1024 .\FinalContact.tmp\gapel1.bas

2048 .\FinalContact.tmp\kel1.bas

--------------------------------------------------------

Run Completed

Tue Mar 06, 2001   17:37:45

--------------------------------------------------------

## Finite Element Analysis Report Files

The following are the Reports produced by ProMechanica indicating the displacements of the points set as measures in the Benchmark Test Models and the Contact analysis Model.

### Carbon Fibre Displacement Results

# MECHANICA Graph Report File

#    Product:		Pro/MECHANICA(R) STRUCTURE 22.3(305)

#    Created:		03/12/01 at 17:17:25

#    Machine:		i486_nt

#    Graphics:		opengl/'window0'

#    Language:		(usascii)

# title = MeasuresCF - ..\..\..\..\WINNT\Profiles\9731318\Desktop\BenchMarkCF - BenchMarkCF

# independent variable label = P Loop Pass

# dependent variable label = Height

# col    data item

#  1     P Loop Pass

#  2     Height

# number of rows = 4

1.0000000000000E+000   -2.0629180595279E-002

2.0000000000000E+000   -1.9941110163927E-002

3.0000000000000E+000   -1.9999999552965E-002

4.0000000000000E+000   -1.9999999552965E-002

### Brass Displacement Results

# MECHANICA Graph Report File

#    Product:		Pro/MECHANICA(R) STRUCTURE 22.3(305)

#    Created:		03/12/01 at 17:20:30

#    Machine:		i486_nt

#    Graphics:		opengl/'window0'

#    Language:		(usascii)

# title = MeasuresBrass - ..\..\..\..\WINNT\Profiles\9731318\Desktop\BenchmarkBrass - BenchmarkBrass

# independent variable label = P Loop Pass

# dependent variable label = Height

# col    data item

#  1     P Loop Pass

#  2     Height

# number of rows = 4

1.0000000000000E+000    4.9085310101509E-001

2.0000000000000E+000    4.7341009974480E-001

3.0000000000000E+000    4.7499999403954E-001

4.0000000000000E+000    4.7499999403954E-001

### Steel Displacement Results

# MECHANICA Graph Report File

#    Product:		Pro/MECHANICA(R) STRUCTURE 22.3(305)

#    Created:		03/12/01 at 17:16:24

#    Machine:		i486_nt

#    Graphics:		opengl/'window0'

#    Language:		(usascii)

# title = Measures - ..\..\..\..\WINNT\Profiles\9731318\Desktop\BenchmarkSteel - BenchmarkSteel

# independent variable label = P Loop Pass

# dependent variable label = Height

# col    data item

#  1     P Loop Pass

#  2     Height

# number of rows = 4

1.0000000000000E+000    2.8332769870758E-001

2.0000000000000E+000    2.7425619959831E-001

3.0000000000000E+000    2.7500000596046E-001

4.0000000000000E+000    2.7500000596046E-001

### Result for displacement of a point at the end of the Split Block calculated using Finite Element Analysis

# MECHANICA Graph Report File

#    Product:		Pro/MECHANICA(R) STRUCTURE 22.3(305)

#    Created:		03/06/01 at 23:09:11

#    Machine:		i486_nt

#    Graphics:		opengl/'window0'

#

#    Language:		(usascii)

#

# title = Measure - ceason\topspring\FinalContact - FinalContact

#

# independent variable label = P Loop Pass

# dependent variable label = BlockDispl

#

# col    data item

#  1     P Loop Pass

#  2     BlockDispl

#

# number of rows = 3

1.0000000000000E+000    1.8873759545386E-003

2.0000000000000E+000    1.8402209971100E-003

3.0000000000000E+000    1.8370089819655E-003

## Summary of Displacement Calculation Results

| Displacement Benchmark Test For Steel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Coefficient of Expansion: |  |  | 1.1E-05 m/m°C |  |  |
| From Finite Element Analysis: |  |  | Manual Calculation: |  |  |
| Model: | Cylinder |  | Change in T | 25°C |  |
| Dimensions: |  |  | Dimensions: |  |  |
| Diameter | 1000 mm |  | Diameter | 1000 mm |  |
| H | 1000 mm |  | H | 1000 mm |  |
| Volume: | 785398163.4 mm³ |  | Volume: | 785398163.4 mm³ |  |
| In millilitres | 785398.1634 ml |  | In millilitres | 785398.1634 ml |  |
| Deformed Dimensions: |  |  | Deformed Dimensions: |  | % Error |
| Diameter | 1000.27500000596 mm |  | Diameter | 1000.275 mm | 5.95885E-10 |
| H | 1000.27500000596 mm |  | H | 1000.275 mm | 5.95885E-10 |
| Volume: | 786046295.1 mm³ |  | Volume: | 786046295.1 mm³ | 1.78765E-09 |
| In millilitres | 786046.2951 ml |  | In millilitres | 786046.2951 ml | 1.78765E-09 |
| Change in Vol: | 648131.7024 mm³ |  | Change in Vol: | 648131.6883 mm³ | 2.16805E-06 |
| In millilitres | 648.1317024 ml |  | In millilitres | 648.1316883 ml | 2.16804E-06 |

| Displacement Benchmark Test For Brass |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Coefficient of Expansion: |  |  | 1.9E-05 m/m°C |  |  |
| From Finite Element Analysis: |  |  | Manual Calculation: |  |  |
| Model: | Cylinder |  | Change in T | 25°C |  |
| Dimensions: |  |  | Dimensions: |  |  |
| Diameter | 1000 mm |  | Diameter | 1000 mm |  |
| H | 1000 mm |  | H | 1000 mm |  |
| Volume: | 785398163.4 mm³ |  | Volume: | 785398163.4 mm³ |  |
| In millilitres | 785398.1634 ml |  | In millilitres | 785398.1634 ml |  |
| Deformed Dimensions: |  |  | Deformed Dimensions: |  | % Error |
| Diameter | 1000.47499999403 mm |  | Diameter | 1000.475 mm | -5.95766E-10 |
| H | 1000.47499999403 mm |  | H | 1000.475 mm | -5.95766E-10 |
| Volume: | 786517887.5 mm³ |  | Volume: | 786517887.5 mm³ | -1.7873E-09 |
| In millilitres | 786517.8875 ml |  | In millilitres | 786517.8875 ml | -1.78729E-09 |
| Change in Vol: | 1119724.069 mm³ |  | Change in Vol: | 1119724.083 mm³ | -1.25543E-06 |
| In millilitres | 1119.724069 ml |  | In millilitres | 1119.724083 ml | -1.25544E-06 |

| Displacement Benchmark Test For Carbon Fibre |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Coefficient of Expansion: |  |  | -8.0E-07 m/m°C |  |  |
| From Finite Element Analysis: |  |  | Manual Calculation: |  |  |
| Model: | Cylinder |  | Change in T | 25°C |  |
| Dimensions: |  |  | Dimensions: |  |  |
| Diameter | 1000 mm |  | Diameter | 1000 mm |  |
| H | 1000 mm |  | H | 1000 mm |  |
| Volume: | 785398163.4 mm³ |  | Volume: | 785398163.4 mm³ |  |
| In millilitres | 785398.1634 ml |  | In millilitres | 785398.1634 ml |  |
| Deformed Dimensions: |  |  | Deformed Dimensions: |  | % Error |
| Diameter | 999.98000000045 mm |  | Diameter | 999.98 mm | 4.47026E-11 |
| H | 999.98000000045 mm |  | H | 999.98 mm | 4.47026E-11 |
| Volume: | 785351040.5 mm³ |  | Volume: | 785351040.5 mm³ | 1.34123E-10 |
| In millilitres | 785351.0405 ml |  | In millilitres | 785351.0405 ml | 1.34123E-10 |
| Change in Vol: | -47122.94628 mm³ |  | Change in Vol: | -47122.9473 mm³ | -2.23529E-06 |
| In millilitres | -47.12294628 ml |  | In millilitres | -47.12294733 ml | -2.23518E-06 |

| Nomenclature: |  |
| --- | --- |
| T | Temperature |
| H | Height of Cylinder |
| Vol | Volume |
| Units are as given with each value. |  |

Conclusion: Finite Element Analysis Results Agree very closely with theoretical answers, but rounding error affects some zero values.

## Raw Data from Low Temperature Tests

| Time | Temperature | Time | Period |  | Time | Temperature | Time | Period |  | Time | Temperature | Time | Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 17.065 | 7.0701959 | 707019.59 |  | 0 | 17.554 | 7.0702388 | 707023.88 |  | 0 | 11.768 | 7.070678 | 707067.82 |
| 5 | 17.798 | 14.1404117 | 707021.58 |  | 5 | 18.091 | 14.1399361 | 706969.73 |  | 5 | 9.985 | 14.13997 | 706928.68 |
| 10 | 17.114 | 21.210431 | 707001.93 |  | 10 | 12.524 | 21.2093337 | 706939.76 |  | 10 | 17.139 | 21.20896 | 706899.34 |
| 15 | 13.574 | 28.2803727 | 706994.17 |  | 15 | 17.7 | 28.278576 | 706924.23 |  | 15 | 17.139 | 28.27778 | 706881.88 |
| 20 | 10.229 | 35.3500917 | 706971.9 |  | 20 | 15.552 | 35.347603 | 706902.7 |  | 20 | 18.188 | 35.34637 | 706859.59 |
| 25 | 17.798 | 42.41972 | 706962.83 |  | 25 | 16.919 | 42.4164963 | 706889.33 |  | 25 | 18.97 | 42.41485 | 706847.66 |
| 30 | 18.799 | 49.4891671 | 706944.71 |  | 30 | 4.443 | 49.4852193 | 706872.3 |  | 30 | 14.014 | 49.48312 | 706827.24 |
| 35 | 19.458 | 56.5585544 | 706938.73 |  | 35 | 17.651 | 56.5538526 | 706863.33 |  | 35 | 16.528 | 56.55128 | 706816.02 |
| 40 | 18.701 | 63.62775 | 706919.56 |  | 40 | 15.063 | 63.6222708 | 706841.82 |  | 40 | 17.48 | 63.61927 | 706798.82 |
| 45 | 17.993 | 70.696897 | 706914.7 |  | 45 | 18.14 | 70.6905929 | 706832.21 |  | 45 | 18.115 | 70.68721 | 706793.58 |
| 50 | 16.895 | 77.7658744 | 706897.74 |  | 50 | 14.966 | 77.7587648 | 706817.19 |  | 50 | 17.505 | 77.75497 | 706776.72 |
| 55 | 17.578 | 84.8348357 | 706896.13 |  | 55 | 17.407 | 84.8268701 | 706810.53 |  | 55 | 9.351 | 84.82264 | 706767.1 |
| 60 | 16.846 | 91.9036348 | 706879.91 |  | 60 | 9.253 | 91.8947789 | 706790.88 |  | 60 | 18.262 | 91.89018 | 706753.34 |
| 65 | 14.844 | 98.9724017 | 706876.69 |  | 65 | 18.237 | 98.9626087 | 706782.98 |  | 65 | 17.871 | 98.95765 | 706747.44 |
| 70 | 17.847 | 106.0410141 | 706861.24 |  | 70 | 17.285 | 106.0303069 | 706769.82 |  | 70 | 17.627 | 106.025 | 706733.84 |
| 75 | 16.992 | 113.1096093 | 706859.52 |  | 75 | 17.261 | 113.097917 | 706761.01 |  | 75 | 18.115 | 113.0923 | 706729.94 |
| 80 | 16.675 | 120.1780275 | 706841.82 |  | 80 | 17.944 | 120.1653718 | 706745.48 |  | 80 | 16.724 | 120.1594 | 706715.74 |
| 85 | 17.798 | 127.2464589 | 706843.14 |  | 85 | 5.859 | 127.232749 | 706737.72 |  | 85 | 17.7 | 127.2266 | 706713.53 |
| 90 | 17.31 | 134.3147067 | 706824.78 |  | 90 | 19.165 | 134.2999556 | 706720.66 |  | 90 | 16.992 | 134.2936 | 706699.36 |
| 95 | 17.7 | 141.3829465 | 706823.98 |  | 95 | 18.481 | 141.3671309 | 706717.53 |  | 95 | 16.846 | 141.3605 | 706695.59 |
| 100 | 17.944 | 148.4510504 | 706810.39 |  | 100 | 17.48 | 148.4341555 | 706702.46 |  | 100 | 17.383 | 148.4274 | 706683.41 |
| 105 | 17.798 | 155.5191607 | 706811.03 |  | 105 | 17.578 | 155.5011704 | 706701.49 |  | 105 | 2.49 | 155.4942 | 706684.37 |
| 110 | 16.602 | 162.5871403 | 706797.96 |  | 110 | 17.725 | 162.5679876 | 706681.72 |  | 110 | 15.723 | 162.5609 | 706669 |
| 115 | 12.769 | 169.6551076 | 706796.73 |  | 115 | 17.407 | 169.6347675 | 706677.99 |  | 115 | 18.091 | 169.6276 | 706670.04 |
| 120 | 13.965 | 176.722955 | 706784.74 |  | 120 | 1.587 | 176.7014116 | 706664.41 |  | 120 | 10.889 | 176.6942 | 706659.48 |
| 125 | 17.92 | 183.7908523 | 706789.73 |  | 125 | 17.822 | 183.7680279 | 706661.63 |  | 125 | 16.846 | 183.7608 | 706657.93 |
| 130 | 16.992 | 190.8585831 | 706773.08 |  | 130 | 18.726 | 190.8344785 | 706645.06 |  | 130 | 17.285 | 190.8273 | 706647.9 |
| 135 | 16.479 | 197.9263397 | 706775.66 |  | 135 | 17.017 | 197.9008853 | 706640.68 |  | 135 | 17.163 | 197.8937 | 706645.04 |
| 140 | 16.87 | 204.9939692 | 706762.95 |  | 140 | 17.065 | 204.9671605 | 706627.52 |  | 140 | 18.628 | 204.9601 | 706636.04 |
| 145 | 18.018 | 212.0615826 | 706761.34 |  | 145 | 12.378 | 212.0334159 | 706625.54 |  | 145 | 12.036 | 212.0264 | 706636.22 |
| 150 | 16.772 | 219.1290629 | 706748.03 |  | 150 | 17.529 | 219.0994955 | 706607.96 |  | 150 | 17.505 | 219.0927 | 706626.82 |
| 155 | 17.505 | 226.1966302 | 706756.73 |  | 155 | 16.992 | 226.1655901 | 706609.46 |  | 155 | 16.968 | 226.1589 | 706624.8 |
| 160 | 16.333 | 233.2640408 | 706741.06 |  | 160 | 17.456 | 233.2315979 | 706600.78 |  | 160 | 18.408 | 233.2251 | 706617.23 |
| 165 | 17.603 | 240.331557 | 706751.62 |  | 165 | 16.943 | 240.2975804 | 706598.25 |  | 165 | 17.31 | 240.2914 | 706623.57 |
| 170 | 18.091 | 247.3988687 | 706731.17 |  | 170 | 15.747 | 247.3634287 | 706584.83 |  | 170 | 13.281 | 247.3575 | 706612.72 |
| 175 | 16.943 | 254.4662833 | 706741.46 |  | 175 | 17.31 | 254.4293066 | 706587.79 |  | 175 | 16.968 | 254.4236 | 706616.34 |
| 180 | 17.676 | 261.5335435 | 706726.02 |  | 180 | 17.212 | 261.4950096 | 706570.3 |  | 180 | 17.041 | 261.4897 | 706607.11 |
| 185 | 16.992 | 268.6008437 | 706730.02 |  | 185 | 17.041 | 268.5607523 | 706574.27 |  | 185 | 11.743 | 268.5558 | 706604.05 |
| 190 | 17.163 | 275.6680066 | 706716.29 |  | 190 | 17.041 | 275.6264053 | 706565.3 |  | 190 | 18.604 | 275.6217 | 706597.97 |
| 195 | 13.916 | 282.7352864 | 706727.98 |  | 195 | 17.358 | 282.6920026 | 706559.73 |  | 195 | 8.765 | 282.6878 | 706604.7 |
| 200 | 20.117 | 289.8023843 | 706709.79 |  | 200 | 17.212 | 289.7574582 | 706545.56 |  | 200 | 16.626 | 289.7537 | 706594.18 |
| 205 | 11.06 | 296.8694913 | 706710.7 |  | 205 | 17.969 | 296.8229326 | 706547.44 |  | 205 | 17.334 | 296.8197 | 706597.15 |
| 210 | 20.435 | 303.9364826 | 706699.13 |  | 210 | 15.723 | 303.8883007 | 706536.81 |  | 210 | 16.016 | 303.8856 | 706592.73 |
| 215 | 13.062 | 311.0036021 | 706711.95 |  | 215 | 17.285 | 310.9537432 | 706544.25 |  | 215 | 16.919 | 310.9516 | 706598.22 |
| 220 | 10.693 | 318.0705509 | 706694.88 |  | 220 | 18.286 | 318.019067 | 706532.38 |  | 220 | 16.553 | 318.0174 | 706583.25 |
| 225 | 7.056 | 325.1376144 | 706706.35 |  | 225 | 17.92 | 325.084456 | 706538.9 |  | 225 | 18.262 | 325.0834 | 706592.42 |
| 230 | 11.011 | 332.2045284 | 706691.4 |  | 230 | 16.797 | 332.1496987 | 706524.27 |  | 230 | 17.358 | 332.1492 | 706581.34 |
| 235 | 17.114 | 339.2715301 | 706700.17 |  | 235 | 17.212 | 339.2149195 | 706522.08 |  | 235 | 16.602 | 339.215 | 706585.52 |
| 240 | 17.432 | 346.3383442 | 706681.41 |  | 240 | 16.699 | 346.2800093 | 706508.98 |  | 240 | 17.212 | 346.2807 | 706571.52 |
| 245 | 17.725 | 353.4053125 | 706696.83 |  | 245 | 11.108 | 353.3450535 | 706504.42 |  | 245 | 17.163 | 353.3465 | 706575.7 |
| 250 | 17.407 | 360.4721223 | 706680.98 |  | 250 | 17.236 | 360.4099925 | 706493.9 |  | 250 | 17.383 | 360.4122 | 706566.58 |
| 255 | 17.505 | 367.5389625 | 706684.02 |  | 255 | 16.992 | 367.475082 | 706508.95 |  | 255 | 18.018 | 367.4779 | 706574.02 |
| 260 | 17.676 | 374.6056565 | 706669.4 |  | 260 | 8.936 | 374.5400092 | 706492.72 |  | 260 | 17.7 | 374.5435 | 706561.78 |
| 265 | 16.772 | 381.672428 | 706677.15 |  | 265 | 17.896 | 381.6050503 | 706504.11 |  | 265 | 12.891 | 381.6092 | 706570.41 |
| 270 | 18.701 | 388.7390036 | 706657.56 |  | 270 | 17.09 | 388.6699251 | 706487.48 |  | 270 | 18.066 | 388.6749 | 706563.68 |
| 275 | 16.992 | 395.8057915 | 706678.79 |  | 275 | 10.547 | 395.7348641 | 706493.9 |  | 275 | 8.765 | 395.7406 | 706570.06 |
| 280 | 17.383 | 402.8723742 | 706658.27 |  | 280 | 19.678 | 402.7997389 | 706487.48 |  | 280 | 16.919 | 402.8062 | 706560.73 |
| 285 | 17.041 | 409.9390007 | 706662.65 |  | 285 | 16.602 | 409.8646763 | 706493.74 |  | 285 | 16.235 | 409.8718 | 706565.94 |
| 290 | 17.041 | 417.0054936 | 706649.29 |  | 290 | 15.283 | 416.929361 | 706468.47 |  | 290 | 17.017 | 416.9375 | 706565.24 |
| 295 | 16.87 | 424.0721674 | 706667.38 |  | 295 | 17.456 | 423.9940572 | 706469.62 |  | 295 | 17.432 | 424.0031 | 706559.81 |
| 300 | 17.871 | 431.1386433 | 706647.59 |  | 300 | 17.651 | 431.0587728 | 706471.56 |  | 300 | 17.261 | 431.0686 | 706547.23 |
| 305 | 16.797 | 438.2052349 | 706659.16 |  | 305 | 2.002 | 438.1235225 | 706474.97 |  | 305 | 16.968 | 438.1341 | 706555.43 |
| 310 | 16.87 | 445.2717011 | 706646.62 |  | 310 | 17.993 | 445.188175 | 706465.25 |  | 310 | 17.603 | 445.1997 | 706558.61 |
| 315 | 17.432 | 452.3382828 | 706658.17 |  | 315 | 17.725 | 452.25293 | 706475.5 |  | 315 | 16.602 | 452.2652 | 706553.02 |
| 320 | 14.771 | 459.4046802 | 706639.74 |  | 320 | 17.48 | 459.3175305 | 706460.05 |  | 320 | 16.699 | 459.3306 | 706540.3 |
| 325 | 15.381 | 466.4712595 | 706657.93 |  | 325 | 17.285 | 466.3821719 | 706464.14 |  | 325 | 16.943 | 466.3962 | 706558.76 |
| 330 | 8.252 | 473.5376668 | 706640.73 |  | 330 | 16.504 | 473.4465827 | 706441.08 |  | 330 | 18.115 | 473.4617 | 706553.31 |
| 335 | 16.382 | 480.6041879 | 706652.11 |  | 335 | 17.7 | 480.5111205 | 706453.78 |  | 335 | 8.618 | 480.5274 | 706561.2 |
| 340 | 17.871 | 487.670554 | 706636.61 |  | 340 | 8.838 | 487.5756067 | 706448.62 |  | 340 | 18.604 | 487.5928 | 706542.13 |
| 345 | 17.7 | 494.7371224 | 706656.84 |  | 345 | 17.31 | 494.6401354 | 706452.87 |  | 345 | 7.544 | 494.6583 | 706554.99 |
| 350 | 17.603 | 501.8034607 | 706633.83 |  | 350 | 17.017 | 501.704514 | 706437.86 |  | 350 | 16.65 | 501.7237 | 706536.2 |
| 355 | 18.335 | 508.8700225 | 706656.18 |  | 355 | 16.699 | 508.7689269 | 706441.29 |  | 355 | 16.919 | 508.7892 | 706547.25 |
| 360 | 17.944 | 515.9363481 | 706632.56 |  | 360 | 17.944 | 515.8332569 | 706433 |  | 360 | 16.26 | 515.8545 | 706536.6 |
| 365 | 17.7 | 523.0028187 | 706647.06 |  | 365 | 16.919 | 522.8977845 | 706452.76 |  | 365 | 17.847 | 522.92 | 706551.32 |
| 370 | 17.847 | 530.0690691 | 706625.04 |  | 370 | 7.764 | 529.962077 | 706429.25 |  | 370 | 9.277 | 529.9854 | 706538.69 |
| 375 | 16.504 | 537.1355155 | 706644.64 |  | 375 | 17.236 | 537.0265552 | 706447.82 |  | 375 | 17.969 | 537.051 | 706554.11 |
| 380 | 17.139 | 544.2016591 | 706614.36 |  | 380 | 17.114 | 544.0909449 | 706438.97 |  | 380 | 17.09 | 544.1163 | 706535.48 |
| 385 | 18.457 | 551.2680494 | 706639.03 |  | 385 | 17.773 | 551.1553541 | 706440.92 |  | 385 | 17.139 | 551.1818 | 706545.24 |
| 390 | 17.456 | 558.3342801 | 706623.07 |  | 390 | 16.968 | 558.2196631 | 706430.9 |  | 390 | 18.115 | 558.2471 | 706536.4 |
| 395 | 17.554 | 565.400651 | 706637.09 |  | 395 | 17.871 | 565.2841591 | 706449.6 |  | 395 | 18.091 | 565.3126 | 706544.03 |
| 400 | 16.528 | 572.466856 | 706620.5 |  | 400 | 16.87 | 572.3483715 | 706421.24 |  | 400 | 14.868 | 572.3779 | 706532.86 |
| 405 | 16.626 | 579.5333806 | 706652.46 |  | 405 | 17.456 | 579.412888 | 706451.65 |  | 405 | 17.261 | 579.4434 | 706543.74 |
| 410 | 17.285 | 586.599598 | 706621.74 |  | 410 | 13.159 | 586.4771258 | 706423.78 |  | 410 | 14.966 | 586.5088 | 706541.47 |
| 415 | 17.603 | 593.6660251 | 706642.71 |  | 415 | 17.017 | 593.5416412 | 706451.54 |  | 415 | 10.547 | 593.5744 | 706559.42 |
| 420 | 17.773 | 600.7322417 | 706621.66 |  | 420 | 17.383 | 600.6059036 | 706426.24 |  | 420 | 17.383 | 600.6397 | 706533.84 |
| 425 | 19.214 | 607.7986635 | 706642.18 |  | 425 | 17.139 | 607.670261 | 706435.74 |  | 425 | 17.676 | 607.7052 | 706549.3 |
| 430 | 17.163 | 614.8649083 | 706624.48 |  | 430 | 18.921 | 614.734431 | 706417 |  | 430 | 8.521 | 614.7706 | 706541.53 |
| 435 | 16.187 | 621.9314991 | 706659.08 |  | 435 | 12.085 | 621.7988548 | 706442.38 |  | 435 | 17.7 | 621.8361 | 706547.76 |
| 440 | 15.283 | 628.9979043 | 706640.52 |  | 440 | 18.066 | 628.8632338 | 706437.9 |  | 440 | 4.321 | 628.9014 | 706530.94 |
| 445 | 12.549 | 636.0645616 | 706665.73 |  | 445 | 17.261 | 635.92749 | 706425.62 |  | 445 | 17.603 | 635.9669 | 706548.11 |
| 450 | 9.229 | 643.1310337 | 706647.21 |  | 450 | 17.09 | 642.9916028 | 706411.28 |  | 450 | 13.599 | 643.0323 | 706544.61 |
| 455 | 17.139 | 650.1976579 | 706662.42 |  | 455 | 17.09 | 650.0559166 | 706431.38 |  | 455 | 17.7 | 650.0979 | 706557.91 |
| 460 | 17.065 | 657.2638973 | 706623.94 |  | 460 | 14.941 | 657.1201108 | 706419.42 |  | 460 | 15.332 | 657.1632 | 706531.53 |
| 465 | 17.432 | 664.3306246 | 706672.73 |  | 465 | 17.31 | 664.1846388 | 706452.8 |  | 465 | 11.67 | 664.2288 | 706559.24 |
| 470 | 17.871 | 671.39685 | 706622.54 |  | 470 | 17.163 | 671.2487804 | 706414.16 |  | 470 | 17.383 | 671.2943 | 706545.93 |
| 475 | 16.919 | 678.4632877 | 706643.77 |  | 475 | 17.603 | 678.3131982 | 706441.78 |  | 475 | 17.065 | 678.36 | 706572.84 |
| 480 | 17.261 | 685.5295869 | 706629.92 |  | 480 | 17.163 | 685.377612 | 706441.38 |  | 480 | 17.187 | 685.4256 | 706556.7 |
| 485 | 18.286 | 692.5961185 | 706653.16 |  | 485 | 13.452 | 692.4422582 | 706464.62 |  | 485 | 17.822 | 692.4911 | 706549.62 |
| 490 | 17.334 | 699.6622493 | 706613.08 |  | 490 | 17.261 | 699.5067429 | 706448.47 |  | 490 | 17.798 | 699.5565 | 706545.88 |
| 495 | 17.578 | 706.7287543 | 706650.5 |  | 495 | 17.285 | 706.5714085 | 706466.56 |  | 495 | 16.919 | 706.6222 | 706566.02 |
| 500 | 19.116 | 713.795136 | 706638.17 |  | 500 | 17.139 | 713.6358595 | 706445.1 |  | 500 | 17.236 | 713.6877 | 706555.36 |
| 505 | 17.676 | 720.8618198 | 706668.38 |  | 505 | 16.821 | 720.7003759 | 706451.64 |  | 505 | 12.964 | 720.7535 | 706572.64 |
| 510 | 17.261 | 727.9281537 | 706633.39 |  | 510 | 17.139 | 727.7646472 | 706427.13 |  | 510 | 17.92 | 727.819 | 706555.86 |
| 515 | 18.14 | 734.9948203 | 706666.66 |  | 515 | 14.087 | 734.8293972 | 706475 |  | 515 | 15.039 | 734.8847 | 706566.86 |
| 520 | 17.456 | 742.0611714 | 706635.11 |  | 520 | 17.187 | 741.893764 | 706436.68 |  | 520 | 17.065 | 741.9503 | 706565.18 |
| 525 | 9.937 | 749.1280736 | 706690.22 |  | 525 | 17.139 | 748.958352 | 706458.8 |  | 525 | 10.229 | 749.0161 | 706577.41 |
| 530 | 16.797 | 756.194374 | 706630.04 |  | 530 | 16.724 | 756.0227119 | 706435.99 |  | 530 | 16.797 | 756.0817 | 706555.34 |
| 535 | 16.846 | 763.2612337 | 706685.97 |  | 535 | 17.48 | 763.0875119 | 706480 |  | 535 | 18.457 | 763.1475 | 706582.82 |
| 540 | 16.895 | 770.3277076 | 706647.39 |  | 540 | 17.725 | 770.1519578 | 706444.59 |  | 540 | 17.627 | 770.213 | 706551.35 |
| 545 | 17.725 | 777.3948382 | 706713.06 |  | 545 | 12.476 | 777.216697 | 706473.92 |  | 545 | 17.432 | 777.2792 | 706616.68 |
| 550 | 17.871 | 784.4614061 | 706656.79 |  | 550 | 17.017 | 784.2811724 | 706447.54 |  | 550 | 16.504 | 784.3447 | 706554.3 |
| 555 | 15.601 | 791.5285708 | 706716.47 |  | 555 | 17.212 | 791.346151 | 706497.86 |  | 555 | 17.383 | 791.4107 | 706601.23 |
| 560 | 14.258 | 798.5953014 | 706673.06 |  | 560 | 16.968 | 798.4105575 | 706440.65 |  | 560 | 9.961 | 798.4766 | 706582.23 |
| 565 | 12.744 | 805.6624871 | 706718.57 |  | 565 | 16.382 | 805.4754157 | 706485.82 |  | 565 | 18.359 | 805.5426 | 706609.02 |
| 570 | 7.983 | 812.729153 | 706666.59 |  | 570 | 18.286 | 812.5400549 | 706463.92 |  | 570 | 17.969 | 812.6085 | 706588.1 |
| 575 | 16.992 | 819.7963693 | 706721.63 |  | 575 | 8.203 | 819.6050363 | 706498.14 |  | 575 | 17.212 | 819.6747 | 706622.34 |
| 580 | 16.992 | 826.8629615 | 706659.22 |  | 580 | 16.968 | 826.669513 | 706447.67 |  | 580 | 17.358 | 826.7406 | 706587.83 |
| 585 | 16.821 | 833.930593 | 706763.15 |  | 585 | 17.896 | 833.7347021 | 706518.91 |  | 585 | 14.087 | 833.807 | 706632.98 |
| 590 | 17.871 | 840.9972205 | 706662.75 |  | 590 | 17.603 | 840.7992652 | 706456.31 |  | 590 | 17.944 | 840.8728 | 706588.22 |
| 595 | 16.968 | 848.0648222 | 706760.17 |  | 595 | 18.237 | 847.8644322 | 706516.7 |  | 595 | 17.798 | 847.9392 | 706633.29 |
| 600 | 17.529 | 855.1316764 | 706685.42 |  | 600 | 16.553 | 854.9290983 | 706466.61 |  | 600 | 16.968 | 855.0051 | 706590.62 |
| 605 | 18.164 | 862.1995841 | 706790.77 |  | 605 | 11.06 | 861.9945951 | 706549.68 |  | 605 | 16.943 | 862.0715 | 706645.54 |
| 610 | 17.773 | 869.2659973 | 706641.32 |  | 610 | 17.017 | 869.0594366 | 706484.15 |  | 610 | 13.965 | 869.1378 | 706622.79 |
| 615 | 17.651 | 876.3325814 | 706658.41 |  | 615 | 16.504 | 876.1247034 | 706526.68 |  | 615 | 17.603 | 876.2047 | 706692.14 |
| 620 | 17.09 | 883.3990776 | 706649.62 |  | 620 | 18.628 | 883.1894574 | 706475.4 |  | 620 | 17.847 | 883.2708 | 706615.08 |
| 625 | 17.017 | 890.4656379 | 706656.03 |  | 625 | 16.992 | 890.2552526 | 706579.52 |  | 625 | 17.725 | 890.3377 | 706689.74 |
| 630 | 16.992 | 897.5318379 | 706620 |  | 630 | 17.896 | 897.32021 | 706495.74 |  | 630 | 17.847 | 897.4041 | 706634.63 |
| 635 | 17.163 | 904.5982646 | 706642.67 |  | 635 | 17.334 | 904.386069 | 706585.9 |  | 635 | 17.285 | 904.4708 | 706672.72 |
| 640 | 17.09 | Avg: | 706717.3942 |  | 640 | 8.813 | Avg: | 706551.62 |  | 640 | 17.09 | Avg: | 706617.81 |
| 645 | 16.895 |  |  |  | 645 | 17.871 |  |  |  | 645 | 16.87 |  |  |
| 650 | 12.964 |  |  |  | 650 | 16.431 |  |  |  | 650 | 11.499 |  | 706628.94 |
| 655 | 15.259 |  |  |  | 655 | 17.847 |  |  |  | 655 | 17.798 |  |  |
| 660 | 16.504 |  |  |  | 660 | 17.847 |  |  |  | 660 | 18.579 |  |  |
| 665 | 16.968 |  |  |  | 665 | 17.407 |  |  |  | 665 | 18.237 |  |  |
| 670 | 16.821 |  |  |  | 670 | 18.213 |  |  |  | 670 | 16.748 |  |  |
| 675 | 17.334 |  |  |  | 675 | 17.114 |  |  |  | 675 | 17.383 |  |  |
| 680 | 18.14 |  |  |  | 680 | 14.917 |  |  |  | 680 | 17.31 |  |  |
| 685 | 17.261 |  |  |  | 685 | 16.748 |  |  |  | 685 | 14.136 |  |  |
| 690 | 17.822 |  |  |  | 690 | 18.018 |  |  |  | 690 | 18.433 |  |  |
| 695 | 16.846 |  |  |  | 695 | 17.261 |  |  |  | 695 | 18.384 |  |  |
| 700 | 17.065 |  |  |  | 700 | 17.163 |  |  |  | 700 | 19.702 |  |  |
| 705 | 16.87 |  |  |  | 705 | 17.31 |  |  |  | 705 | 18.335 |  |  |
| 710 | 17.554 |  |  |  | 710 | 16.724 |  |  |  | 710 | 16.528 |  |  |
| 715 | 17.358 |  |  |  | 715 | 17.041 |  |  |  | 715 | 17.285 |  |  |
| 720 | 15.527 |  |  |  | 720 | 10.01 |  |  |  | 720 | 17.92 |  |  |
| 725 | 14.38 |  |  |  | 725 | 17.236 |  |  |  | 725 | 16.992 |  |  |
| 730 | 13.794 |  |  |  | 730 | 17.456 |  |  |  | 730 | 14.502 |  |  |
| 735 | 16.821 |  |  |  | 735 | 17.383 |  |  |  | 735 | 17.09 |  |  |
| 740 | 17.358 |  |  |  | 740 | 17.065 |  |  |  | 740 | 14.38 |  |  |
| 745 | 17.09 |  |  |  | 745 | 13.452 |  |  |  | 745 | 16.846 |  |  |
| 750 | 17.603 |  |  |  | 750 | 17.773 |  |  |  | 750 | 17.358 |  |  |
| 755 | 17.407 |  |  |  | 755 | 17.773 |  |  |  | 755 | 17.334 |  |  |
| 760 | 18.262 |  |  |  | 760 | 17.749 |  |  |  | 760 | 17.847 |  |  |
| 765 | 17.285 |  |  |  | 765 | 10.864 |  |  |  | 765 | 16.943 |  |  |
| 770 | 9.839 |  |  |  | 770 | 16.943 |  |  |  | 770 | 17.603 |  |  |
| 775 | 17.969 |  |  |  | 775 | 17.969 |  |  |  | 775 | 17.969 |  |  |
| 780 | 16.968 |  |  |  | 780 | 17.334 |  |  |  | 780 | 17.603 |  |  |
| 785 | 16.602 |  |  |  | 785 | 8.423 |  |  |  | 785 | 14.282 |  |  |
| 790 | 17.163 |  |  |  | 790 | 19.727 |  |  |  | 790 | 17.749 |  |  |
| 795 | 17.603 |  |  |  | 795 | 17.725 |  |  |  | 795 | 17.505 |  |  |
| 800 | 17.7 |  |  |  | 800 | 17.163 |  |  |  | 800 | 17.383 |  |  |
| 805 | 17.065 |  |  |  | 805 | 16.675 |  |  |  | 805 | 16.626 |  |  |
| 810 | 17.554 |  |  |  | 810 | 18.188 |  |  |  | 810 | 16.943 |  |  |
| 815 | 17.065 |  |  |  | 815 | 16.943 |  |  |  | 815 | 16.943 |  |  |
| 820 | 15.063 |  |  |  | 820 | 11.963 |  |  |  | 820 | 9.497 |  |  |
| 825 | 17.383 |  |  |  | 825 | 17.065 |  |  |  | 825 | 17.017 |  |  |
| 830 | 16.943 |  |  |  | 830 | 15.942 |  |  |  | 830 | 17.676 |  |  |
| 835 | 18.042 |  |  |  | 835 | 17.187 |  |  |  | 835 | 17.603 |  |  |
| 840 | 15.112 |  |  |  | 840 | 16.943 |  |  |  | 840 | 17.48 |  |  |
| 845 | 17.212 |  |  |  | 845 | 19.604 |  |  |  | 845 | 17.432 |  |  |
| 850 | 16.895 |  |  |  | 850 | 16.943 |  |  |  | 850 | 17.871 |  |  |
| 855 | 17.212 |  |  |  | 855 | 9.937 |  |  |  | 855 | 15.601 |  |  |
| 860 | 17.578 |  |  |  | 860 | 16.797 |  |  |  | 860 | 17.432 |  |  |
| 865 | 11.67 |  |  |  | 865 | 17.7 |  |  |  | 865 | 7.812 |  |  |
| 870 | 12.524 |  |  |  | 870 | 17.31 |  |  |  | 870 | 18.701 |  |  |
| 875 | 16.821 |  |  |  | 875 | 17.114 |  |  |  | 875 | 13.989 |  |  |
| 880 | 16.87 |  |  |  | 880 | 16.87 |  |  |  | 880 | 18.091 |  |  |
| 885 | 16.772 |  |  |  | 885 | 15.698 |  |  |  | 885 | 17.187 |  |  |
| 890 | 17.114 |  |  |  | 890 | 17.041 |  |  |  | 890 | 16.968 |  |  |
| 895 | 17.92 |  |  |  | 895 | 18.018 |  |  |  | 895 | 17.505 |  |  |
| 900 | 15.649 |  |  |  | 900 | 17.7 |  |  |  | 900 | 14.844 |  |  |
| Avg: | 16.56665193 |  |  |  | Avg: | 16.21781768 |  |  |  | Avg: | 16.18738674 |  |  |

## Raw Data from High Temperature Tests

| Time | Temperature | Time | Period | Time + 90 | Time | Temperature | Time | Period |  | Time | Temperature | Time | Period |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 41.968 | 97.0649925 | 706499.25 | 90 | 0 | 42.432 | 97.06425 | 706424.9 |  | 0 | 45.679 | 97.06367 | 706367.5 |
| 5 | 41.284 | 104.1307935 | 706580.1 | 95 | 5 | 41.016 | 104.1293 | 706501.4 |  | 5 | 45.239 | 104.1282 | 706451.4 |
| 10 | 41.87 | 111.1957076 | 706491.41 | 100 | 10 | 39.697 | 111.1934 | 706414.4 |  | 10 | 45.923 | 111.1918 | 706361.7 |
| 15 | 41.895 | 118.2613518 | 706564.42 | 105 | 15 | 39.771 | 118.2583 | 706491.9 |  | 15 | 44.653 | 118.2562 | 706443.3 |
| 20 | 40.063 | 125.3261124 | 706476.06 | 110 | 20 | 39.258 | 125.3224 | 706408.5 |  | 20 | 35.62 | 125.3198 | 706355 |
| 25 | 36.06 | 132.3915932 | 706548.08 | 115 | 25 | 41.797 | 132.3873 | 706484.8 |  | 25 | 41.577 | 132.3842 | 706439.8 |
| 30 | 33.569 | 139.4562 | 706460.68 | 120 | 30 | 43.97 | 139.4512 | 706391.1 |  | 30 | 45.41 | 139.4476 | 706345.4 |
| 35 | 41.699 | 146.5215302 | 706533.02 | 125 | 35 | 40.161 | 146.5159 | 706475.7 |  | 35 | 45.361 | 146.512 | 706437 |
| 40 | 41.064 | 153.5859468 | 706441.66 | 130 | 40 | 40.601 | 153.5798 | 706390 |  | 40 | 44.653 | 153.5754 | 706338.4 |
| 45 | 41.992 | 160.6511688 | 706522.2 | 135 | 45 | 40.625 | 160.6445 | 706470.6 |  | 45 | 44.824 | 160.6397 | 706433.4 |
| 50 | 41.724 | 167.7154351 | 706426.63 | 140 | 50 | 40.259 | 167.7083 | 706373.5 |  | 50 | 37.378 | 167.7031 | 706338.1 |
| 55 | 41.382 | 174.7805322 | 706509.71 | 145 | 55 | 31.104 | 174.7729 | 706465.8 |  | 55 | 44.238 | 174.7674 | 706428.4 |
| 60 | 42.261 | 181.8446616 | 706412.94 | 150 | 60 | 41.797 | 181.8366 | 706366.4 |  | 60 | 44.58 | 181.8307 | 706327.7 |
| 65 | 40.698 | 188.9096476 | 706498.6 | 155 | 65 | 41.431 | 188.9012 | 706456 |  | 65 | 45.581 | 188.895 | 706429.5 |
| 70 | 41.577 | 195.9736206 | 706397.3 | 160 | 70 | 40.601 | 195.9647 | 706358.4 |  | 70 | 44.312 | 195.9582 | 706325.1 |
| 75 | 40.601 | 203.0384383 | 706481.77 | 165 | 75 | 21.606 | 203.0293 | 706452.7 |  | 75 | 44.531 | 203.0224 | 706419.5 |
| 80 | 41.309 | 210.102284 | 706384.57 | 170 | 80 | 40.259 | 210.0928 | 706351.9 |  | 80 | 33.838 | 210.0856 | 706317.1 |
| 85 | 40.967 | 217.1669942 | 706471.02 | 175 | 85 | 40.625 | 217.1572 | 706444.9 |  | 85 | 43.164 | 217.1498 | 706417.2 |
| 90 | 41.504 | 224.230703 | 706370.88 | 180 | 90 | 40.527 | 224.2207 | 706344.1 |  | 90 | 44.629 | 224.2128 | 706308.4 |
| 95 | 40.991 | 231.295346 | 706464.3 | 185 | 95 | 41.553 | 231.2851 | 706446.2 |  | 95 | 44.409 | 231.277 | 706414.8 |
| 100 | 41.992 | 238.3588511 | 706350.51 | 190 | 100 | 31.03 | 238.3485 | 706335.8 |  | 100 | 44.556 | 238.3401 | 706306.8 |
| 105 | 41.113 | 245.4234177 | 706456.66 | 195 | 105 | 39.941 | 245.4129 | 706441.1 |  | 105 | 45.166 | 245.4042 | 706413.7 |
| 110 | 41.016 | 252.4869166 | 706349.89 | 200 | 110 | 41.675 | 252.4762 | 706329.3 |  | 110 | 39.429 | 252.4672 | 706299.8 |
| 115 | 40.576 | 259.5513494 | 706443.28 | 205 | 115 | 41.675 | 259.5405 | 706432.3 |  | 115 | 44.019 | 259.5313 | 706411.6 |
| 120 | 40.723 | 266.6147096 | 706336.02 | 210 | 120 | 38.159 | 266.6038 | 706324.9 |  | 120 | 43.921 | 266.5943 | 706297.6 |
| 125 | 41.138 | 273.6790603 | 706435.07 | 215 | 125 | 40.967 | 273.6681 | 706434.2 |  | 125 | 44.531 | 273.6583 | 706402.4 |
| 130 | 41.187 | 280.7422494 | 706318.91 | 220 | 130 | 35.205 | 280.7313 | 706319.7 |  | 130 | 44.824 | 280.7212 | 706293.7 |
| 135 | 40.186 | 287.8064923 | 706424.29 | 225 | 135 | 40.088 | 287.7956 | 706426.3 |  | 135 | 44.922 | 287.7853 | 706408 |
| 140 | 40.161 | 294.8696074 | 706311.51 | 230 | 140 | 40.845 | 294.8588 | 706318.3 |  | 140 | 33.789 | 294.8482 | 706288.1 |
| 145 | 41.846 | 301.9338516 | 706424.42 | 235 | 145 | 29.321 | 301.923 | 706422.6 |  | 145 | 38.062 | 301.9123 | 706406.5 |
| 150 | 40.649 | 308.9967878 | 706293.62 | 240 | 150 | 40.771 | 308.9861 | 706314.3 |  | 150 | 33.716 | 308.9751 | 706284.7 |
| 155 | 41.357 | 316.0609319 | 706414.41 | 245 | 155 | 40.796 | 316.0504 | 706424.4 |  | 155 | 36.304 | 316.0392 | 706405.7 |
| 160 | 40.601 | 323.1238027 | 706287.08 | 250 | 160 | 40.649 | 323.1134 | 706304.1 |  | 160 | 34.18 | 323.102 | 706282.1 |
| 165 | 41.602 | 330.1879065 | 706410.38 | 255 | 165 | 40.063 | 330.1776 | 706424 |  | 165 | 43.018 | 330.1661 | 706408.1 |
| 170 | 41.235 | 337.2506803 | 706277.38 | 260 | 170 | 26.489 | 337.2407 | 706300.5 |  | 170 | 43.091 | 337.2288 | 706272.6 |
| 175 | 40.527 | 344.3147043 | 706402.4 | 265 | 175 | 40.039 | 344.3049 | 706422.1 |  | 175 | 45.264 | 344.2929 | 706405.6 |
| 180 | 40.015 | 351.3774986 | 706279.43 | 270 | 180 | 39.771 | 351.3679 | 706297.9 |  | 180 | 44.849 | 351.3556 | 706273.8 |
| 185 | 40.479 | 358.4414483 | 706394.97 | 275 | 185 | 41.187 | 358.432 | 706418.8 |  | 185 | 43.677 | 358.4196 | 706400.3 |
| 190 | 41.602 | 365.5040809 | 706263.26 | 280 | 190 | 40.161 | 365.495 | 706295.2 |  | 190 | 45.337 | 365.4823 | 706267.5 |
| 195 | 40.674 | 372.5679436 | 706386.27 | 285 | 195 | 38.159 | 372.5592 | 706423.3 |  | 195 | 42.773 | 372.5463 | 706400.9 |
| 200 | 41.089 | 379.6305118 | 706256.82 | 290 | 200 | 34.448 | 379.6221 | 706285.5 |  | 200 | 43.53 | 379.609 | 706270.3 |
| 205 | 40.576 | 386.6944324 | 706392.06 | 295 | 205 | 40.137 | 386.6862 | 706415.6 |  | 205 | 43.457 | 386.6731 | 706410.1 |
| 210 | 40.405 | 393.7568414 | 706240.9 | 300 | 210 | 39.99 | 393.749 | 706280.6 |  | 210 | 45.386 | 393.7357 | 706260.9 |
| 215 | 40.381 | 400.8207 | 706385.86 | 305 | 215 | 35.083 | 400.8132 | 706419.5 |  | 215 | 44.092 | 400.7997 | 706402.5 |
| 220 | 36.157 | 407.8831444 | 706244.44 | 310 | 220 | 40.332 | 407.876 | 706277.8 |  | 220 | 44.922 | 407.8623 | 706257.6 |
| 225 | 40.601 | 414.9468678 | 706372.34 | 315 | 225 | 40.674 | 414.9402 | 706413.6 |  | 225 | 43.408 | 414.9263 | 706398.1 |
| 230 | 40.503 | 422.0092372 | 706236.94 | 320 | 230 | 39.6 | 422.0029 | 706275.9 |  | 230 | 43.042 | 421.9888 | 706254.6 |
| 235 | 39.819 | 429.0730722 | 706383.5 | 325 | 235 | 40.088 | 429.067 | 706409.3 |  | 235 | 39.16 | 429.0529 | 706411.3 |
| 240 | 40.503 | 436.1353136 | 706224.14 | 330 | 240 | 40.82 | 436.1297 | 706272 |  | 240 | 36.841 | 436.1154 | 706250.3 |
| 245 | 37.378 | 443.1990527 | 706373.91 | 335 | 245 | 39.99 | 443.1938 | 706407.8 |  | 245 | 43.408 | 443.1795 | 706409.8 |
| 250 | 40.552 | 450.2611849 | 706213.22 | 340 | 250 | 39.624 | 450.2565 | 706268.4 |  | 250 | 46.509 | 450.242 | 706244.7 |
| 255 | 40.283 | 457.3249256 | 706374.07 | 345 | 255 | 40.259 | 457.3207 | 706417.4 |  | 255 | 43.652 | 457.306 | 706405.6 |
| 260 | 40.576 | 464.38707 | 706214.44 | 350 | 260 | 33.96 | 464.3833 | 706263.7 |  | 260 | 44.312 | 464.3685 | 706247.4 |
| 265 | 40.21 | 471.4507961 | 706372.61 | 355 | 265 | 39.868 | 471.4475 | 706423.7 |  | 265 | 44.531 | 471.4325 | 706403.1 |
| 270 | 40.088 | 478.5128264 | 706203.03 | 360 | 270 | 40.112 | 478.5101 | 706259 |  | 270 | 44.067 | 478.4949 | 706239 |
| 275 | 31.396 | 485.5764642 | 706363.78 | 365 | 275 | 40.552 | 485.5743 | 706420.9 |  | 275 | 43.311 | 485.5591 | 706415.8 |
| 280 | 40.43 | 492.6385573 | 706209.31 | 370 | 280 | 39.673 | 492.6369 | 706259 |  | 280 | 44.287 | 492.6214 | 706233.7 |
| 285 | 40.015 | 499.7021792 | 706362.19 | 375 | 285 | 41.699 | 499.7012 | 706428.1 |  | 285 | 44.263 | 499.6856 | 706416.4 |
| 290 | 32.178 | 506.7640606 | 706188.14 | 380 | 290 | 31.177 | 506.7637 | 706253 |  | 290 | 43.848 | 506.7479 | 706234.2 |
| 295 | 40.356 | 513.8277116 | 706365.1 | 385 | 295 | 40.356 | 513.8279 | 706420.6 |  | 295 | 43.457 | 513.812 | 706410.9 |
| 300 | 37.769 | 520.8895083 | 706179.67 | 390 | 300 | 39.551 | 520.8904 | 706245.7 |  | 300 | 40.82 | 520.8743 | 706229 |
| 305 | 40.112 | 527.9531851 | 706367.68 | 395 | 305 | 40.43 | 527.9546 | 706424.7 |  | 305 | 40.527 | 527.9385 | 706418.3 |
| 310 | 41.016 | 535.0149586 | 706177.35 | 400 | 310 | 40.088 | 535.0171 | 706241.3 |  | 310 | 42.554 | 535.0008 | 706230.2 |
| 315 | 40.625 | 542.0785274 | 706356.88 | 405 | 315 | 30.078 | 542.0814 | 706438.5 |  | 315 | 34.741 | 542.065 | 706420.9 |
| 320 | 41.919 | 549.1402762 | 706174.88 | 410 | 320 | 39.624 | 549.1439 | 706244.6 |  | 320 | 29.932 | 549.1272 | 706221.8 |
| 325 | 31.86 | 556.2039876 | 706371.14 | 415 | 325 | 39.453 | 556.2083 | 706438.1 |  | 325 | 35.107 | 556.1916 | 706433 |
| 330 | 40.894 | 563.2656533 | 706166.57 | 420 | 330 | 39.795 | 563.2706 | 706233.2 |  | 330 | 41.382 | 563.2538 | 706221.6 |
| 335 | 40.088 | 570.3293424 | 706368.91 | 425 | 335 | 40.723 | 570.3349 | 706427.6 |  | 335 | 44.507 | 570.3181 | 706426.9 |
| 340 | 39.893 | 577.3909863 | 706164.39 | 430 | 340 | 43.066 | 577.3973 | 706240.8 |  | 340 | 44.312 | 577.3803 | 706223.3 |
| 345 | 41.528 | 584.4546068 | 706362.05 | 435 | 345 | 38.11 | 584.4617 | 706443 |  | 345 | 43.188 | 584.4447 | 706441.1 |
| 350 | 40.967 | 591.5161079 | 706150.11 | 440 | 350 | 37.109 | 591.524 | 706228 |  | 350 | 43.091 | 591.5069 | 706216.3 |
| 355 | 39.722 | 598.5798217 | 706371.38 | 445 | 355 | 30.127 | 598.5885 | 706447.5 |  | 355 | 41.553 | 598.5713 | 706444.1 |
| 360 | 41.602 | 605.6413065 | 706148.48 | 450 | 360 | 39.429 | 605.6508 | 706235.7 |  | 360 | 44.141 | 605.6334 | 706212.4 |
| 365 | 39.722 | 612.7050889 | 706378.24 | 455 | 365 | 40.015 | 612.7153 | 706446.2 |  | 365 | 35.01 | 612.6978 | 706438.2 |
| 370 | 40.186 | 619.7665664 | 706147.75 | 460 | 370 | 41.406 | 619.7776 | 706227.9 |  | 370 | 43.262 | 619.7599 | 706210.2 |
| 375 | 38.086 | 626.8302987 | 706373.23 | 465 | 375 | 39.551 | 626.8423 | 706468.9 |  | 375 | 43.481 | 626.8245 | 706458.4 |
| 380 | 31.396 | 633.8918165 | 706151.78 | 470 | 380 | 39.062 | 633.9045 | 706227.3 |  | 380 | 42.871 | 633.8866 | 706208.7 |
| 385 | 39.893 | 640.9556351 | 706381.86 | 475 | 385 | 40.234 | 640.9693 | 706481.8 |  | 385 | 44.189 | 640.9513 | 706472.9 |
| 390 | 41.504 | 648.0169975 | 706136.24 | 480 | 390 | 40.308 | 648.0316 | 706223.8 |  | 390 | 42.993 | 648.0135 | 706215.9 |
| 395 | 40.869 | 655.0808083 | 706381.08 | 485 | 395 | 41.04 | 655.0965 | 706488.3 |  | 395 | 43.164 | 655.0784 | 706497.1 |
| 400 | 40.503 | 662.1421925 | 706138.42 | 490 | 400 | 39.16 | 662.1586 | 706215.8 |  | 400 | 43.408 | 662.1405 | 706201.2 |
| 405 | 39.795 | 669.2062254 | 706403.29 | 495 | 405 | 39.551 | 669.2236 | 706498.4 |  | 405 | 45.166 | 669.2055 | 706500.4 |
| 410 | 39.136 | 676.2676013 | 706137.59 | 500 | 410 | 39.502 | 676.2857 | 706211.8 |  | 410 | 40.918 | 676.2675 | 706205.9 |
| 415 | 41.04 | 683.3318877 | 706428.64 | 505 | 415 | 39.624 | 683.3509 | 706515.7 |  | 415 | 44.385 | 683.3329 | 706537.8 |
| 420 | 34.619 | 690.3931886 | 706130.09 | 510 | 420 | 39.331 | 690.4131 | 706219 |  | 420 | 33.936 | 690.395 | 706211.5 |
| 425 | 40.063 | 697.4576993 | 706451.07 | 515 | 425 | 42.163 | 697.4782 | 706513.8 |  | 425 | 42.603 | 697.4602 | 706521.3 |
| 430 | 33.032 | 704.5189317 | 706123.24 | 520 | 430 | 39.478 | 704.5403 | 706205.8 |  | 430 | 33.618 | 704.5222 | 706201.8 |
| 435 | 27.222 | 711.5833629 | 706443.12 | 525 | 435 | 39.16 | 711.6059 | 706559.8 |  | 435 | 42.92 | 711.588 | 706579.2 |
| 440 | 39.551 | 718.6445666 | 706120.37 | 530 | 440 | 39.16 | 718.6679 | 706207.2 |  | 440 | 42.651 | 718.65 | 706198.1 |
| 445 | 38.843 | 725.7088807 | 706431.41 | 535 | 445 | 40.283 | 725.7338 | 706581.9 |  | 445 | 43.14 | 725.7162 | 706617.9 |
| 450 | 39.648 | 732.7698508 | 706097.01 | 540 | 450 | 39.136 | 732.7958 | 706200.7 |  | 450 | 43.433 | 732.7782 | 706199.6 |
| 455 | 39.453 | 739.8349862 | 706513.54 | 545 | 455 | 39.99 | 739.862 | 706622.1 |  | 455 | 43.506 | 739.8447 | 706654.8 |
| 460 | 40.894 | 746.8959944 | 706100.82 | 550 | 460 | 40.479 | 746.924 | 706202.3 |  | 460 | 42.847 | 746.9067 | 706195.3 |
| 465 | 40.088 | 753.9611236 | 706512.92 | 555 | 465 | 39.502 | 753.9909 | 706686.8 |  | 465 | 33.569 | 753.974 | 706729.8 |
| 470 | 39.819 | 761.0221298 | 706100.62 | 560 | 470 | 39.453 | 761.0528 | 706187.9 |  | 470 | 42.456 | 761.036 | 706196.8 |
| 475 | 40.698 | 768.0877268 | 706559.7 | 565 | 475 | 39.233 | 768.1207 | 706789.2 |  | 475 | 37.524 | 768.104 | 706805.1 |
| 480 | 40.747 | 775.1487438 | 706101.7 | 570 | 480 | 34.473 | 775.1825 | 706189.5 |  | 480 | 42.896 | 775.1659 | 706187.8 |
| 485 | 39.575 | 782.2150112 | 706626.74 | 575 | 485 | 39.746 | 782.2516 | 706909.4 |  | 485 | 30.737 | 782.2262 | 706027.2 |
| 490 | 39.331 | 789.2759916 | 706098.04 | 580 | 490 | 38.843 | 789.3134 | 706175.7 |  | 490 | 42.456 | 789.2881 | 706192.8 |
| 495 | 39.966 | 796.3434824 | 706749.08 | 585 | 495 | 39.355 | 796.3856 | 707216.8 |  | 495 | 42.871 | Ignore | 707815.80 |
| 500 | 39.819 | 803.4044412 | 706095.88 | 590 | 500 | 39.648 |  | 706192.19 | Ignore | 500 | 43.579 | OL | 1271137.23 |
| 505 | 40.479 | 810.473712 | 706927.08 | 595 | 505 | 40.43 |  | 169571.30 | OL | 505 | 42.676 |  |  |
| 510 | 39.575 | 817.5345874 | 706087.54 | 600 | 510 | 39.233 |  |  |  | 510 | 42.676 |  |  |
| 515 | 40.503 |  | 707736.71 | 605 | 515 | 38.94 |  |  |  | 515 | 42.48 |  |  |
| 520 | 39.6 |  | Ignore Last | 610 | 520 | 38.696 |  |  |  | 520 | 42.969 |  |  |
| 525 | 39.331 |  |  | 615 | 525 | 37.622 |  |  |  | 525 | 42.7 |  |  |
| 530 | 40.283 |  |  | 620 | 530 | 39.136 |  |  |  | 530 | 43.018 |  |  |
| 535 | 39.526 |  |  | 625 | 535 | 39.014 |  |  |  | 535 | 42.163 |  |  |
| 540 | 39.551 |  |  | 630 | 540 | 39.136 |  |  |  | 540 | 42.31 |  |  |
| 545 | 40.186 |  |  | 635 | 545 | 39.893 |  |  |  | 545 | 43.14 |  |  |
| 550 | 36.89 |  |  | 640 | 550 | 39.941 |  |  |  | 550 | 43.823 |  |  |
| 555 | 39.453 |  |  | 645 | 555 | 39.722 |  |  |  | 555 | 33.911 |  |  |
| 560 | 39.526 |  |  | 650 | 560 | 39.819 |  |  |  | 560 | 42.773 |  |  |
| 565 | 39.844 |  |  | 655 | 565 | 36.719 |  |  |  | 565 | 40.405 |  |  |
| 570 | 40.479 |  |  | 660 | 570 | 39.575 |  |  |  | 570 | 43.359 |  |  |
| 575 | 39.282 |  |  | 665 | 575 | 28.564 |  |  |  | 575 | 42.456 |  |  |
| 580 | 39.917 |  |  | 670 | 580 | 33.35 |  |  |  | 580 | 43.335 |  |  |
| 585 | 39.331 |  |  | 675 | 585 | 39.868 |  |  |  | 585 | 41.968 |  |  |
| 590 | 39.258 |  |  | 680 | 590 | 40.063 |  |  |  | 590 | 42.285 |  |  |
| 595 | 39.844 |  |  | 685 | 595 | 38.818 |  |  |  | 595 | 43.188 |  |  |
| 600 | 39.136 |  |  | 690 | 600 | 38.721 |  |  |  | 600 | 33.203 |  |  |
| 605 | 39.014 |  |  | 695 | 605 | 39.648 |  |  |  | 605 | 43.188 |  |  |
| 610 | 38.916 |  |  | 700 | 610 | 39.99 |  |  |  | 610 | 42.505 |  |  |
| 615 | 39.404 |  |  | 705 | 615 | 38.696 |  |  |  | 615 | 44.604 |  |  |
| 620 | 39.99 |  |  | 710 | 620 | 41.333 |  |  |  | 620 | 42.31 |  |  |
| 625 | 38.892 |  |  | 715 | 625 | 39.307 |  |  |  | 625 | 43.652 |  |  |
| 630 | 38.989 |  |  | 720 | 630 | 38.745 |  |  |  | 630 | 42.554 |  |  |
| 635 | 38.696 |  |  | 725 | 635 | 30.542 |  |  |  | 635 | 41.528 |  |  |
| 640 | 39.331 | Avg: | 706344.2596 | 730 | 640 | 37.573 | Avg: | 706385.6 |  | 640 | 42.261 | Avg: | 706351.6 |
| 645 | 37.671 |  |  | 735 | 645 | 31.738 |  |  |  | 645 | 42.505 |  |  |
| 650 | 38.843 |  |  | 740 | 650 | 38.867 |  |  |  | 650 | 42.603 |  | 706360.5 |
| 655 | 39.038 |  |  | 745 | 655 | 39.844 |  |  |  | 655 | 42.139 |  |  |
| 660 | 39.062 |  |  | 750 | 660 | 38.452 |  |  |  | 660 | 42.48 |  |  |
| 665 | 39.014 |  |  | 755 | 665 | 38.721 |  |  |  | 665 | 40.942 |  |  |
| 670 | 38.94 |  |  | 760 | 670 | 39.233 |  |  |  | 670 | 42.261 |  |  |
| 675 | 31.25 |  |  | 765 | 675 | 40.894 |  |  |  | 675 | 37.109 |  |  |
| 680 | 28.564 |  |  | 770 | 680 | 38.647 |  |  |  | 680 | 42.31 |  |  |
| 685 | 37.646 |  |  | 775 | 685 | 40.723 |  |  |  | 685 | 33.521 |  |  |
| 690 | 39.331 |  |  | 780 | 690 | 38.94 |  |  |  | 690 | 42.358 |  |  |
| 695 | 40.112 |  |  | 785 | 695 | 33.179 |  |  |  | 695 | 38.281 |  |  |
| 700 | 39.99 |  |  | 790 | 700 | 29.736 |  |  |  | 700 | 42.993 |  |  |
| 705 | 39.697 |  |  | 795 | 705 | 33.057 |  |  |  | 705 | 40.43 |  |  |
| 710 | 40.259 |  |  | 800 | 710 | 30.225 |  |  |  | 710 | 42.187 |  |  |
| 715 | 38.696 |  |  | 805 | 715 | 38.403 |  |  |  | 715 | 38.452 |  |  |
| 720 | 37.158 |  |  | 810 | 720 | 38.599 |  |  |  | 720 | 42.407 |  |  |
| 725 | 42.187 |  |  | 815 | 725 | 38.892 |  |  |  | 725 | 42.651 |  |  |
| 730 | 39.697 |  |  | 820 | 730 | 38.647 |  |  |  | 730 | 42.603 |  |  |
| 735 | 39.087 |  |  | 825 | 735 | 40.21 |  |  |  | 735 | 39.697 |  |  |
| 740 | 38.77 |  |  | 830 | 740 | 38.647 |  |  |  | 740 | 43.384 |  |  |
| 745 | 37.256 |  |  | 835 | 745 | 38.452 |  |  |  | 745 | 42.041 |  |  |
| 750 | 29.492 |  |  | 840 | 750 | 38.672 |  |  |  | 750 | 38.599 |  |  |
| 755 | 39.282 |  |  | 845 | 755 | 36.816 |  |  |  | 755 | 42.017 |  |  |
| 760 | 38.623 |  |  | 850 | 760 | 30.2 |  |  |  | 760 | 44.092 |  |  |
| 765 | 39.551 |  |  | 855 | 765 | 35.278 |  |  |  | 765 | 33.154 |  |  |
| 770 | 39.307 |  |  | 860 | 770 | 39.893 |  |  |  | 770 | 41.65 |  |  |
| 775 | 38.623 |  |  | 865 | 775 | 41.064 |  |  |  | 775 | 42.31 |  |  |
| 780 | 39.087 |  |  | 870 | 780 | 39.062 |  |  |  | 780 | 42.041 |  |  |
| 785 | 38.647 |  |  | 875 | 785 | 38.208 |  |  |  | 785 | 41.724 |  |  |
| 790 | 38.867 |  |  | 880 | 790 | 38.892 |  |  |  | 790 | 33.472 |  |  |
| 795 | 39.038 |  |  | 885 | 795 | 39.917 |  |  |  | 795 | 39.087 |  |  |
| 800 | 39.429 |  |  | 890 | 800 | 38.403 |  |  |  | 800 | 42.358 |  |  |
| 805 | 38.574 |  |  | 895 | 805 | 39.478 |  |  |  | 805 | 43.237 |  |  |
| 810 | 36.572 |  |  | 900 | 810 | 38.452 |  |  |  | 810 | 41.699 |  |  |
| 815 | 31.128 |  |  | 905 | 815 | 29.517 |  |  |  | 815 | 43.335 |  |  |
| 820 | 38.11 |  |  | 910 | 820 | 36.157 |  |  |  | 820 | 41.87 |  |  |
| 825 | 38.501 |  |  | 915 | 825 | 37.842 |  |  |  | 825 | 42.456 |  |  |
| 830 | 38.843 |  |  | 920 | 830 | 38.501 |  |  |  | 830 | 41.699 |  |  |
| 835 | 40.063 |  |  | 925 | 835 | 38.257 |  |  |  | 835 | 32.178 |  |  |
| 840 | 38.599 |  |  | 930 | 840 | 38.721 |  |  |  | 840 | 42.773 |  |  |
| 845 | 38.501 |  |  | 935 |  |  |  |  |  |  |  |  |  |
| 850 | 38.77 |  |  | 940 |  |  |  |  |  |  |  |  |  |
| 855 | 38.745 |  |  | 945 |  |  |  |  |  |  |  |  |  |
| 860 | 30.103 |  |  | 950 |  |  |  |  |  |  |  |  |  |
| 865 | 38.428 |  |  | 955 |  |  |  |  |  |  |  |  |  |
| 870 | 30.859 |  |  | 960 |  |  |  |  |  |  |  |  |  |
| 875 | 39.551 |  |  | 965 |  |  |  |  |  |  |  |  |  |
| 880 | 39.478 |  |  | 970 |  |  |  |  |  |  |  |  |  |
| 885 | 38.354 |  |  | 975 |  |  |  |  |  |  |  |  |  |
| 890 | 38.672 |  |  | 980 |  |  |  |  |  |  |  |  |  |
| 895 | 32.495 |  |  | 985 |  |  |  |  |  |  |  |  |  |
| 900 | 35.889 |  |  | 990 |  |  |  |  |  |  |  |  |  |
| Avg: | 39.15923204 |  |  |  | Avg: | 38.52971598 |  |  |  | Avg: | 41.77810059 |  |  |